6.  HARDWARE DESCRIPTION

6.1.  ON-BOARD PERIPHERALS

6.1.1.  BEEPER

The Bell sound is normally generated by the build-in beeper of the VT220 keyboard. On applications where the keyboard is not used. the bell sound may be generated by the on-board beeper of the AP988200 controller board. You may also have both the keyboard an on-board beeper simultaneously.

The on-board beeper also generates a key-click for the Touch Screen Overlay.

6.1.2.  POWER SUPPLY

The on-board Swithed Mode Power Supply (SMPS) generates the 5 VDC operating voltage for both the AP988200 display controller and the display panel. The 12 V input voltage is not used internally by the AP988200. Instead of 12V, you may use any voltage required by the display panel, up to 40 VDC. If required, the SMPS may be omitted and the 12V and 5V voltages supplied externally.

The +12V and -12V voltages for the RS232C interface are generated on-board by the RS232C interface chip.

Fig. 6.1:  AP988200 printed circuit board

6.2.  JUMPER SETTINGS

The following jumper options are available on-board. The default setting are underlined.

Mode Select  jumper block  (J10):

o o o o o o o o

J10	o o o o o o o o

Bit position:	7 6 5 4 3 2 1 0

Position	explanation				Open		Closed

Bit 0.	(reserved)				     -		     -

Bit 1.	(reserved)				     -		     -

Bit 2.	(reserved)				     -		     -

Bit 3.	TSO y-coord. inverse			Normal		Inversed

Bit 4.	TSO enable				Disabled	Enabled

Bit 5.	RS485 mode				Disabled	Enabled

Bit 6.	Battery backup				Enabled		Disabled

Bit 7.	Display power-up screen			No		Yes

Note:  The J10 jumper block is read by software only at power-up or reset.

Other jumpers:

Pins connected:

Jumper		explanation			  1 to 2		2 to 3

J1		LCA reset input select		  (manual)	system

J2		LCA load data input		  external	CPU

J3		LCA external load input conn.	  (open)

J4		LCA load clock input		  external	CPU

J5		LCA load mode select		  external	CPU

J6		Char Gen RAM size		  32K		8K

J8		Host port data input select	  RS232C	Current loop

J9		Host port DTR output, force high	  (open)

J21		EPROM type			  27512		27256

Jumper	explanation				1  -  2  -  3  -  4  -  5

J7		Host port CTS/DCD input select

a)  Disabled		   X    -       -     X

b)  CTS			   X    -      X     -

c)  DCD		    -     X     -     X

Jumper		explanation				default setting

J11		Host port  CTS / RTS pin select		1 to 2,  3 to 4

J12		Host port TxD / RxD pin select		1 to 2,  3 to 4

J13		Printer port TxD pin select

J14		Printer port handshake input select

J15		IBM AT keyboard reset enable		open

J16,J17,J23	Display connector pin configuration	open

J18		Display connector Black pin select		2 to 3

J19		Host port DCD / CTS pin select		open

J20		Battery voltage detect			open, if battery installed

J22		Watchdog enable			open

6.3.  INTERFACES

K1  -  MEMBRANE T.S.O. INTERFACE

Membrane switch type Touch Screen Overlay input device (TSO) is connected to this connector. You may also connect an undecoded matrix keyboard here. A special version of the Alphasoft-200 EPROM is required for this interface.

The membrane TSO is a matrix of contacts, each of which connects one of the column lines to one of row lines. The AP988200 scanns the TSO matrix by supplying low level voltage to one of the 12 column lines at a time and then reading the 8 row lines.

Note: If there is need for multiple keys to be pressed simultaneously (shift or control keys, etc.), isolation diodes must be installed to each of the column lines.

Enable the TSO by connecting jumper J10.4.

Membrane TSO specifications:

Max. resolution (matrix size):		8 by 12

Contact resistance, max:			2  kOhm

Connector K1:  20 pin single line:

oooooooooooooooooooo

1                  20

1	Row 7		 9	Colum 0		17	Colum 8

2	Row 6		10	Colum 1		18	Colum 9

3	Row 5		11	Colum 2		19	Colum 10

4	Row 4		12	Colum 3		20	Colum 11

5	Row 3		13	Colum 4

6	Row 2		14	Colum 5

7	Row 1		15	Colum 6

8	Row 0		16	Colum 7

K2  -  OPTICAL T.S.O. INTERFACE

Optical TSO (AP988030/A, AP988030/B or AP988030/BB) is connected to this interface. The TSO is enabled by connecting the jumper J10.4. The AP988200 then performs the scanning of the TSO and sends coordinates to the host serial port.

Connector K2:  20 pin dual line:

2        20

oooooooooo

oooooooooo

1        19

1	+5V		 2	+5V

3	Data 4		 4	Data 3

5	Data 2		 6	Data 1

7	Data 0		 8	Addr 2

9	Addr 1		10	Addr 0

11	Clock		12	--

13	Addr 4		14	--

15	GND		16	GND

17	+12V		18	+12V

19	tx/rx sel	20	Beeper

K3  -  VT220 KEYBOARD INTERFACE

A VT220 compatible keyboard can be connected to this interface. The interface is asynchronous serial interface, 4800 bps, RS232C compatible.

Connector K3:  4 pin modular jack  (telephone connector)

1	RxD

2	+12V

3	GND

4	TxD

K4  -  SERIAL PRINTER INTERFACE

RS232C compatible serial printer output. Baud rate is fixed 4800 bps. This interface has no Receive Data input. Therefore, an XON-XOFF handshake is not possible, and hardware DTR handshake must be used. Use jumper J14 to select which pin is used for handshake input. A 9-pin D-type connector may be connected here by using an ISD connector and flat cable. Pin assignment is compatible with standard 9-pin connectors.

Connector K4:  10 pin dual inline

2   10

ooooo

ooooo

1   9

1	--		 2	DTR *

3	TxD **		 4	DTR *

5	TxD **		 6	DTR *

7	DTR *		 8	--

9	GND		10	--

* Use J14 to select a pin for DTR.

** Use J13 to select a pin for TxD.

K5  -  COMMON CONNECTOR

This connector contains all the inputs and outputs of K3, K4, K6 and K7, plus TTL level I/O signals. K5 may be used, if the connector module is not installed. Ask for full details.

K6  -  HOST SERIAL PORT (RS232C)

An RS232C compatible serial interface for the host computer. Communication parameters for this interface are selected from the SETUP menu. Jumpers J7, J11, J12 and J19 are used to select the pin assignment. A 9-pin D-type connector may be connected here by using an ISD connector and flat cable; pin assignment is compatible with standard 9-pin D-connectors used on many computers.

Connector K6:  10 pin dual inline

2   10

ooooo

ooooo

1   9

1	DCD		 2	DTR

3	RxD/TxD 		 4	RTS/CTS

5	TxD/RxD 		 6	CTS/RTS

7	DTR 		 8	--

9	GND		10	--

* Use jumper J12 to select pin for TxD and RxD.

Use jumper J11 to select pin for RTS and CTS

Use jumper J19 to use DCD input instead of CTS

Use jumper J9 to tie DTR to +12V, if required.

K7  -  IBM AT KEYBOARD INTERFACE

An IBM AT compatible keyboard may be connected here. The interface is synchronous serial interface, using TTL levels. Note: IBM PC/XT keyboard can not be used.  A special version of the Alphasoft-200 EPROM is required in order to use this interface.

Connector K7:  4 pin DIN connector

1	CLK

2	Data in/out

3	Reset

4	GND

5	+5V

K8  -  HOST SERIAL PORT, CURRENT LOOP   (optional)

Passive 20 mA opto isolated current loop interface. The current must be supplied externally.  Note:  Maximum voltage for the external current source is 24 V.

Use jumper J8 to select RxD from RS232C or Current  Loop.

Connector K8:  10 pin dual inline

2   10

ooooo

ooooo

1   9

1	+12V		 2	RxD (+)

3	+12V	 	 4	TxD (-)

5	GND	 	 6	TxD (+)

7	GND 		 8	GND

9	RxD (-)		10	--

K9  -  DISPLAY CONNECTOR

Interface for the Flat Panel Display. This interface is software configurable to suit most display types. Pin assignments may wary between different display types and manufacturers. Display type is selected by installing corresponding Alphasoft EPROM and setting jumpers J16, J17, J18 and J23. 10 to 22 pin connectors may be assembled on the PCB. The pin assignments for Finlux / Planar / Sharp EL displays and for Panasonic Plasma display are given below.

Connector K9:

a)  EL-display: 					b)  Panasonic PG3 Plasma display:

2      16					2        20

oooooooo						oooooooooo

oooooooo						oooooooooo

1      15					1        19

1  +12V		   2  +12V			 1  +5V 		   2  +5V

3  +5V	 	   4  +5V			 3  ENA		   4  BACKG

5  (+25V) 	   6  (+25V)			 5  Vsync	   6  GND

7  GND 		   8  GND			 7  Hsync	   8  GND

9  VSync	  10  GND			 9  BLANK	  10  GND

11  HSync	  12  GND			11  CLOCK	  12  GND

13  CLK		  14  GND			13  DATA0	  14  GND

15  Data		  16  GND			15  DATA1	  16  GND

17  DATA2	  18  GND

19  DATA3	  20  GND

K10  -  HOST SERIAL PORT,  RS485   (optional)

RS485 compatible serial interface for host computer. (The RS485 is also compatible with RS422 standard.)

RS485 allows the 38400 bps transfer rate to be used at distances of up to 1500 m. Up to 32 terminals may be connected to the same party line. Single twisted pair cable is used to connect the terminals and the host computer. Optionally, an add-on board is available for full duplex communication with host via two twisted pair cables.

Use jumper J8 to select RxD from either RS232C or RS485.  Note: RS485 and Current Loop may not co-exist on the same board.

Connector K10:  10 pin dual inline

2   10

ooooo

ooooo

1   9

1   +12V		  2   +Data

3   +12V	 	  4   --

5   GND	 	  6   GND

7   GND 		  8   GND

9   -Data	 10   --

6.4.  TECHNICAL SPECIFICATIONS:

Technology:		CMOS

CPU:			Z180,  6.144 MHz

EPROM:			32K, expandable to 64K

RAM:			32K static, with battery backup

Video controller:		HD63B45 + Custom LCA

Video RAM:		8K character RAM, 8K attribute RAM, static

Character generator:	32K static RAM

Supply voltage:		9 to 24 VDC   (depending on display type)

or optionally:	5  VDC ,  ±5 %   (if on-board power module not installed)

Supply current:		80 mA (typ)  @ 12V  (excluding display and keyboard)

5V output for display:	500 mA  (max)

Operating temperature:	0 to 70 °C      *)

Storege temperature:	-25 to 85 °C

Humidity:			0 to 90 %, non-condensing

Dimensions:		228 x 139 x 17 mm

Weight:			170 g

*)  Extented temperature range available on request.

DIMENSIONS:

Board dimensions:

A	140.0	mm

B	228.0	mm

C	43.0	mm

D	53.0	mm

E	36,0	mm

F	8.5	mm

G	65.0	mm

Screw positions:

f1	3.8	mm

h	116.0	mm

i	12.0	mm

j	4.0	mm

k	220.0	mm

f2	3.0	mm

l	215.0	mm

m	6.5	mm

n	3.0	mm