4.  P R O G R A M M E R S   R E F E R E N C E

This section describes the AP988200 terminal from the point of view of a programmer, who is using the terminal as a data input/output device.

Chapter 4.1  list all the commands in ANSI (VT220 and VT100) mode,

Chapter 4.2  list the commands in VT52 mode,

Chapter 4.3 describes the output generated by each key on the keyboard.

4.1.  ANSI (VT-220) COMMANDS

4.1.1.  CONTROL FUNCTIONS

The control functions are used to make the terminal  perform special  actions such as editing data, query terminal status etc.  There are single-character and multiple-character control functions.

CONTROL CHARACTERS

The single-character control functions are the C0 and C1 control characters. The C0 control characters are in range 00 to 1F (hex) and the C1 control characters in range 80 to 9F (hex). The C1 characters can be used directly only in 8-bit environment. In 7-bit environment, the 7-bit code extensions of the C1 control characters must be used. The control codes are listed in tables 4.1 and 4.2.

CONTROL SEQUENCES

There are three basic types of multiple-character control functions:  escape sequences, control sequences and device control strings.

An escape sequence uses two or more bytes to define a control function. Escape sequences do not include variable parameters. The format of an escape sequence is:

ESC I F

where ESC (hex 1B) introduces the sequence, I is zero or more intermediate characters in range 20 to 2F hex, and F is the final character in range 30 to 7E hex.

The control sequence uses two or more bytes to define a control function. Control sequences usually include variable parameters. The general format for a control sequence is:

CSI P...P I...I F

Where CSI is the Control Sequence Introducer character ( 9B in 8-bit environment or 1B 5B in 7-bit environment).

P...P  is parameter string, which may contain zero or more numeric parameters, separated by semicolons (;). Each parameter is a positive integer of up to four digits. If the first character in the parameter string is in range 3C to 3F ('<' to '?'), it indicates that private parameters follow.

I...I  are zero or more intermediate characters in range 20 to 2F hex.

F  is the final character in range 40 to 7E hex.

The command is executed after the final character has been received. If the sequence contains an error, it is ignored and, if the error display is enabled, an error character (checkerboard) is displayed.

The device control strings (DCS), like control sequences, use two or more bytes to define specific control functions. However, it also includes a data string. The format is:

DCS P...P I...I F Data_string ST

DCS is the device control string introducer. It is either an C1 control character 90 hex, or equivalent 7-bit sequence ESC P (hex 1B 50).

P...P  is parameter string, as in the control strings above.

I...I  are zero or more intermediate characters in range 20 to 2F hex.

F  is the final character in range 40 to 7E hex. The intermediate and final characters define the control function. All the characters after final character are interpreted as the data string.

Data_string contains the data definitions. It may contain any number of bytes, depending on the function it is defining.

ST is the string terminator, which indicates the end of the string. It is either an C1 control character 9C hex or equivalent 7-bit sequence  ESC \ (hex 1B 5C).

Any undefined device control strings are ignored. None of the characters between the DCS and ST are displayed.

On the following pages, each control sequence is explained in detail. using following format:  __________________________________________________________________________________

Nr	Name									Mnem

ASCII				HEX

Description

__________________________________________________________________________________

Where Nr is the number of the command (for reference only), name is the name of the command, mnem is the ANSI mnemonic for the command, ASCII is the command sequence in the ASCII format, HEX is the sequence in hexadecimal format.  After these is the description of the command.

The hex code for CSI is shown as 7-bit code extension (Esc [, hex 1B 5B). If 8-bit environment is selected, it may be replaced with the 8-bit code (hex 9B).

Pn, Pn1, Pn2 etc. represent  a numeric parameter. It is used directly as numeric value.

Ps, Ps1, Ps2 etc. represents a selective parameter. It is a numeric value that selects the action of the commad.

Commands are sorted in functional groups. Commands available only in the VT220 mode are marked with an asterisk (*).

Table 4.1:  C0 (ASCII) CONTROL CODES:

hex  	ascii 	key 		function

00	NUL 	Ctrl-@	      	No effect

01	SOH	Ctrl-A		Start of header (network command)

05	ENQ	Ctrl-E	Send answerback message

07	BEL	Ctrl-G		Sound bell

08	BS	Ctrl-H 		Backspace

09	HT 	Ctrl-I  or  TAB  	Tabulation

0A	LF 	Ctrl-J     		Line feed

0B	VT 	Ctrl-K  		Treated as LF

0C	FF 	Ctrl-L       		Treated as LF

0D	CR 	Ctrl-M  or Return  	Carriage return

0E 	SO 	Ctrl-N      	 	Shift Out, select G1 char set

0F	SI 	Ctrl-O       	Shift In,  select G0 char set

11	DC1	Ctrl-Q       	XON  (restart transmission)

12	DC2	Ctrl-R		Enable transmission (network command)

13 	DC3 	Ctrl-S       		XOFF (halt transmission)

18 	CAN	Ctrl-X       	Cancel escape sequence

1A	SUB	Ctrl-Z       		Cancel escape sequence

1B	ESC 	ESC          		Start escape sequence

Table 4.2:  C1  CONTROL CODES (8-bit):

Equivalent

7-bit code

hex    mnem.	extension	Name	function

84	IND	Esc D	Index 	Move cursor down. Scroll at bottom.

85	NEL	Esc E	Next line	Move cursor to beginning of next line.

88	HTS	Esc H	Horizontal	Sets one horizontal tab stop at current

tab set 	 cursor location.

8D	RI	Esc M	Reverse	Move cursor up one line. If at top

index	  margin, scroll down.

8E	SS2	Esc N	Singleshift2	Selects G2 set for next character only

8F	SS3	Esc O	Singleshift3	Selects G3 set for next character only

90	DCS	Esc P	Device Control String introducer

9B	CSI	Esc [	Control Sequence Introducer

9C	ST	Esc \	String Terminator

4.1.2.	C U R S O R  C O N T R O L

__________________________________________________________________________________

1.  CURSOR UP   						CUU

CSI Pn A   			1B 5B Pn 41

This sequence moves the cursor Pn (any decimal number) lines up. If the cursor is attempted to move above the top margin, the cursor will stop at the top margin. No scroll down will occur. If the  Pn  is not given,  the default is 1.

__________________________________________________________________________________

2.  CURSOR DOWN  						CUD

CSI Pn B   			1B 5B Pn 42

This sequence moves the cursor Pn lines down. The cursor will not move below the bottom   margin. No scroll up will occur. If Pn is not given, the default is 1.

__________________________________________________________________________________

3.  CURSOR FORWARD  					CUF

CSI Pn C   			1B 5B Pn 43

This sequence moves the cursor Pn columns to the right. The cursor will stop at the rightmost   column. If Pn  is not given, the default is 1.

__________________________________________________________________________________

4.  CURSOR BACKWARD  					CUB

CSI Pn D   			1B 5B Pn 44

This sequence moves the cursor Pn of columns to the left. The cursor will stop at the leftmost   column. If Pn is not given, default is 1.

_________________________________________________________________________________

5.  CURSOR POSITION  					CUP

CSI Pn1 ; Pn2  H  		1B 5B Pn1 3B Pn2 48

This sequence will position the cursor at the Pn1 line in the Pn2 column (respectively). Line   and column numbers must be separated by a semi-colon. Parameters not given defaults to line or   column 1. Parameters outside the range of the screen or buffer are ignored.

See also command 43 (Cursor position report).

__________________________________________________________________________________

6.  CURSOR HOME 						CUP

CSI H   			1B 5B 48

This function moves the cursor to the Home position.

__________________________________________________________________________________

_________________________________________________________________________________

7.  INDEX  						IND

ESC D   			1B 44

or		IND			84

This sequence moves the cursor one line down in the same column position. If the cursor is in the last   line, a scroll up will occur.

__________________________________________________________________________________

8.  NEXT LINE						NEL

ESC E   			1B 45

or		NEL			85

This sequence moves the cursor to the first (left-most) position on the next line down. If the cursor is in   the bottom margin, a scroll up will occur.

__________________________________________________________________________________

9.  REVERSE INDEX  						RI

ESC M   			1B 4D

or		RI			8D

This sequence moves the cursor one line up in the same column. If the cursor is in the topmost line, a   scroll down will occur.

__________________________________________________________________________________

10.  SAVE CURSOR  						SC

ESC 7 			1B 37

This sequence causes the cursor position, attributes (graphic rendition, newline enable, wrap around, cursor on/off ) and character set selection to be saved for later use.

__________________________________________________________________________________

11.  RESTORE CURSOR					RC

ESC 8   			1B 38

This sequence restores the previously saved cursor position, attributes and character set.

__________________________________________________________________________________

12.  CURSOR ON/OFF  					DECTCEM

To set: 	CSI ? 2 5 h   		1B 5B 3F 32 35 68

To reset:	CSI ? 2 5 l   		1B 5B 3F 32 35 6C

When set, the cursor is displayed normally.  When reset, cursor is not displayed.

__________________________________________________________________________________

__________________________________________________________________________________

13.  CURSOR BLINK / STEADY					-

To set: 	CSI ? 3 4 h   		1B 5B 3F 33 34 68

To reset:	CSI ? 3 4 l   		1B 5B 3F 33 34 6C

When set, selects the blinking cursor. When reset, selects non-blinking cursor.

__________________________________________________________________________________

14.  CURSOR LINE / BLOCK 					-

To set:	CSI ? 3 3 h   		1B 5B 3F 33 33 68

To reset:	CSI ? 3 3 l   		1B 5B 3F 33 33 6C

When set,  selects block type cursor. When reset, selects underline type cursor.

__________________________________________________________________________________

4.1.3.	I N S E R T   &   D E L E T E

__________________________________________________________________________________

15.  DELETE CHARACTER					DCH

CSI Pn P  			1B 5B Pn 50

Deletes Pn characters on a single line, starting with the character at the cursor position. Any   characters remaining on the line  to the right of the last deleted character are moved Pn spaces  left, to fill the gap created by the deletion.  If Pn is not given, it defaults to 1.

__________________________________________________________________________________

16. INSERT CHARACTER					ICH	*

CSI Pn @  			1B 5B Pn 40

Inserts Pn  blank characters at the current cursor position. All characters to the right of the cursor at the same line are moved Pn spaces right.  If Pn is not given, it defaults to 1.

__________________________________________________________________________________

17. DELETE LINE						DL

CSI Pn M  			1B 5B Pn 4D

Deletes Pn complete lines from the display, starting with the line which contains the cursor. Remaining lines below cursor to the bottom margin move up to fill the gap, and Pn blank  lines are added to the bottom of screen or scrolling region.  If Pn is not given, it defaults to 1.

__________________________________________________________________________________

18. INSERT LINE						IL

CSI Pn L  			1B 5B Pn 4C

Creates a blank area of Pn lines by moving all lines from the cursor line to bottom margin down by Pn  lines.

__________________________________________________________________________________

19. INSERT / REPLACEMENT MODE				IRM

Insert mode:	CSI 4 h 		1B 5B 43 68

Replace mode:	CSI 4 l 		1B 5B 43 6C

In insert mode, each new character received is inserted at the cursor position and all characters from the cursor position to the end of line are shifted one place to the right.

In replace mode, the character received replaces the character at current cursor position.

__________________________________________________________________________________

__________________________________________________________________________________

20. ERASE IN DISPLAY					ED

CSI Ps  J			1B 5B Ps 4A

This sequence erases certain parts of the display screen according to the parameter Ps and the   cursor location. If Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the screen.

1	Erases from the start of the screen to the cursor.

2	Erases the entire screen.

__________________________________________________________________________________

21. ERASE IN LINE						EL

CSI Ps K			1B 5B Ps 4B

This sequence erases certain parts of the current line according to the parameter Ps and the cursor location. If Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the line

1	Erases from the start of the line to the cursor

2	Erases the entire line

__________________________________________________________________________________

22. ERASE CHARACTERS					ECH	*

CSI Pn X 			1B 5B Pn 58

Erases Pn characters starting from the current cursor position by replacing them with blank spaces. The cursor position is not moved.

__________________________________________________________________________________

23. SELECTIVE ERASE IN DISPLAY					DECSED	*

CSI ? Ps  J			1B 5B 3F Ps 4A

This sequence erases some or all of the erasable characters in the display, according to the parameter Ps and the cursor location. Characters that are defined as protected characters by DECSCA command are not erased. By default, all characters are erasable. If the parameter Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the screen.

1	Erases from the start of the screen to the cursor.

2	Erases the entire screen.

__________________________________________________________________________________

__________________________________________________________________________________

24. SELECTIVE ERASE IN LINE					DECSEL	*

CSI ? Ps K			1B 5B 3F Ps 4B

This sequence erases some or all of the erasable characters on the current line, according to the parameter Ps and the cursor location. Characters that are defined as protected characters by DECSCA command are not erased.  If the parameter Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the line

1	Erases from the start of the line to the cursor

2	Erases the entire line

__________________________________________________________________________________

25. SELECT CHARACTER PROTECTION ATTRIBUTE			DECSCA	*

CSI Ps " q			1B 5B Ps 22 71

This command defines successive characters written to display as protected or unprotected. Protected characters are not erased by DECSED and DECSEL commands. By default, all characters are erasable (not protected).

Ps	Resulting action

0	Not protected (default)

1	Protected

2	Not protected

__________________________________________________________________________________

4.1.4.	T A B U L A T I O N

__________________________________________________________________________________

26. SET TAB STOP						HTS

ESC H   			1B 48

or		HTS			88

This sequence sets one horizontal tab stop at the current cursor location.

__________________________________________________________________________________

27. CLEAR TAB STOPS					TBC

CSI Ps g 			1B 5B Ps 67

This sequence clears tab stops as specified by the parameter Ps:

Ps	Resulting action

0		Clear tab stop at current cursor column

3		Clear all tab stops

If Ps is not given, it defaults to 0.

__________________________________________________________________________________

28. TABULATION FORWARD					HT

HT   			09

The cursor is moved to the right, to the next tab stop on the same line. Does not wrap around to the  next line.

__________________________________________________________________________________

29. TABULATION BACKWARD

ESC B   			1B 42

The cursor is moved left to the first tab stop backwards, on the same line. If the cursor is in the first   column of the line, it will not move.

__________________________________________________________________________________

4.1.5.	C H A R A C T E R   A N D   L I N E   A T T R I B U T E S

__________________________________________________________________________________

30. SELECT GRAPHIC RENDITION					SGR

CSI Ps m 			1B 5B Ps 6D

This sequence selects the character video attributes.  Ps  is a string of parameters, separated by semi-colons. Each parameter selects an attribute:

Ps	Result

0	Attributes off  (normal text)

1	Bold

4	Underlined

5	Blinking

7	Reverse video

22	Bold off *

24	Underline off  *

25	Blinking off *

27	Reverse video off *

Parameter options marked with an asterisk (*) are available on the VT220 mode only.

All characters following the sequence are displayed with the attributes selected, until the next SGR sequence. Ps can be any number of attributes, separated by semi-colons. If no parameter is given, Ps defaults to 0, turning all attributes off.

Example:

ESC [ 0 ; 1 ; 4 m

The above sequence clears all previous attributes, then sets bold and underline attributes.

On color displays, this command can also be used to select display colors. In that case, only either reverse or blink attribute is available (depending on system configuration). Other attributes are used to change color.  You can also change the color with ANSI-PC compatible parameter values:

Ps	Result

31	Red

32	Green

33	Yellow

34	Blue

35	Magenta

36	Cyan

37	White

Note:  Depending on the system configuration, some of the colors may also cause blink attribute to be selected. If blinking is not required, it can be turned off with BLINK RATE command.  See page 4-Error: Reference source not found.

On red-green EL display, the color selection is different to the full color display. The attrubutes selected cause the following colors to be displayed:

(1)	(4)	(5)

Bold	Underline	 Blink		Resulting color

-	  -	  -	Yellow

ON	  -	  -	Orange

-	ON	  -	Green

ON	ON	  -	Green,  half brightness

-	  -	ON	Red

ON	  -	ON	Red,  half brightness

-	ON	ON	Greenish Yellow

ON	ON	ON	Yellow,  half brightness

Inverse attribute (7) works normally and causes characters to be black and the background to be selected color.

Example:  to select greenish yellow color on red-green EL display, enter command

ESC [ 0 ; 4 ; 5 m

__________________________________________________________________________________

_______________________________________________________________________________

31. DOUBLE HEIGHT LINE - TOP					DHLT

ESC # 3   			1B 23 33

This sequence causes the cursor line to become the top half of a double-width, double-height line. If the line was single-width, all characters to the right of the center of the screen are lost. If the cursor is located to the left of the screen center, it will remain in the same character position. If the cursor is to  the right of the screen center, it is moved to the rightmost column of the line.

__________________________________________________________________________________

32. DOUBLE HEIGHT LINE - BOTTOM				DHLB

ESC # 4   			1B 23 34

This sequence causes the line to become the bottom half of a double-width, double-height line. NOTE: the ESC # 3 and ESC # 4 sequences are to be used in conjunction with two equal lines of text to produce one double-width, double-height line. Double-width limits a line to 40 characters.

__________________________________________________________________________________

33. SINGLE-WIDTH LINE					SWL

ESC # 5   			1B 23 35

This sequence causes the cursor line to become single-width, single-height  (default character size).

__________________________________________________________________________________

34. DOUBLE-WIDTH LINE					DWL

ESC # 6   			1B 23 36

This sequence causes the cursor line to become a double-width line. Double-width limits a line to 40 characters. If the line was single-width, all characters to the right of the center of the screen are lost.

__________________________________________________________________________________

4.1.6.	C H A R A C T E R   S E T S

Text and semigraphic symbols are displayed on screen by sending their corresponding 8-bit character code (single byte) to AP988200. With 8 bits, only limited number of different characters can be displayed. There are 256 possible character codes available with 8 bits. Values 0 to 31 and 128 to 159 (decimal) are reserved for control characters. This leaves only 192 possible codes for displayable characters.

More characters can be displayed by using different character sets. At any point, you can select the character set you are using and write text with that character set. You can then select another character set and continue writing with the new character set. This way, you can write much more than 192 different symbols on screen.

There are 14 build-in character sets (hard character sets) in AP988200. In addition, you can download your own characters sets (soft character sets).  Each character set contains 96 characters. Two character sets can be active simultaneously: GL  and GR sets. GL set is used for character codes 32 to 127 (hex 20 to 7F), GR set is used for character codes 160 to 255 (hex A0 to FF).

§

Character sets are selected by following these two steps:

1.	Designate the character sets you need to G0, G1, G2 and G3 character generators.

G0 through G3 are logical sets AP988200 uses to access character sets. Up to four

character sets can be designated.

2.	Map the designated set into GL or GR set. You can switch between the four

designated sets easily by just using the mapping commands.

By default, ASCII set is mapped to GL set and DEC supplemental set to GR set. Together, these sets make up the DEC Multinational set, which can be used to display text in several languages. The commands for designating and mapping character sets are described on the following pages.

__________________________________________________________________________________

35.  DESIGNATING CHARACTER SET				SCS

To select G0 set:	ESC ( Dscs 		1B 28 Dscs

To select G1 set:	ESC ) Dscs 		1B 29 Dscs

To select G2 set:	ESC * Dscs 		1B 2A Dscs *

To select G3 set:	ESC + Dscs		1B 2B Dscs *

These sequences are used to designate the G0, G1, G2 and G3 character generators at the hard character sets. For each character generator, the character set is selected by parameter Dscs.  The following character sets are available:

Dscs		Character set

B		ASCII character set

0		DEC special graphics character set

1		IBM PC extended character set   

<		DEC supplemental character set *

National replacement sets:  

A		British

4		Dutch

5 or C	Finnish

R		French

K		German

Y		Italian

6 or E	Norwegian

Z		Spanish

7 or H	Swedish

=		Swiss

*	G2 and G3 character sets can be designated only in VT220 emulation mode.

DEC multinational character set is available only in VT220 mode.

	IBM character set is not a standard VT220 feature.

Note: on IBM character set, double height characters are not available.

	Only one national replacement set is available at a time. If the character set is changed from one national set to another, all national characters on screen are changed to that of new national set. Note: National character set commands are effective only when NATIONAL MODE is set (see DECNRCM on page 4-28)

In addition to the character sets above, this sequence is also used to designate any soft font downloaded to terminal. The soft font name (Dscs) is defined in the download string.

Examples:

The following sequence designates the special graphic character set as the G1 logical set:

ESC ) 0

The following sequence designates the DEC supplemental character set as the G3 logical set:

ESC + <

__________________________________________________________________________________

__________________________________________________________________________________

36. MAPPING THE CHARACTER SETS

After you designate the character set as G0, G1, G2 or G3, you must map the set into the in-use table as GL or GR using the locking-shift or single-shift control functions. GL defines the character set to be used for characters in range 20 to 7F (hex), GR defines the character set to be used for characters in range A0 to FF (hex).

Locking-Shifts:

When you use a locking shift, the character set remains in GL or GR until you use another locking shift. The table below lists all locking shifts available:

Sequence

Name		ascii	Hex	Function

LS0		SI	0F	Map G0 into GL  (default)

LS1		SO	0E	Map G1 into GL

LS1R		ESC ~	1B 7E	Map G1 into GR *

LS2 		ESC n	1B 6E	Map G2 into GL *

LS2R		ESC }	1B 7D	Map G2 into GR  (default) *

LS3		ESC o	1B 6F	Map G3 into GL *

LS3R		ESC |	1B 7C	Map G3 into GR *

Single-Shifts:

A single shift maps the G2 or G3 set into GL. The character set is active for only one character. Then the terminal returns to the previous character set in GL. Two single-shift control functions are available:

8-bit	7-bit equivalent sequence

Name 	character	ASCII	Hex	Function

SS2		8E	ESC N	1B 4E	Maps G2 into GL for next character *

SS3		8F	ESC O	1B 4F	Maps G3 into GR for next character *

*  Sequences marked with an asterisk are available on the VT220 mode only.

__________________________________________________________________________________

Default mapping of character generators:

After power-on reset, the character generators are mapped as follow:

Character	Character		Mapped

generator	set		to

G0		National or ASCII	GL

G1		ASCII		 -

G2		DEC supplemental	GR

G3		Semigraphic		 -

__________________________________________________________________________________

S O F T   C H A R A C T E R   S E T S

In addition to the build-in character sets, you may design your own character sets and download them to the terminal. You can download full character sets, or replace just one or few characters on the existing character sets.  (A special program, Alpha-CHED, is available from Alpha Point for creating and downloading soft character sets.)

You may use VT220 or VT320 character sets on the AP988200, but because the character cell sizes are different on these terminals, the characters may appear different than in the original environment.

The character cell size varies depending on the display panel used as follows:

640.400	640.350	640.200	512.256	320.256

Cell width		8	8	8	7	8

Cell height		16	14	8	10	10

Body width		7	7	7	6	7

Body height		9	9	7	7	7

Ascender height		3	3	2	2	2

Descender height		2	2	1	2	2

Spacing after character	1	1	1	1	1

Spacing above character	2	1	0	1	1

Spacing below character	3	2	0	2	1

While downloading, each character is divided into sixtels. A sixtel is a 6-bit binary value that represents a vertical column of 6 pixels on the screen. For each bit in sixtel, value 1 represents a pixel that is lit and value 0 pixel that is not lit. Eight adjacent sixtels horizontally forms a group. The character downloaded contains up to three groups.  See the picture below.

The 8 by 16 pixel character cell shown above consists of 3 groups. On the last group, only the 4 least significant bits are used. The least significant bit of the first sixtel is the upper left corner of the character.

__________________________________________________________________________________

37. DOWNLOAD SOFT CHARACTERS				      DECDLD

DCS Pfn ; Pcn  ; Pe ; Pcms ; Pw ; Pt  { Dscs Sxbp1 ; ... ; Sxbpn ST

The soft characters are downloaded using the above device control string, where

DCS	=	Device control string introducer (hex 90). You can use the equivalent 7-bit sequence ESC P (hex 1B 50) in the 7-bit environment.

Pfn	=	Font buffer number:     0 or 1

Pcn	=	Starting character, 0 to 95 (ascii-code - 20h)

Pe	=	Erase control:

0  =  Erase all characters in the character set before downloading

1  =  Erase only characters in locations being reloaded

Pcms	=	Character matrix size:

0  =  device default  (8 x 16)

1  =  illegal

2  =  5 x 10 pixel cell  (VT220)

3  =  6 x 10 pixel cell  (VT220)

4  =  7 x 10 pixel cell  (VT220)

Pw	=	Font width:

0 = device default  (80 columns)

Pt	=	Text or full cell

0 = device default  (text)

1 = text  - characters are auto-centered to the cell

2 = full cell  - all pixels addressable

Dscs	=	Character set name, in the format

I F

where

I	= 0, 1 or 2 intermediate characters in range 20 to 2F hex,

F	= the final character in range 30 to 7E hex.

The character set name is used when designating the character set with SCS command.

Sxbp1 ; ... ; Sxbpn

=	The sixtel bit patterns for individual characters, separated by semi-colons (hex 3B). 1 to 96 patterns can be sent in one DECDLD string. Each sixtel bit patern is in the following format:

S...S/S...S/S...S

Where the first S...S represents the sixtels in Group A of the character (eight bytes typ.),  / (hex 2F) advances the sixtel pattern to Group B, etc.

Each sixtel is coded as ascii character, whose code is the bit pattern of the vertical row of six pixels + 3Fh. The uppermost bit is the least significant bit.

ST	= The string terminator (hex  9C). You can also use  ESC \  (hex 1B 5C).

4.1.7.	S C R E E N   C O N T R O L   C O M M A N D S

__________________________________________________________________________________

38. SET SCROLLING REGION					SCAR

CSI Pn1 ; Pn2  r 		1B 5B Pn1 3B Pn2 72

This sequence sets the top and bottom margins for the scrolling region. After setting the region, only the lines inside the margins will be scrolled. Also, the cursor may not be moved outside the region by means of cursor up/down commands.

The parameter Pn1 refers to the top line and Pn2 to the bottom line of the scrolling region. The numbers are line numbers, ranging from 1 to 24. The minimum permitted size of the region is two lines, in which  case Pn1 is the uppermost line of the region, and Pn2  is Pn1 + 1.

Upon setting the scrolling region, the cursor will move to the home position, depending on the setting of Origin Mode. If Origin Mode is set, the cursor will move to the upper left corner of the scrolling region. If origin mode is reset, the cursor moves to line 1, column 1, regardless of where the scroll area is.

__________________________________________________________________________________

39. SET LEFT AND RIGHT MARGINS					-

CSI Pn1 ; Pn2 v 		1B 5B Pn1 3B Pn2 76

This sequence sets the left and right margins. It is not possible to move the cursor outside these margins by means of CURSOR MOVEMENT commands, including cursor Left/Right, TAB, Line Feed and Return. The minimum size of the vertical region between the margins is 2 columns. The default margin columns are 1 to 80, i.e. the complete  screen.

Note:  This is not a standard VT220 command.

__________________________________________________________________________________

40. ORIGIN MODE						OM

To set:	CSI ? 6 h   		1B 5B 3F 36 68

To reset:	CSI ? 6 l  		1B 5B 3F 36 6C

When set, the origin (Home position) is at the upper left character position within the scrolling region.  When reset, the origin is at the upper left corner of the screen.

Once margins have been defined and the scrolling mode set,  the line and column numbers are relative to those margin settings. The cursor is not allowed to be positioned outside those margins.

The cursor is moved to the new Home position when this sequence is received. Lines and columns are numbered consequentially, with the origin (Home) being line 1, column 1 upper left position.

__________________________________________________________________________________

__________________________________________________________________________________

41. SELECT ACTIVE STATUS DISPLAY				DECSASD	*

CSI Ps $ }			1B 5B Ps 24 7D

This function selects whether the terminal sends data to the main display or the status line. The main display  is the first 24 lines on the screen. The status line is the 25th line.

The parameter Ps selects the display the data is sent to:

Ps	Display selected

0	Main display

1	Status line

All displayable text after this command string is directed to the display window selected, until next DACSASD command string is received. If parameter Ps is not given, it defaults to 0. Writing to the status line is possible only if host writeable status line  is selected using the DECSSDT command below.

All normal cursor control, display attribute etc. commands work on the status line. However, the row address is always ignored. When the main display is selected, the cursor position and attributes are restored. The status line is common to all display pages.

Note:  this is a VT320 command not found on the standard VT220 terminal.

__________________________________________________________________________________

42. SELECT STATUS LINE TYPE					DECSSDT	*

CSI Ps $ ~			1B 5B Ps 24 7E

The parameter Ps selects the type of status line displayed on the screen:

Ps	Status line type

0	None

1	Indicator

2	Host writeable

After power on or reset, the Indicator status line is selected.

Note:  This is a VT320 command not found on the standard VT220 terminal.

__________________________________________________________________________________

__________________________________________________________________________________

43. NEXT PAGE						NP

CSI Pn U			1B 5B Pn 55

Moves Pn pages forward in the display memory. Contents of the next page is instantly displayed, and all output is directed to that page. Cursor is moved to the home position (line 1, column 1).

There are total of 4 pages available. If trying to move past fourth page, wrap around to page 1 occurs. If display page and write page are set to different values using PPA command, the display page is first set equal to write page. If parameter Pn is not given, it defaults to 1.

Each page has its own margin settings and origin mode flag. Status line, if enabled, is common to all pages.

__________________________________________________________________________________

44. PREVIOUS PAGE						PP

CSI Pn V			1B 5B Pn 56

Moves Pn pages backward in the display memory. Cursor is moved to the home position. If trying to move beyond the page 1, wrap around to page 4 occurs. If parameter Pn is not given, it defaults to 1.

Note:  NP and PP are ANSI / VT320 commands not found on the standard VT220 terminal.

__________________________________________________________________________________

45. PAGE POSITION ABSOLUTE					PPA	*

CSI Pd ; Pw sp P		1B 5B Pd 3B Pw 20 50

Selects page Pd for display and Pw for output. Cursor remains on the same position as at the previous page. Both page numbers must be in range 1 to 4. The page defined by Pd is displayed on the screen. All subsequent output is directed to the page defined by Pw. It is thus possible to write to a hidden page and then instantly display the new page.

If either parameter is zero, the corresponding page selection is not changed. If only one parameter is given, the display page and write page will be set to the same value.

For fast animation, you should change the display page only. This is faster than changing both display page and write page.

Example:  To select page 3 for display, without changing the write page, use the following command sequence (sp = space character):

CSI 3 ; 0 sp P

Note:	This is not a VT220 command. VT320 terminal allows this command with single parameter only.

_________________________________________________________________________________

4.1.8.	S E T   M O D E

__________________________________________________________________________________

46. KEYBOARD ACTION MODE					KAM

To set:		CSI 2 h 		1B 5B 32 68

To reset:	CSI 2 l 		1B 5B 32 6C

When set, the keyboard is locked and the Wait indicator is lit on the keyboard. No key codes are transmitted to the host. Since the keyboard is locked, the keyboard unlock command has to come from the host.

__________________________________________________________________________________

47. SEND - RECEIVE MODE					SRM

To set:		CSI 1 2 h   		1B 5B 31 32 68

To reset:	CSI 1 2 l   		1B 5B 31 32 6C

When set, the characters sent to the host are not echoed to the screen locally. The host must echo the characters back to the terminal, if they are to appear on the screen (Full duplex).

When reset, the characters sent to the host are also echoed locally on the screen (Half duplex).

This command has exactly the same effect as selecting Full / Half Duplex on the Setup screen.

__________________________________________________________________________________

48.  LINE FEED - NEW LINE MODE					LNM

To set:		CSI 2 0 h   		1B 5B 32 30 68

To reset:	CSI 2 0 l   		1B 5B 32 30 6C

When set, the terminal is in new line mode. A carriage return (CR, hex 0D) is interpreted as a carriage return and a line feed, positioning the cursor in the first column of the next line.

When reset, the terminal is in line feed mode. A carriage return is interpreted as a carriage return only, positioning the cursor in column 1 at the current line.

__________________________________________________________________________________

49. KEYPAD APPLICATION MODE				      DECKPAM

ESC =  			1B 3D

Set numeric keypad to application mode. Control sequences are transmitted instead of numeric key codes. See chapter 4.3 for sequences sent.

__________________________________________________________________________________

50. KEYPAD NUMERIC MODE

ESC > 			1B 3E

Set numeric keypad to numeric mode. ASCII-codes marked on keycaps for numeric keys are transmitted.  __________________________________________________________________________________

__________________________________________________________________________________

51. CURSOR KEY MODE					CKM

To set:		CSI ? 1 h 		1B 5B 3F 31 68

To reset:	CSI ? 1 l 		1B 5B 3F 31 6C

This sequence determines the escape sequences transmitted by the cursor arrow keys. When reset, the normal cursor control sequences are sent. When set, special sequences are sent. See below for the sequences sent:

Cursor key	When set	When reset

Up Arrow	ESC O A	CSI A

Down Arrow	ESC O B	CSI B

Right Arrow	ESC O C	CSI C

Left Arrow	ESC O D	CSI D

__________________________________________________________________________________

52. ANSI MODE						     DECANM

To reset:	CSI ? 2 l 		1B 5B 3F 32 6C

This command causes terminal to exit ANSI (VT100 or VT220) mode and enter the VT52 mode, exactly as if VT52 option was selected from the SETUP. In VT52 mode, the terminal executes special VT52 command set, listed in the chapter 4.2. Also, some of the function- and special keys send VT52 strings instead of VT100 or VT220 strings (see chapter 4.3). The character set sent by terminal is restricted to 7-bit sets.

__________________________________________________________________________________

53. SCROLLING MODE					     DECSCLM

To set: 		CSI ? 4 h 		1B 5B 3F 34 68

To reset: 	CSI ? 4 l 		1B 5B 3F 34 6C

When set, smooth scroll is selected. When reset, jump scroll is selected.

__________________________________________________________________________________

54. SCREEN MODE						     DECSCNM

To set:		CSI ? 5 h 		1B 5B 3F 35 68

To reset:	CSI ? 5 l 		1B 5B 3F 35 6C

When set, the whole screen is displayed in inverse video.

When reset, the screen is displayed in normal video.

__________________________________________________________________________________

__________________________________________________________________________________

55. WRAP AROUND						WRP

To enable: 	CSI ? 7 h   		1B 5B 3F 37 68

To disable:	CSI ? 7 l  		1B 5B 3F 37 6C

When enabled, any displayable character received when the cursor is at the right margin causes the cursor to advance to the beginning of the next line, performing a scroll up if required.

When disabled, any displayable character received when the cursor is at the right margin overwrites any previous character at this position. The cursor will not move to the next line until an explicit carriage return / line feed is received.

__________________________________________________________________________________

56. AUTO REPEAT MODE					DECARM

To set	 	CSI ? 8 h   		1B 5B 3F 38 68

To reset:	CSI ? 8 l  		1B 5B 3F 38 6C

When set, the key repeat is on.

When reset, the key repeat is off.

__________________________________________________________________________________

57. NATIONAL REPLACEMENT CHARACTER SET MODE	 		DECNRCM

To set : 	CSI ? 4 2 h   		1B 5B 3F 34 32 68

To reset: 	CSI ? 4 2 l  		1B 5B 3F 34 32 6C

When the national mode is set, the terminal sends 7-bit characters from the national replacement character set currently active (selected by LANGUAGE on the SETUP mode) and uses national replacement character set to display the 7-bit characters on the screen. National mode restricts the characters terminal can send to those available in the selected NRC set.

When reset, the terminal is in multinational mode, and uses 8-bit characters from USASCII set and DEC supplemental character set.

The default state after power-on is set by the NAT.MODE field on the setup screen.

Setting this mode resets all the character sets to their default (power-on) states.

Note: Unlike VT220 terminal, AP988200 can receive and display 8-bit characters even if the national mode is selected.

__________________________________________________________________________________

58. KEYBOARD USAGE MODE					DECKBUM

Data processing: 	CSI ? 6 8 h   		1B 5B 3F 36 38 68

Typewriter:	CSI ? 6 8 l  		1B 5B 3F 36 38 6C

In data processing mode, the terminal sends key codes marked on the right side of the keycaps.

In typewriter mode, the key codes marked on the left side of the keycaps are used.

If North American keyboard is in use, this command has no effect.

__________________________________________________________________________________

__________________________________________________________________________________

59. SELECT OPERATING LEVEL				     DECSCL

The operating level of the terminal is selected by using one of the following control sequences:

CSI 6 1 " p 	1B 5B 36 31 22 70	VT100 mode  (Level 1)

CSI 6 2 " p	1B 5B 36 32 22 70	VT200 mode , 8-bit C1 codes (Level 2)

CSI 6 2 ; 0 " p	1B 5B 36 32 3B 30 22 70	VT200 mode , 8-bit C1 codes (Level 2)

CSI 6 2 ; 1 " p	1B 5B 36 32 3B 30 22 70	VT200 mode , 7-bit C1 codes (Level 2)

In VT100 mode, some of the commands are ignored (marked with an asterisk after the mnemonic of each command) and some of the function keys have different meaning.

Optional second parameter defines the transmission of 8-bit (C1) control codes when some special or function key is pressed. If the second parameter is 1, the C1 codes are converted to 7-bit equivalents. Otherwise, 8-bit codes are used.

Note: you may also change the operating level from the SETUP screen by changing the field EMULATION.  See chapter 3.3.

__________________________________________________________________________________

60. SELECT 7-BIT C1 CHARACTERS			     	S7C1T	*

ESC sp F    			1B 20 46

After receiving this command, the terminal sends the C1 control codes to host as 7-bit escape sequences.

__________________________________________________________________________________

61. SELECT 8-BIT C1 CHARACTERS			     	S8C1T	*

ESC sp G    			1B 20 47

After receiving this command, the terminal sends the C1 control codes to host as 8-bit single byte characters.

__________________________________________________________________________________

62. ERROR CHARACTER DISPLAY					   -

To disable:	CSI > 0 E   		1B 5B 3E 30 45

To enable:   	CSI > 1 E   		1B 5B 3E 31 45

If the error character display is enabled, the terminal displays an error character (checkerboard), when it receives an escape sequence that contains an error. If the error character display is disabled, the erroneous sequences are just ignored. After power-on or reset, the error character display is disabled.

Note: This is an Alpha Point enhanced command not found on standard VT220 terminal.

__________________________________________________________________________________

4.1.9.	U S E R - D E F I N E D   K E Y S

There are 20 function keys on the top row of the keyboard.  15 of them can be defined by user:

F6 through F14, Do, Help and F17 through F20 (shifted).  Keys F1 through F5 and all unshifted function keys  have fixed functions and can not be programmed.

There are 256 bytes of memory available for the 15 user-defined keys. When the 256 bytes are full, you can not define any more until you clear some of the old definitions.

After power-up, all UDKs hold an empty string.

__________________________________________________________________________________

63.  DEFINE KEYS						       DECUDK	*

DCS Pc ; Pl | Ky1 / St1 ; ... ; Kyn / Stn ST

90 Pc 3B Pl 7C Ky1 2F St1 3B ... 3B Kyn 2F Stn 9C

The function key definitions are downloaded by using the above device control string, where

DCS	=	Device control string introducer, hex 90. You can also use ESC P (hex 1B 50) in 7-bit environment.

Pc		= 	Clear parameter:

0  =  Clear all keys before loading new values (default)						1  =  Clear only the keys that are loaded

Pl		=	Locking parameter:	0 or 1.  (Currently not used.)

|		=	The final character, hex 7C.

Ky1 / St1 ; ... ; Kyn / Stn  are the key definition strings, where

Kyn	=	The key selector number:

Key       Value	Key       Value

F6	17	F14	 26

F7	18	Help	 28

F8	19	Do	 29

F9	20	F17	 31

F10	21	F18	 32

F11	23	F19	 33

F12	24	F20	 34

F13	25

Stn	=	String parameter, holds the key definition encoded in hex. Each byte to be programmed to the key is coded as 2-byte hex value. This allows any character, including control characters to be programmed to function keys.

ST	=	String terminator, hex 9C. You can also use ESC \ (hex 1B 5C) in 7-bit environment.

Each key definition string consist of a key selector number (Kyn) and a string parameter (Stn), separated by slash (/, hex 2F).  One or more key definition strings may be sent on a single device control string. Different key definition strings are separated with semi-colons (hex 3B). The String terminator ends the sequence.

Example:

DCS 1 ; 1 | 3 4 / 4 5 5 8 4 9 5 4 0 D ST

The above device control string defines the F20 key to send a string "EXIT", followed by carriage return. The other function keys are left unchanged.

After you define a key, you can use the new function by pressing Shift key together with the key you defined.

__________________________________________________________________________________

4.1.10.	R E S E T   A N D   T E S T   C O M M A N D S

__________________________________________________________________________________

64. RESET TERMINAL					RIS

ESC c  			1B 63

This command resets the terminal to power up default states and clears all the display pages. Sets the character sets to default, erases the attributes to default and puts cursor to Home position.

After issuing this command, the host computer should wait the command to be executed (about 0.5 seconds), since characters received from host during the reset operation are lost.

Note: using this command is not recommended. Use Soft Terminal Reset and Clear Screen instead.

__________________________________________________________________________________

65. SOFT TERMINAL RESET					DECSTR	*

CSI ! P  			1B 5B 21 70

Resets terminal to power up default states. Does not clear the screen.

__________________________________________________________________________________

66. SCREEN ALIGNMENT DISPLAY					DECALN

ESC # 8  			1B 23 38

This sequence fills the entire display screen with upper case E's for screen testing.

__________________________________________________________________________________

4.1.11.	R E P O R T   C O M M A N D S

__________________________________________________________________________________

67.  DEVICE ATTRIBUTES REPORT					DA

CSI Pn  c			1B 5B Pn 63

or		ESC Z			1B 5A

By sending one of the above sequences, the host request the device attributes report (Terminal Identification) from the terminal. The first sequence is recommended. For report request, the parameter Pn must either zero or omitted (no parameter). The terminal responds by sending a sequence depending on the emulation mode selected and the options installed. Currently, following sequences are sent:

In VT100 mode:

CSI ? l ; 2 c  			1B 5B 3F 31 3B 32 63

In VT220 mode:

CSI ? 6 2 ; 2 ; 7 ; 8 ; 9 c  		1B 5B 3F 36 32 3B 32 3B 37 3B 38 3B 39 63

meaning:  level 2 terminal (62) with printer port (2), soft character sets (7), user defined keys (8) and national replacement character sets (9).

__________________________________________________________________________________

68.  SECONDARY DEVICE ATTRIBUTES REPORT			DA	*

CSI > Pn c   			1B 5B 3E Pn 63

With this DA sequence,  the host requests the terminal's identification code, firmware version and hardware options. Parameter Pn must be zero, or it may be omitted. The terminal responds with the following sequence:

CSI > Pp ; Pv ; Po c   		1B 5B 3E Pp 3B Pv ; Po 63

Where:

Pp	= Identification for the terminal. For VT220, the id code is 1.

Pv	= Firmware version  (two digits, without comma)

Po	= Hardware options

Example:

CSI > 1 ; 1 2 ; 0 c	= VT220, version 1.2, no options.

__________________________________________________________________________________

__________________________________________________________________________________

69.  CURSOR POSITION REPORT					CPR

CSI 6 n   			1B 5B 36 6E

When this DSR sequence is received, the cursor position report (CPR) is sent to the Host in the format

CSI Pl ; Pc R   		1B 5B Pl 3B Pc 52

Where the line and column numbers are represented by Pl  and Pc, respectively.

__________________________________________________________________________________

70.  EXTENDED CURSOR POSITION REPORT				DECXCPR	*

CSI ? 6 n   			1B 5B 3F 36 6E

When this DSR sequence is received, the extended cursor position report, including the current page number,  is sent to the Host in the format

CSI Pl ; Pc  ;Pp R   		1B 5B Pl 3B Pc 3B Pp 52

Where Pl and Pc are current line and column, Pp is the current page number (1 to 4).

__________________________________________________________________________________

71.  DEVICE STATUS REPORT					DSR

CSI Ps n   			1B 5B Ps 6E

Both the device status request from the host and the device status report from the ELT-220 use the  same format. The parameter Ps indicates the status:

Ps		Result

0		Response from the terminal:  All is ready, no errors detected.

3		Response from the terminal:  Error detected, please try again.

5		Request from the Host.

__________________________________________________________________________________

__________________________________________________________________________________

72.  PRINTER STATUS REPORT					DSR

CSI ? 1 5 n   			1B 5B 3F 31 35 6E

With this sequence, the host asks for the status of the printer connected to the terminal's printer port. The status is checked by testing the DTR or CTS handshake line on the printer port (depends on the settings of the on-board jumpers). The terminal responds with one of the following reports:

Sequence	Meaning

CSI ? 1 0 n	Printer ready.

CSI ? 1 1 n	Printer not ready.

CSI ? 1 3 n	No printer. Printer has not been ready since the last power-up or reset.

__________________________________________________________________________________

73.  USER DEFINED KEYS					DSR	*

CSI ? 2 5 n   			1B 5B 3F 32 35 6E

the host asks if the user-defined keys (UDKs) are locked or unlocked. The terminal responds with one of the following reports:

CSI ? 2 0 n	UDKs are unlocked.

CSI ? 2 1 n 	UDKs are locked.

__________________________________________________________________________________

74.  KEYBOARD DIALECT					DSR

CSI ? 2 6 n   			1B 5B 3F 32 36 6E

The host asks for the current keyboard dialect. The terminal responds with following sequence:

CSI ? 2 7 ; Pla n   		1B 5B 3F 32 37 Pla 6E

where Pla indicates the keyboard dialect:

Pla	Dialect		Pla	Dialect

1	North American		10	Swiss (French)

2	British		11	Swiss (German)

5	Danish		12	Swedish

6	Finnish		12	Norwegian

7	German		14	French

8	Dutch		15	Spanish

9	Italian

__________________________________________________________________________________

__________________________________________________________________________________

75.  LOCATOR DEVICE STATUS REPORT				DSR

CSI ? 5 5 n   		1B 5B 3F 35 35 6E

With this sequence, the host asks for the status of the locator device. The terminal responds with one of the following reports:

Sequence	Meaning

CSI ? 5 3 n   	No locator device.

CSI ? 5 0 n   	Locator device ready.

__________________________________________________________________________________

76.  LOCATOR DEVICE ID					DSR

CSI ? 5 6 n   		1B 5B 3F 35 36 6E

With this sequence, the host asks what kind of locator device is connected to the controller. The terminal responds with one of the following reports:

Sequence	Meaning

CSI ? 5 7 ; 0 n   	Can not identify the locator device.

CSI ? 5 7 ; 1 n   	Locator device is Digital's mouse.

CSI ? 5 7 ; 2 n   	Locator device is Digital's tablet.

CSI ? 5 7 ; 4 n   	Locator device is Alpha Point optical Touch Screen Overlay.

CSI ? 5 7 ; 8 n   	Locator device is membrane switch Touch Screen Overlay.

CSI ? 5 7 ; 14 n   	Locator device is external optical Touch Screen Overlay.

This report does not indicate the presence of locator device. Before using the locator device, the application should check the locator device status.

__________________________________________________________________________________

4.1.12.	P R I N T E R   C O M M A N D S

Following commands are used to control the interaction between the terminal and the printer.

__________________________________________________________________________________

77.  PRINT LINE						MC

CSI ? 1 i 			1B 5B 3F 31 69

Media Copy:  Send the line containing the cursor  to the printer.

__________________________________________________________________________________

78.  PRINT PAGE						MC

CSI i   			1B 5B 69

Media Copy:  Send the contents of the screen to the printer.

__________________________________________________________________________________

79.  ENTER AUTO-PRINT MODE					MC

CSI ? 5 i  			1B 5B 3F 35 69

Media Copy:  enter auto print mode. In the auto-print mode, the contents of cursor line is sent to the printer whenever a Line Feed is  performed.

__________________________________________________________________________________

80.  EXIT AUTO-PRINT MODE					MC

CSI ? 4 i  			1B 5B 3F 34 69

This command terminates the Auto-print Mode.

__________________________________________________________________________________

81.  ENTER PRINTER CONTROLLER MODE				MC

CSI 5 i   			1B 5B 35 69

Media Copy: enter printer controller mode. In printer controller mode the terminal will send all received characters to the printer without displaying them. None of the data is interpreted as a terminal command except the Exit Printer Controller Mode  command.

__________________________________________________________________________________

__________________________________________________________________________________

82.  EXIT PRINTER CONTROLLER MODE				MC

CSI 4 i     			1B 5B 34 69

This command is detected by the terminal when in the Printer Controller Mode and causes the   controller to display all subsequent data instead of printing it. No character of this command is passed to the printer.

__________________________________________________________________________________

83.  PRINT FORM FEED MODE					   DECPFF

To enable:	CSI 1 8 h   		1B 5B 31 38 68

To disable:	CSI 1 8 l   		1B 5B 31 38 6C

When enabled, the terminal adds a Form Feed character (hex 0C) to the end of the data sent  to the printer after each PRINT PAGE command.  When disabled, no form feed is added.

__________________________________________________________________________________

84.  PRINT EXTENT						 DECPEX

To set:		CSI 1 9 h  		1B 5B 31 39 68

To reset:	CSI 1 9 l  		1B 5B 31 39 6C

When this function is set, any subsequent PRINT PAGE command will print the entire   screen contents.

When reset, the PRINT PAGE command prints the contents of the scrolling region (data inside the top and bottom margins defined by SCAR command).

__________________________________________________________________________________

4.1.13.	S E T   T E R M I N A L   P A R A M E T E R S

__________________________________________________________________________________

85.  SET LINE PARAMETERS						-

CSI baud ; parity ; databits ; stopbits; handshake ; duplex e

This command allows host computer to set the serial line communications parameters of the terminal. Minimum of two parameters must be given. The parameters are as follows:

baud: 				stopbits:

0  =	no change 			0  =	no change

1  =	19200 bps               	    	1  =	0 start/stop bits

2  = 	  9600 bps          	         	2  =	1 bit

3  =	  4800 bps            	       	3  =	(reserved)

4  =	  2400 bps           	       	4  =	2 bits

5  =	  1200 bps

6  =	    600 bps                   	    	handshake:

7  =	    300 bps  			0  =	no change

1  =	XON/XOFF

parity: 				2  =	CTS

0  =	no change			3  =	(reserved)

1  =	no parity 			4  =	none

2  =	odd parity

3  =	even parity 			duplex:

0  =	no change

databits: 	     		1  =	full duplex

0  =	no change 			2  =	half duplex

1  =	8 bits

2  =	7 bits

Example:

CSI 5 ; 0 ; 0 ; 0 ; 1 ; 0 e

Selects 1200 bps, xon-xoff protocol without changing other parameters.

Warning: After changing the line parameters, the line parameters of the host computer must be changed accordingly. Otherwise it may be impossible to communicate between the host and the  terminal

Note:  This is an Alpha Point enhanced command not found on standard VT-220 terminal.

__________________________________________________________________________________

__________________________________________________________________________________

86. SCREEN SAVER TIME						-

CSI > Pn S			1B 5B 3E Pn 53

Set the screen saver time constant, just as the SCR SAV option in the SETUP. Parameter Pn defines the time, in minutes, after which the screen is blanked, if there is no activity on the terminal. Pn may be any value in range 1 to 255, but if it is not one of the values available in the SETUP, the time is not displayed correctly in the SETUP screen.

__________________________________________________________________________________

87. SCREEN SAVER MODE						-

CSI > 1 h			1B 5B 3E 31 53 68

Sets the screen saver mode, i.e. blanks the screen just as if the screen saver time had been passed. The screen is restored, when a character is received from the host, keyboard or locator device (TSO etc.).

__________________________________________________________________________________

4.1.14.  T O U C H   S C R E E N   C O M M A N D S

__________________________________________________________________________________

88.  SET TSO MODE   						-

To set:		CSI > Ps h   		1B 5B 3E Ps 68

To reset:	CSI > Ps l   		1B 5B 3E Ps 6C

This sequence selects the Touch Screen Overlay (TSO) operating mode. Parameter list Ps may contain one or more of the following parameters:

Ps	Mode, when set

14	Report only on query

15	TSO enabled

16	Debug mode  (send also incomplete coordinate pairs)

17	Exit report mode  (send ?? ?? message, when the finger is removed)

18	Sound click on report

20	Double resolution mode  (using interpolation)

21	NET mode  (insert net address to message)

22	Single character mode  (get character code from tsotable in EPROM)

Multiple modes may be set or reset by giving multiple parameters, separated by semi-colons. If the single character mode is set, the double resolution mode and net mode are ignored.

After power-on or reset, all modes are reset, except TSO enable, which depends on tso enable jumper on-board, and click mode, which is set by default. The default TSO modes may also be set by patching the EPROM location TSOMODE.

__________________________________________________________________________________

89.  TSO TIMING							-

CSI > Pn1 ; Pn2 ; Pn3 T   		1B 5B 3E Pn1 3B Pn2 3B Pn3 54

Set TSO timing, according to the parameters:

Param	Explanation	Allowed values	          Reset value

Pn1		Repeat delay,	1 to 255.  			9

Pn2		Repeat rate, 	1 to 255,  0 = no repeat.	 	3

Pn3		Tracking speed	1 to 255,  0 = no tracking mode	1

Parameters Pn2 and Pn3 are optional. If parameter Pn3, or both Pn2 and Pn3 are omitted, these values are not changed. The table above gives parameters for the optical TSO. For membrane TSO, Pn3 sets the scan rate.

The reset values may be changed by patching the EPROM locations TSODEL, TSORAT and TSOSPD.

Note:  these two commands only affect Alpha Point optical TSO or Membrane TSO connected to the TSO interface of AP988200. For external third party TSO, see the external TSO command on next page.

__________________________________________________________________________________

__________________________________________________________________________________

90.  SEND COMMAND TO EXTERNAL TSO				-

DCS Pn t  c1 c2 ... cn ST  		1B 50 Pn 74 c1 c2 ... cn 1B 5C

This device control string is used to send commands to external locator device connected e.g. to the keyboard interface of the terminal. All characters after the final character (t) are sent directly to the external device, until the String Terminator (ST) is reached.

DCS	=	Device control string introducer (hex 90). You can use the equivalent 7-bit sequence ESC P (hex 1B 50) in the 7-bit environment.

Pn	=	Selects the i/o port for the locator device.

0  = 	Device default  (VT220 keyboard port)

1  =	VT220 keyboard port

2  =	IBM-PC keyboard port  (if installed)

t	=	The final character. All characters after this are sent to the external device.

c1 c2 ... cn

=	Characters sent to external device. May be any value except ST.

ST	= The string terminator (hex  9C). You can also use ESC \ (hex 1B 5C).

See your locator device manual for interpreting of the command characters c1 ... cn.

Programming hint:  When this command is entered from another terminal, you may give any number of commands to the locator device from your keyboard and examine the results immediately on screen. Enter Esc \  to end the host-to-device session.

Note:  This is not a standard VT220 command. Use of this command is dependent on the system configuration.

__________________________________________________________________________________

See also:  Locator Device Status and Locator Device ID reports.

4.1.15.  M U L T I P O I N T   C O M M U N I C A T I O N S

__________________________________________________________________________________

91.  SELECT NET TARGET  					-

SOH a a ... a STX   		01 a a ... a 02

This sequence is used to select receiver(s) of any following messages on the multipoint communication systems (e.g. terminal network using the optional RS485 interface).

After this command, only the terminals listed in the string will receive the characters transmitted through the serial line, until next SOH ... STX header is received. The net address for each terminal is selected from the NET ADDR field on the SETUP screen.

Any number of terminals may be selected by entering any number of address bytes (a) in the header string. Each address is a single byte coded as net address + 30H. Addresses may vary in the range 1 to 79, thus, each address byte in the string may be in range 31H to 7EH (ASCII character '1' to '~'). A special address zero (30h, ASCII '0') is used as broadcast address. After that, all the terminals will receive the messages (the default state).

If the first character after SOH is '+', the following addresses are appended to the list of active terminals. Otherwise, the list replaces the old list.

Examples:

SOH 2 STX		- Selects terminal No. 2

SOH 1 5 A STX		- Selects terminals 1, 5 and 17

SOH + 7 8 STX		- Adds terminals 7 and 8 to list of the receivers.

__________________________________________________________________________________

92. 	ENABLE TRANSMISSION					-

DC2 a  		12 a

This sequence enables transmission for given terminal on the network (selective XON). Only the selected terminal is able to send anything on the serial line.

Parameter a, which selects the transmitting terminal, is a single byte coded as net address + 30h, and may vary in range 30 to 7F hex, '0' to '~' ASCII. Address '0' selects all the terminals. Any value below 30h disables all the terminals from sending. A space (20H) is recommended.

If 'Report Only On Query' mode is selected using the TSO MODE command (parameter 14), the terminal immediately sends the last TSO coordinate report. No further coordinate reports are sent, until the next DC2 sequence is received. This may be used to poll the TSO messages from each terminal.

__________________________________________________________________________________

4.1.16.  E X T R A   C O M M A N D S

The commands listed in this section depend on system configuration and may not be available on all firmware versions.

__________________________________________________________________________________

93.  BLINK RATE 							-

CSI > Pn b   		1B 5B 3E Pn 62

Sets the blink rate for characters with blink attribute:

Pn	Blink rate

0	No blinking

1	Fast blink

2	Normal blink rate

3	Slow blink

Option to turn blinking off is useful especially for color displays, where blink attribute may have been used for color selection.

__________________________________________________________________________________

94.  ZOOM 						-

CSI > Pn1 ; Pn2 z   		1B 5B 3E Pn1 3B Pn2 7A

Sets the display zoom factor, i.e. enlarge characters on the whole screen. Parameters Pn1 and Pn2 select the zoom factor:

Pn1	= Horizontal zoom factor:

0 or 1	Zoom factor 1  (normal width)

2	Zoom factor 2  (double width)

Pn2	= Vertical zoom factor:

0 or 1	Zoom factor 1  (normal height)

2	Zoom factor 2  (double height)

If a parameter is not entered, default value of 0 is assumed.

When the horizontal zoom factor is set to 2, only the first 40 columns of each line are visible. Double wide characters are displayed in quadruple wide (20 characters / line).

When the vertical zoom factor is set to 2, only the first 12 lines are visible, and there is no status line. Double height characters are displayed in quadruple height.

The cursor can be moved and data entered within the whole 24 by 80 character screen area. When the normal screen size is restored, all data on this screen area becomes visible again.

Note:  This is not a standard VT220 command.

__________________________________________________________________________________

### 4.2.  VT52 ESCAPE SEQUENCES

__________________________________________________________________________________

1. CURSOR ADDRESSING					CUA

ESC Y row col    		1B 59 row col

Position the cursor a the address given by row and col where row and col must be two ASCII characters representing the line and column numbers respectively. The character code for row and col is calculated by adding 1Fh to the line or column number. Row or column 1 is thus represented by space (20h), etc.

__________________________________________________________________________________

2. CURSOR BACKWARD					CUB

ESC D			1B 44

Moves cursor one column to the left.

__________________________________________________________________________________

3. CURSOR DOWN						CUD

ESC B			1B 42

Moves cursor down one line.

__________________________________________________________________________________

4. CURSOR FORWARD					CUF

ESC C			1B 43

Moves cursor one column to the right.

__________________________________________________________________________________

5. CURSOR HOME						CUH

ESC H			1B 48

Moves cursor to home position (upper left corner of the screen).

__________________________________________________________________________________

6. CURSOR UP						CUU

ESC A			1B 41

Moves cursor up one line.

__________________________________________________________________________________

__________________________________________________________________________________

7. REVERSE LINE FEED					RVLF

ESC I			1B 49

Moves cursor up one line. If at the top of the screen, scroll down is performed.

__________________________________________________________________________________

8. CLEAR SCREEN						CRS

ESC J			1B 4A

Erases from cursor to end of screen.

__________________________________________________________________________________

9. ERASE LINE						EL

ESC K			1B 4B

Erases from cursor to end of line.

__________________________________________________________________________________

10. SPECIAL GRAPHICS					GRF

ESC F			1B 46

Selects special graphics character set. All subsequently received data will be mapped to the special graphics characters.

__________________________________________________________________________________

11. EXIT SPECIAL GRAPHICS					ASCII

ESC G			1B 47

Returns to ASCII character set, if special graphics set was selected using the above command.

__________________________________________________________________________________

12. AUTO PRINT MODE					APM

To set:	ESC ^		1B 5E

To reset: 	ESC _		1B 5F

When set, the current line is printed (transmitted out the printer port) following every line feed operation.

__________________________________________________________________________________

13. CHANGE MODE						CM

ESC <			1B 3C

Exits the VT52 mode and returns terminal to VT100 or VT220 mode, whichever was active when the VT52 mode was entered.

__________________________________________________________________________________

__________________________________________________________________________________

14. KEYBOARD APPLICATION MODE				KAM

ESC =			1B 3D

Sets the numeric keypad to application mode. In application mode the keypad sends special escape sequences instead of the characters marked on the keycaps. These escape sequences are different from those in VT220 mode. See chapter 4.3.

__________________________________________________________________________________

15. RESET KEYBOARD APPLICATION MODE				RKAM

ESC >			1B 3E

Returns numeric keypad to normal mode, where each key sends the values marked on the keycaps.

__________________________________________________________________________________

16. TERMINAL IDENTIFICATION					TID

ESC Z			1B 5A

The host request terminal to identify itself. In VT52 mode the terminal responds with the sequence:

ESC / Z			1B 2F 5A

__________________________________________________________________________________

### 4.3.  TRANSMITTED CODES

This chapter describes the codes transmitted to host computer, when a key is pressed on keyboard. VT220 type keyboard is assumed. On IBM-PC-keyboard, some keys may have different markings on keycaps and some keys may not be available. Most keys, however, work equally on both keyboards.

Main keypad Function Keys:

Key			Code transmitted

Õ			DEL character

Tab			HT character

¿  Return  	CR character only or CR and LF, depending on the state

of line feed / new line mode (LNM).

Ctrl			Does not send code. Used in conjunction with other keys

to generate control characters.

Lock			Does not send code. Used to toggle caps lock on / off.

Shift			Does not send code. Used in conjunction with other keys

to generate capital letters or special symbols.

Space bar		SP character.

Compose		Does not send a code. Used in special compose character

character		sequences.

Editing Keys:	(VT220 mode only)

Key			Code transmitted

Find			CSI 1 ~

Insert Here	CSI 2 ~

Remove		CSI 3 ~

Select		CSI 4 ~

Prev Screen	CSI 5 ~

Next Screen	CSI 6 ~

Cursor Control Keys:

ANSI mode *)		VT52 mode

Cursor key mode:

Key			Normal	Application

ñ	(up)		CSI A	SS3 A		ESC A

ò	(down)	CSI B	SS3 B		ESC B

ð	(right)	CSI C	SS3 C		ESC C

ï	(left)	CSI D	SS3 D		ESC D

*) ANSI mode applies to VT220 and VT100 modes.  VT52 mode is an ANSI-incompatible mode.

Auxiliary Keypad Keys:

ANSI mode			 VT52 mode

Keypad mode:		Keypad mode:

Key			Numeric       Application		Numeric	Application

0			  0	SS3 p		0	ESC ? p

1			  1	SS3 q		1	ESC ? q

2			  2	SS3 r		2	ESC ? r

3			  3	SS3 s		3	ESC ? s

4			  4	SS3 t		4	ESC ? t

5			  5	SS3 u		5	ESC ? u

6			  6	SS3 v		6	ESC ? v

7			  7	SS3 w		7	ESC ? w

8			  8	SS3 x		8	ESC ? x

9			  9	SS3 y		9	ESC ? y

-			 -  (minus)	SS3 m		-	ESC ? m

,			 ,  (comma)	SS3 l.		,	ESC ? l

.			 .  (period)	SS3 n		.	ESC ? n

Enter		 CR	SS3 M		CR 	ESC ? M

or CR LF *)			or CR LF *)

PF1			 SS3 P	SS3 P		ESC P	ESC P

PF2			 SS3 Q	SS3 Q		ESC Q	ESC Q

PF3			 SS3 R	SS3 R		ESC R	ESC R

PF4			 SS3 S	SS3 S		ESC S	ESC S

*)  In numeric keypad mode, ENTER generates the same code as RETURN. You can change the code generated by RETURN with the line feed / new line mode. When reset, only CR is transmitted. When set, CR and LF are transmitted.

Top Row Function Keys:

Code generated:

VT52 and

Legend          	Name		VT220 mode	VT100 modes

Hold Screen	(F1) *		-	-

Print Screen	(F2) *		-	-

Set-Up		(F3) *		-	-

Data/Talk	(F4) *		ESC	-

Break		(F5) *		(break signal) **	(break signal) **

F6			F6		CSI 1 7 ~	-

F7			F7		CSI 1 8 ~	-

F8			F8		CSI 1 9 ~	-

F9			F9		CSI 2 0 ~	-

F10			F10		CSI 2 1 ~	-

F11			F11		CSI 2 3 ~	ESC

F12			F12		CSI 2 4 ~	BS

F13			F13		CSI 2 5 ~	LF

F14			F14		CSI 2 6 ~	-

Help			(F15)		CSI 2 8 ~	-

Do			(F16)		CSI 2 9 ~	-

F17			F17		CSI 3 1 ~	-

F18			F18		CSI 3 2 ~	-

F19			F19		CSI 3 3 ~	-

F20			F20		CSI 3 4 ~	-

*	F1 through F3 and F5 are local function keys and do not generate codes.  F4 sends an ESC  (the Data/Talk function is currently not used).

**	Break signal is generated by holding host serial line at low state for 270 milliseconds.

The above strings are sent, when function key is pressed unshifted. If function key is pressed simultaneously with SHIFT key, user downloaded function key strings are sent. If no function key definitions are downloaded, nothing is sent for shifted function keys. (For defining function keys, see page 4-30.)

Alphanumeric Keys:

Each of the alphanumeric keys send the codes marked on the keycaps. The code for each key varies among the different national versions of the keyboard.

Many of the national keyboard versions have different symbols marked on the left and right side of the keycap. Which symbol is sent, depends on the Typewriter / Data Processing mode selection. See chapter 3.3 (SETUP) and chapter 4.1 (DECKBUM command, page 4-28).