2.  INSTALLATION

2.1  CONFIGURATION OPTIONS

The default configuration of the AP988200 controller board contains:

- Display panel interface for the display type selected

- Host serial interface, RS232C.

- Serial printer interface, RS232C

- Optical Touch Screen Overlay interface

- Membrane type Touch Screen Overlay interface (requires special EPROM)

- VT220 keyboard interface

- On-board beeper

- On-board DC/DC converter for RS232C voltages

- On-board Switched Mode Power Supply, to generate the 5V operating voltage

for both the AP988200 and  the display panel

- CMOS RAM with battery backup

- Alphasoft-200 EPROM for the selected display type and resolution

Optionally, the AP988200 controller board may include:

- IBM-AT compatible keyboard interface

- RS485 or Current Loop for host serial interface

- Common-connector for many of the interface signals

- Power LED indicator

- Customized Alphasoft-200 version

- Stripped-down versions of the AP988200 are also available. The board may be manufactured without the power-supply module and connector module to fit in smaller space.

Optional accessories:

-  Display cable for the selected display type (EL, Plasma, etc.)

-  Host serial cable

-  Printer cable

-  Mechanical assembly accessories

-  Keyboard

-  An AC power supply

(Input: 115 or 230 VAC,  Output: 12 VDC, 3 Amps).

2.2  ELECTRICAL CONNECTIONS

Fig. 2.1:  I/O connectors

When installing the AP988200 in your system, the following electrical connections are needed. (Some of the connections are optional, depending on your system requirements.)

1.	Connect the power supply to the Power Input -connector (PK2). Do not switch the power on until all the connections have been made. The operating voltage may be in range 12 to 24 volts, depending on the display type used (check your display panel data sheet for correct voltage). The 5V operating voltage is generated by an on-board Switched Mode Power Supply (SMPS).  The AP988200 uses only the 5V voltage, and also supplies 5V to the display panel. However, the VT220 keyboard and TSO use the 12V supply. If these peripherals are used with a 24V power supply, an additional 12V supply is needed.

If the Power Supply Module is not installed, the 5V voltage must also be supplied. In this case, the operating voltages are connected to connector PK1.

Some display panels, such as plasma panels, may require additional operating voltages. Those voltages must be connected externally to the display panel.

2.	Connect the display cable from the display connector (K9) to the display panel. Make sure that the pin1 of the cable connector is connected to the pin 1 of the connector on the PCB.

3.	Connect the host serial cable to connector K6. A flat cable with IDC connectors may be used to connect to an 9-pin D connector. The pin order is compatible with a standard 9-pin serial port connector. The AP988200 may act either as a Data Terminal Equipment (DTE) or as Data Communications Equipment (DCE), as selected by jumpers J11 and J12. You must be sure to use correct type of cable (direct or crossed wire), depending on the type of host serial port (DTE or DCE). Only three wires need to be connected: the data lines (TxD, RxD) and ground. If the hardware handshake is used, the handshake lines (CTS, RTS, DTR) may also be needed. See chapter 6 for pin descriptions.

4.	Connect the serial printer to port K4 (if required). Set printer to operate at 4800 bps, 8 bit, no parity. Select the DTR handshake input pin, using jumper J14, to suit your printer. The printer must supply hardware handshake; the Xon-Xoff handshake is not supported.

5.	Connect the VT220 compatible keyboard (if required) to connector K6 (a 4-pin telephone type connector), or an IBM-AT compatible keyboard to connector K7 (a 5-pin DIN connector). A serial ASCII keyboard may also be used on connector K6 (requires special EPROM version). WARNING: The VT220 keyboard uses the 12V power supply. If you are using other than 12V power supply voltage, an additional 12V regulator is needed for the keyboard.

6.	Connect Touch Screen Overlay (TSO), if required. Optical TSO is connected to connector K2. Membrane TSO or switch matrix type keyboard is connected to connector K1. Only one type of TSO may be connected at a time. The standard Alphasoft-200 firmware contains support for the optical TSO. Membrane TSO requires a special version of the Alphasoft-200 EPROM.

See chapter 6.3 for the pin descriptions for each connector.

2.3  MECHANICAL ASSEMBLY

The AP988200 is designed to fit directly on the back of Finlux MD640.200, MD640.350 and MD640.400 display panels. For other display types, some additional accessories may be needed to assemble the board.

Finlux MD640.200, MD640.350 or MD640.400:

Fit the AP988200 to the Finlux display panel using four screws at position B (see fig. 2.2). A short flat cable may be used for connecting the AP988200 display output to the display input of the display panel power supply unit.

The AP988200 may also be fitted to a display panel that does not have the aluminium profile frame. This may be necessary, if the assembly space is very limited. In this case, use the screw positions marked A.

Other Finlux EL-panels:

Some additional mechanical parts may be needed to fit controller board to different sized panels.

Sharp EL-panels with detachable dc/dc converter:

Remove the dc/dc converter from the back of the display panel. Fit the AP988200 to the display panel using four screws at position B. Fit the dc/dc converter to its original position on top of the AP988200 using long screws.

Fig 2.2:  Screw positions

2.4  TESTING THE INSTALLATION

After you have gone through chapters 2.2 - 2.3 of the installation successfully, turn ON the power of the AP988200 terminal.

You should get a cursor at the upper left corner of the display and a status line at the bottom of the display with yellow background. (Note, however, that on some special versions of the firmware, either the cursor or status line may be disabled.)

In order to the system to work properly, correct serial port parameters need to be set. The parameters may be set from the keyboard by using the SETUP mode, see chapter 3.3. If your system configuration does not have the keyboard, and the standard serial port parameters (see chapter 3.3)  are not sufficient, the setup parameters has to be set by patching the EPROM.

If the keyboard is available, press function key F3 in the upper row of the keyboard to invoke the SETUP-mode. (On other than VT220 keyboard, some other key may be used as setup-key. See your keyboard specific application note. If the keyboard does not have the setup-key, the SETUP-mode may be invoked by pressing Ctrl-Shift-@ twice.)

Use the SETUP-menu according to chapter 3.3 instructions to modify the host communications and AP988200 local parameters to match the requirements of your host hardware/software. Set the serial parameters so that they match the settings on the host computer. Also make sure that your host computer has been set up to recognize the AP988200 either as a VT220-, VT100- or VT52 type terminal.

If you are using a VT220 compatible keyboard, make sure that the KEYBOARD field in the SETUP-menu contains the proper setting for your national keyboard. If the setting is not correct, use the up/down cursor keys in your keyboard to select the correct national setting. If you are using other keyboard type (ASCII-keyboard, IBM-AT keyboard or matrix keyboard connected to TSO interface), the settings for the keyboard must be installed in the EPROM.

Once you are satisfied with the SETUP parameter settings, press the F3-key once again to change from SETUP mode into the ON-LINE mode and to save the settings in nonvolatile memory.

Now you are ready to login to your host application. For using the actual application and/or the operating system services of your host computer, you need the reference manuals that have been supplied by your hardware/software vendor.