6.  HARDWARE DESCRIPTION

6.1.  ON-BOARD PERIPHERALS

6.1.1.  BEEPER

The Bell sound is generated by the build-in beeper of the VT220 keyboard. The AP988200 controller board also has an on-board beeper, which generates the bell sound, when the keyboard is not connected. The on-board beeper also generates a key-click for the Touch Screen Overlay.

6.1.2.  POWER SUPPLY

The on-board SMPS (Switched Mode Power Supply) generates the 5 VDC operating voltage for both the AP988200 display controller and the display panel. The 12 V input voltage is not used internally by the AP988200. Instead of 12V, you may use any voltage required by the display panel, up to 40 VDC. If required, the SMPS may be omitted and the 12V and 5V voltages supplied externally.

Warning:  Both VT220 keyboard and TSO frame board use the 12V operating voltage. If higher supply voltage is used and one of above devices is connected, an additional regulator must be used for the input devices.

The +12V and -12V voltages for the RS232C interface are generated on-board by the RS232C interface chip.

Fig. 6.1:  AP988200 printed circuit board

6.2.  JUMPER SETTINGS

The following jumper options are available on-board. The default setting are underlined.

Mode Select  jumper block  (J10):

o o o o o o o o

J10	o o o o o o o o

Bit position:	7 6 5 4 3 2 1 0

Position	explanation				Open		Closed

Bit 0.	(reserved)				     -		     -

Bit 1.	IBM-PC keyboard test mode		Normal		Scan code

Bit 2.	(reserved)				     -		     -

Bit 3.	TSO y-coord. inverse			Normal		Inverse

Bit 4.	TSO enable				Disabled	Enabled

Bit 5.	RS485 mode				Disabled	Enabled

Bit 6.	Battery back-up				Enabled		Disabled

Bit 7.	Display power-up screen			No		Yes

It the TSO (Touch Screen Overlay) is not in use, the J10 position 4 should be left open. Otherwise false coordinates may be generated. TSO scanning also consumes some CPU time.

The origin point of the TSO is normally the lower left corner of the screen. If J10 position 3 is closed, the origin point is at the upper left corner of the screen.

The RS485 mode should be enabled, if multipoint RS485 interface is used. When point-to-point connection (RS232C or RS422) is used, this jumper should be left open.

The Battery back-up disable jumper may be used, when a new EPROM is changed on the AP988200 board. Closing this jumper ensures that new SETUP defaults are read from the EPROM. After that, the jumper may be removed. The default values can also be read from EPROM by pressing Ctrl-D at the SETUP mode.  (If battery backup is not needed, the battery may  be removed and replaced with a short circuit over battery terminals.)

Note:  The J10 jumper block is read by software only at power-up or reset.

Other jumpers:

Pins connected:

Jumper	explanation				  1 to 2		2 to 3

J1	LCA reset input select			  (manual)	system

J2	LCA load data input			  external	CPU

J3	LCA external load input connector		(open)

J4	LCA load clock input			  external	CPU

J5	LCA load mode select			  external	CPU

J6	Char Gen RAM size			  32K		8K

J8	Host port data input select		  RS232C	Current loop or RS485

J9	Host port DTR output, force high	  (open)

J21	EPROM type				  27512		27256

The jumpers J1 to J9 are set at factory, and should not be changed by the user.  J21 may need to be changed, when upgrading the Alphasoft-200 EPROM.

Jumper	explanation	1	2	3	4	5

J7	Host port CTS/DCD input select:

a)  Disabled	o----o    o    o----o					b)  CTS	o----o    o----o    o

c)  DCD	o    o----o    o----o

If hardware (CTS) handshake is to be used, jumper J7 should be set according to option b). If the hardware handshake is not used,  J7 should be set according the option a), or connection should be made on the cable to set the handshake line always ON. The handshake output is enabled by using SETUP screen. See also J11 and J19.

Jumper	explanation	pins connected

J13	Printer port TxD pin select	1 to 2 = DTE,  2 to 3 = DCE

J15	IBM PC keyboard reset enable	open = disabled

J16, J17	Display connector pin configuration	open = EL,  closed = PG3

J18	Display connector Blank pin select	1 to 2 = GND,   2 to 3 = enabled

J19	Host port CTS input pin select	1 to 2 = DCD,   2 to 3 = CTS

J22	Watch dog enable  *)	open = disabled

J23	Display drive voltage 2 (25V)	open

DTE		  DCE

Jumpers J11 and J12 can be used to set		J11:	1 o---o 2		1 o   o 2

the RS232C  pin configuration to either			3 o---o 4		3 o   o 4

DTE or DCE. The default setting is DTE.		J12:	1 o---o 2		1 o   o 2

3 o---o 4		3 o   o 4

J14  Printer port handshake input select:		K4 pin 	D-sub

Select one of the four jumper positions to		J14:	1 o---o 2		6	( 8 )

choose the hardware handshake pin suitable			3 o---o 4		7	( 4 )

for your printer. (See also J13.)				5 o---o 6		2	( 6 )

7 o---o 8		4	( 7 )

*)  Ask for special EPROM version for watch dog support.

6.3.  INTERFACES

K1  -  MEMBRANE T.S.O. INTERFACE

Membrane switch type Touch Screen Overlay input device (TSO) is connected to this connector. You may also connect an undecoded matrix keyboard here. A special version of the Alphasoft-200 EPROM is required for this interface.

The membrane TSO is a matrix of contacts, each of which connects one of the column lines to one of row lines. The AP988200 scanns the TSO matrix by supplying low level voltage to one of the 12 column lines at a time and then reading the 8 row lines.

Note: If there is need for multiple keys to be pressed simultaneously (shift or control keys, etc.), isolation diodes must be connected at each of the column lines.

Enable the TSO by connecting jumper J10 bit 4.

Membrane TSO specifications:

Max. resolution (matrix size):		8 by 12

Contact resistance, max:			2  kOhm

Connector K1:  20 pin single line pin array:

oooooooooooooooooooo

1                  20

1	Row 7		 9	Column 0	 17	Column 8

2	Row 6		10	Column 1	 18	Column 9

3	Row 5		11	Column 2	 19	Column 10

4	Row 4		12	Column 3	 20	Column 11

5	Row 3		13	Column 4

6	Row 2		14	Column 5

7	Row 1		15	Column 6

8	Row 0		16	Column 7

K2  -  OPTICAL T.S.O. INTERFACE

Optical TSO (AP988030/A, AP988030/B or AP988030/BB) is connected to this interface. The TSO is enabled by connecting the jumper J10.4. The AP988200 then performs the scanning of the TSO and sends coordinates to the host serial port.

Connector K2:  20 pin dual line pin array:

2        20

oooooooooo

oooooooooo

1        19

1	+5V		 2	+5V

3	Data 4		 4	Data 3

5	Data 2		 6	Data 1

7	Data 0		 8	Addr 2

9	Addr 1		10	Addr 0

11	Clock		12	--

13	Addr 4		14	--

15	GND		16	GND

17	+12V		18	+12V

19	tx/rx sel	20	Beeper

K3  -  VT220 KEYBOARD INTERFACE

A VT220 compatible keyboard can be connected to this interface. The interface is asynchronous serial interface, 4800 bps, RS232C compatible.

Connector K3:  4 pin modular jack  (telephone connector)

1	RxD

2	+12V

3	GND

4	TxD

K4  -  SERIAL PRINTER INTERFACE

RS232C compatible serial printer output. This interface has no Receive Data input. Therefore, an XON-XOFF handshake is not possible, and hardware DTR handshake must be used. Use jumper J14 to select which pin is used for handshake input. A 9-pin D-type connector may be connected here by using an ISD connector and flat cable. Pin assignment is compatible with standard 9-pin connectors. (Pin numbers for D-connector are shown in parenthesis.)

Connector K4:  10 pin dual inline pin array

2   10

ooooo

ooooo

1   9

1 (1)	--	 2 (6)	DTR *

3 (2)	TxD **	 4 (7)	DTR *

5 (3)	TxD **	 6 (8)	DTR *

7 (4)	DTR *	 8 (9)	--

9 (5)	GND	10	--

* Use J14 to select a pin for DTR handshake line.

** Use J13 to select a pin for TxD.

K5  -  COMMON CONNECTOR

This connector contains all the inputs and outputs of K3, K4, K6 and K7, plus TTL level I/O signals. K5 may be used, if the connector module is not installed. Ask for full details.

K6  -  HOST SERIAL PORT (RS232C)

An RS232C compatible serial interface for the host computer. Communication parameters for this interface are selected from the SETUP menu. Jumpers J7, J11, J12 and J19 are used to select the pin assignment. A 9-pin D-type connector may be connected here by using an ISD connector and flat cable; pin assignment is compatible with standard 9-pin D-connectors used on many computers. (D-connector pin positions are shown in parenthesis.)

Connector K6:  10 pin dual inline pin array

2   10

ooooo

ooooo

1   9

1 (1)	DCD  ¤	 2 (6)	--

3 (2)	RxD/TxD *	 4 (7)	RTS/CTS **

5 (3)	TxD/RxD *	 6 (8)	CTS/RTS **

7 (4)	DTR 	 8 (9)	--

9 (5)	GND	10	--

*	Use jumper J12 to select pins for TxD and RxD.

**	Use jumper J11 to select pins for RTS and CTS

¤ 	Use jumper J19 to select DCD input instead of CTS

Use jumper J9 to tie DTR to +12V, if required.

K7  -  IBM PC KEYBOARD INTERFACE

An IBM PC/XT compatible keyboard may be connected here. The interface is synchronous serial interface, using TTL levels. A special version of the Alphasoft-200 EPROM is required in order to use this interface.

Connector K7:  4 pin DIN connector

1	CLK

2	Data in/out

3	Reset

4	GND

5	+5V

K8  -  HOST SERIAL PORT, CURRENT LOOP   (optional)

Passive 20 mA opto isolated current loop interface. The current must be supplied externally.  Note:  Maximum voltage for the external current source is 24 V.

Use jumper J8 to select RxD from RS232C or Current  Loop.

Connector K8:  10 pin dual inline pin array

2   10

ooooo

ooooo

1   9

1 (1)	+12V	 2 (6)	RxD (+)

3 (2)	+12V	 4 (7)	TxD (-)

5 (3)	GND	 6 (8)	TxD (+)

7 (4)	GND 	 8 (9)	GND

9 (5)	RxD (-)	10	--

K9  -  DISPLAY CONNECTOR

Interface for the Flat Panel Display. This interface is software configurable to suit most display types. Pin assignments may wary between different display types and manufacturers. Display type is selected by installing corresponding Alphasoft EPROM and setting jumpers J16, J17, J18 and J23. 10 to 22 pin connectors may be assembled on the PCB. The pin assignments for Finlux / Planar / Sharp EL displays and for Panasonic Plasma display are given below.

Connector K9:

a)  EL-display: 				b)  Panasonic PG3 Plasma display:

2      16				2        20

oooooooo				oooooooooo

oooooooo				oooooooooo

1      15				1        19

1  +12V		  2  +12V		 1  +5V  	   2  +5V

3  +5V		  4  +5V			 3  ENA	 	   4  BACKG

5  (+25V)	  6  (+25V)		 5  Vsync	   6  GND

7  GND  	  8  GND			 7  Hsync	   8  GND

9  VSync	 10  GND			 9  BLANK	  10  GND

11  HSync	 12  GND			11  CLOCK	  12  GND

13  CLK  	 14  GND			13  DATA0	  14  GND

15  Data 	 16  GND			15  DATA1	  16  GND

17  DATA2	  18  GND

19  DATA3	  20  GND

K10  -  HOST SERIAL PORT,  RS485   (optional)

An RS485 compatible serial interface for host computer. (The RS485 interface is also electrically compatible with the RS422 standard.)

RS485 allows the 38400 bps transfer rate to be used at distances of up to 1500 m. Up to 32 terminals may be connected at the same party line. Single twisted pair cable is used to connect the terminals and the host computer. Optionally, an add-on board is available for full duplex communication with host via two twisted pair cables.

Use jumper J8 to select RxD from either RS232C or RS485. Jumper J10 position 5 must be closed, if RS485 interface is used for multipoint communications. If point-to-point connection is used (RS232C or RS422), J10.5 must be left open.

When long cable is used, the cable must be terminated from both ends using a resistor of approximately 150 ohms. For multi-point communications, biasing resistors must be connected to one end of the cable (an application note is available for more information). If full duplex board is used for point-to-point communications (J10.5 open), the biasing resistors are not needed.

Note: RS485 and Current Loop may not co-exist on the same board.

Connector K10:  10 pin dual inline pin array

2   10

ooooo

ooooo

1   9

1 (1)	+12V	 2 (6)	+Data

3 (2)	+12V	 4 (7)	--

5 (3)	GND	 6 (8)	GND

7 (4)	GND 	 8 (9)	GND

9 (5)	-Data	10	--

6.4.  TECHNICAL SPECIFICATIONS:

AP988200		- Standard version

AP988200-EXT	- Extended temperature range

AP988200-F	- High speed version

Technology:		CMOS

CPU:			Z180,	  6.144 MHz

12.288 MHz   (AP988200-F)

EPROM:			32K, expandable to 64K

RAM:			32K static, with battery back-up

Video controller:		HD63B45 + Custom LCA

Video RAM:		8K character RAM, 8K attribute RAM, static

Character generator:	32K static RAM

|  | AP988200 | AP988200-EXT |  |
| --- | --- | --- | --- |
| Supply voltage: | 9 to 35 VDC | 9 to 35 VDC | 1) |
| - Without on-board power module: | 5 VDC , ±5 % | 5 VDC , ±5 % | 2) |
| Supply current: @ 12V | 80 mA | 80 mA | typ 3) |
| 5V output for display: cont. | 500 mA | 500 mA | max |
| Peak (3 sec) | 1000 mA | 1000 mA | max |
| Operating temperature: | 0 to 70 °C | -20 to 75 °C | 4) |
| Storage temperature: | -25 to 85 °C | -40 to 125 °C |  |
| Humidity: (non-condensing) | 0 to 90 % | 0 to 90 % |  |
| Dimensions: | 228 x 140 x 25 mm | 228 x 140 x 25 mm | 5) |
| Weight: | 170 g | 170 g |  |

1)  Supply voltage has to be set as required by the display panel and keyboard in use.

2)  In addition, the voltage(s) required by display panel must be supplied.

3)  Excluding display and keyboard

4)  Temperature range may depend on the display panel in use

5)  The highest components are the keyboard connectors.

§

DIMENSIONS:

Board dimensions:

A	140.0	mm

B	228.0	mm

C	43.0	mm

D	53.0	mm

E	36,0	mm

F	8.5	mm

G	65.0	mm

Screw positions:

f1	3.8	mm

h	116.0	mm

i	12.0	mm

j	4.0	mm

k	220.0	mm

o	58.5	mm

p	110.0	mm

f2	3.0	mm

l	215.0	mm

m	6.5	mm

n	3.0	mm