CARROLL TOUCH DEMO FOR THE AP988200

Syntax:

CARROLL [2] [/Saa /Ebb /Xcc] [/T]

where:

2	= Use serial port COM2 (default: COM1)

aa	= Start of TSO message identifier in HEX

(default = FE)

bb	= End of TSO message identifier in HEX

(default = FF)

cc	= Start of Exit Point Report identifier in HEX

(default = FD)

/T	= Show TSO message

Normally, the demo is started by simply entering command

CARROLL

If the TSO does not use the default values for the identifiers, you

must give them on the command line.

Use the /T option to check the TSO message format.

Use environment variable BAUD to set the baud rate, e.g.

SET BAUD=38400

If the environment variable BAUD does not exist, default value of

9600 bps is used.

If environment variable CARROLL exists, its contents is appended

to the command string. This may be used to avoid entering the

command line flags every time the program is started.