2.  INSTALLATION

2.1  CONFIGURATION OPTIONS

The default configuration of the AP988200 controller board contains:

- Display panel interface for the display type selected

- Host serial interface, RS232C.

- Serial printer interface, RS232C

- Optical Touch Screen Overlay interface

- Membrane type Touch Screen Overlay interface (requires special EPROM)

- VT220 keyboard interface

- On-board beeper

- On-board DC/DC converter for RS232C voltages

- On-board Switched Mode Power Supply, to generate the 5V operating voltage

for both the AP988200 and  the display panel

- CMOS RAM with battery backup

- Alphasoft-200 EPROM for the selected display type and resolution

Optionally, the AP988200 controller board may include:

- IBM-PC compatible keyboard interface

- RS485 or Current Loop for host serial interface

- Common-connector for many of the interface signals

- Power LED indicator

- Customized Alphasoft-200 version

- Stripped-down versions of the AP988200 are also available. The board may be manufactured without the power-supply module and connector module to fit in smaller space.

Optional accessories:

-  Display cable for the selected display type (EL, Plasma, etc.)

-  Host serial cable

-  Printer cable

-  Mechanical assembly accessories

-  Keyboard

-  An AC power supply

(Input: 115 or 230 VAC,  Output: 12 VDC, 3 Amps).

2.2  ELECTRICAL CONNECTIONS

Fig. 2.1:  I/O connectors

When installing the AP988200 in your system, the following electrical connections are needed. (Some of the connections are optional, depending on your system requirements.)

1.	Connect the power supply to the Power Input -connector (PK2). Do not switch the power on until all the connections have been made. The operating voltage may be in range 12 to 24 volts, depending on the display type used (check your display panel data sheet for correct voltage). The 5V operating voltage is generated by an on-board Switched Mode Power Supply (SMPS).  The AP988200 uses only the 5V voltage, and also supplies 5V to the display panel. However, the VT220 keyboard and TSO both use the 12V supply. If these peripherals are used with a 24V power supply, an additional 12V supply is needed.

If the Power Supply Module is not installed, the 5V voltage must also be supplied. In this case, the operating voltages are connected to connector PK1.

Some display panels, such as plasma panels, may require additional operating voltages. Those voltages must be connected externally to the display panel.

2.	Connect the display cable from the display connector (K9) to the display panel. Make sure that the pin1 of the cable connector is connected to the pin 1 of the connector on the PCB.

3.	Connect the host serial cable to connector K6. A flat cable with IDC connectors may be used to connect to an 9-pin D connector. The pin order is compatible with a standard 9-pin serial port connector. The AP988200 may act either as a Data Terminal Equipment (DTE) or as Data Communications Equipment (DCE), as selected by jumpers J11 and J12. You must be sure to use correct type of cable (direct or crossed wire), depending on the type of host serial port (DTE or DCE). Only three wires need to be connected: the data lines (TxD, RxD) and ground. If the hardware handshake is required, the handshake lines (CTS, RTS, DTR) may also need to be connected.  See chapter 6 for pin descriptions and jumper settings.

4.	Connect the serial printer to port K4 (if required). Select the DTR handshake input pin, using jumper J14, to suit your printer. The printer must supply hardware handshake; the Xon-Xoff handshake is not supported.

5.	Connect the VT220 compatible keyboard (if required) to connector K6 (a 4-pin telephone type connector), or an IBM-PC/XT compatible keyboard to connector K7 (a 5-pin DIN connector). A serial ASCII keyboard may also be used on connector K6 (requires special EPROM version). WARNING: The VT220 keyboard uses the 12V power supply. If you are using other than 12V power supply voltage, an additional 12V regulator is needed for the keyboard.

6.	Connect Touch Screen Overlay (TSO), if required. Optical TSO is connected to connector K2. Membrane TSO or switch matrix type keyboard is connected to connector K1. Only one type of TSO may be connected at a time. The standard Alphasoft-200 firmware contains support for the optical TSO. Membrane TSO requires a special version of the Alphasoft-200 EPROM.

See chapter 6.3 for the pin descriptions for each connector.

2.3  MECHANICAL ASSEMBLY

10 screw holes are available on the AP988200 printed circuit board for fitting the board. These match directly to the screw holes on many Sharp and Planar EL panels (EL640.400, EL640.350 etc.) with or without aluminium frames. Some additional mechanical parts may be needed to fit controller board to other display panels of different size.

See chapter 6.4 for board dimensions and exact locations of the screws.

The AP988200 controller board, display panel and (optional) Touch Screen Overlay are also available as complete module for easy fitting on your mechanics. See fig. 2.2 on next page.  A complete, cased terminal (AP200) is also available.

Fig. 2.2:  AP200MT module, assembly diagram

2.4  TESTING THE INSTALLATION

	After you have gone through chapters 2.2 - 2.3 of the installation successfully, turn ON the power on your new AP988200 terminal.

You should see a cursor at the upper left corner of the display and a status line at the bottom of the display with light background. (Note, however, that on some special versions of the firmware, either the cursor or status line may be disabled. In that case, proceed to step 2)

	If the keyboard is available, press function key F3 in the upper row of the keyboard to invoke the SETUP-mode.

On other than VT220 or IBM-PC keyboard, some other key may be used as setup-key. See your keyboard specific application note. If the keyboard does not have the setup-key, the SETUP-mode may be invoked by pressing Ctrl-Shift-@ twice.

SETUP screen should appear (see fig. 3.8 at chapter 3.3). Use cursor keys to move the highlight bar  around on the display. You now know, that both display and keyboard are working.

	If you have Touch Screen Overlay (TSO) installed, touch the screen and move your finger around. You should hear click sounds when you move your finger.

	For testing the serial port, you may perform a loop test. Connect the Transmit Data pin of the serial port directly to Receive Data pin. (On D-connector, short circuit pins 2 and 3. Or, on AP988200 board connector K6, short circuit pins 3 and 5. See chapter 6.3.)

Now enter characters from keyboard or from the TSO. The characters should appear on screen (even in Full Duplex mode).

In order to the system to work properly, correct serial port parameters need to be set. The parameters may be set from the keyboard by using the SETUP mode, see chapter 3.3.

If your system configuration does not include a keyboard, and the standard serial port parameters (see chapter 3.3)  are not suitable, the setup parameters may be set by patching the EPROM (see chapter 5.2). Some of the parameters can also be changed by sending commands from host computer (See chapter 4.1).

Use the SETUP-menu according to chapter 3.3 instructions to modify the terminal settings to match the requirements of your host hardware/software. Set the serial parameters so that they match the settings on the host computer. Also make sure that your host computer has been set up to recognize the AP988200 either as a VT220-, VT100- or VT52 type terminal.

If  you are going to use a printer, check that the printer port settings on SETUP match the settings on your printer.  Recommended settings are: 4800 baud, 8+1 bits, No parity.

If you are using a VT220 or IBM-PC keyboard, check that the KEYBOARD field in the SETUP-menu contains the proper setting for your national keyboard.

Once you are satisfied with the SETUP parameter settings, press the F3-key once again to change from SETUP mode to the ON-LINE mode and to save the settings in nonvolatile memory.

	Now you are ready to login to your host application. For using the actual application and/or the operating system services of your host computer, you need to refer to the manuals that have been supplied by your hardware/software vendor.

2.5  TROUBLESHOOTING

Before calling Alpha Point or authorized service center, please check the following items:

| Problem | Check these items |
| --- | --- |
| No picture, no response from keyboard or TSO | - Correct operating voltage connected? (See chapter 2.2) - EPROM properly in socket? |
| No picture, keyboard and TSO responding OK | - Any additional voltages required by display panel connected? - Display cable (K9) properly connected? - Correct EPROM version installed? (See chapter 5.1) - Display panel undamaged? |
| Keyboard not responding | - Correct keyboard type in use (e.g. PC/XT vs. PC/AT kbd)? - Correct EPROM version for selected keyboard? - VT220 keyboard: 12 Voperating voltage available? (e.g. when using 24 V for display panel) |
| Does not send/receive characters from host | - SETUP field DUPLEX must not be set to LOCAL. - Perform loop test (see previous page). If loop test fails, the serial port drivers may be broken and the board needs servicing. |
| Does not send/receive characters from host. Loop test passed. | - Correct type of cable in use (direct / crossing)? - Correct handshake mode (Xon-Xoff / CTS) - Does host computer require hardware handshake lines? - Jumpers J8, J11, J12 (and J7, J19, if CTS handshake used) correctly set? (see chapter 6.2) |
| Garbage characters received from host | - Baud, parity, bits settings on SETUP correct? - Jumper J8 connected? - Cable OK? (check ground wire!) |
| RS485: garbage characters received | - Biasing resistors installed (for multipoint comm.)? - Cable OK? (correct polarity, ground connected) |
| Touch Screen not responding | - Mode select jumper J10 bit 4 installed? - Correct EPROM version installed? - External TSO interface: operating voltage connected? |
| Printer not responding | - Printer baud, parity etc. correct? - Jumpers J13 and J14 set correctly? |
| Scrambled display, unsynchronized or flickering display | - Correct EPROM for selected display? (See chapter 5.1) - Correct crystal frequency for selected display? |
| Does not execute commands correctly. | - Correct emulation selected from SETUP? |

APTEST program (for IBM-PC compatibles) is available from Alpha Point for easier testing of the terminal configuration.