5.  TECHNICAL INFORMATION

5.1  PATCHING THE EPROM

Several default parameters on the Alphasoft-200 may be changed by patching the EPROM contents. The list below gives information about each parameter. Parameters are referred by labels. See Appendix A for the addresses corresponding each label.

SER_INI+0:	Databits, stopbits, parity

Bit:

7

6

5

4

3

2

1

0

CkP

CkF

0

EVP

0

DB

PE

SP

Where:

SP	(bit 0)	=  1 stopbit (0) or 2 stopbits (1)

PE	(bit 1)	=  Parity disabled (0) or enabled (1).

DP	(bit 2)	=  7 databits (0) or 8 databits (1)

EVP	(bit 4)	=  Even parity (0) or odd parity (1).

CkF	(bit 6)	=  Check Framing Errors: 0 = yes, 1 = no

CkP	(bit 7)	=  Check Parity Errors: 0 = yes, 1 = no

Example:

To set 8 bits, 1 stopbit, Even parity:  change this location to hold value 16H.

SER_INI+1:	Baud rate

00H	= 38400 bps		05H	=  1200 bps

01H	= 19200 bps		06H	=    600 bps

02H	=   9600 bps		0DH	=    300 bps

03H	=   4800 bps		0EH	=    150 bps

04H	=   2400 bps

SER_INI+2:	Handshake

00	= none

01	= Xon-Xoff

02	= Hardware (CTS)

03	= Both

SER_INI+3:	Duplex

00	= Full duplex

01	= Half duplex (local echo)

03	= Local mode

09	= Block mode

SER_INI+4:	Status line type

00	= No status line displayed

01	= Indicator status line

02	= Host writeable status line

SER_INI+5:	Language (national replacement character set)

00	= USASCII			06	= French

01	= Finnish			07	= Norwegian

02	= German			08	= Swiss

03	= Danish			09	= Italian

04	= British			0A	= Dutch

05	= Spanish			0B	= Swedish

SER_INI+6:	Screen saver

00	= Screen saver disabled

>00	= Screen saver time in minutes (1 to 255)

note: in order to screen saver time to be displayed correctly on SETUP, the time should be set to one of: 00, 05, 0F, 1E or 3C (off, 5, 15, 30 or 60 minutes)

SER_INI+7:	National / multinational mode

00	= Multinational mode. Use 8-bit multinational character set.

01	= National mode.  Use 7-bit NRC sets

SER_INI+8:	Cursor mode and Scrolling mode

Bit:

7

6

5

4

3

2

1

0

0

0

0

CE

0

SC

B

T

Where:

T 	(bit 0)	=  Cursor type:	 	0 = line,	1 = block

B	(bit 1)	=  Cursor blink:	 	0 = steady,	1 = blinking

SC 	(bit 2)	=  Scrolling: 	 	0 = jump,	1 = smooth

CE	(bit 4)	=  Cursor disable: 	0 = enabled,	1 = disabled

SER_INI+9:	Emulation

01h	= VT100

02h	= VT220, 7-bit control codes

22h	= VT220, 8-bit control codes

04h	= VT52

80h	= Monitor

SER_INI+10:	Keyboard type, language and mode:

Language:				    Mode:

00	= USA				00	= Typewriter

01	= Finnish			80	= Data Processing

02	= French

03	= UK

04	= German			    Type:

05	= Swiss German			00	= VT220

06	= Dutch			40	= ASCII

07	= Italian

08	= Norwegian

09	= Danish

0A	= Swiss French

0B	= Swedish

SER_INI+11:	Net address:

00	= Wildcard address, receive all messages

01 - 4F	= Address of the terminal

SER_INI+12:	Screen mode and Blink rate

Bit:

7

6

5

4

3

2

1

0

0

0

0

0

0

SM

BL1

BL0

Where:

BL1,BL0	= Blink rate:	00  =  Fast

01  =  Normal

10  =  Slow

11  =  Very slow

SM		= Screen Mode:	0  =  Normal

1  =  Reverse

SER_INI+13:	Keyboard  status:

Bit:

7

6

5

4

3

2

1

0

CA

KA

IM

0

NL

0

CL

0

Where:

CL	(bit 1)	=  Caps Lock status after reset:  0 = off, 1 = on

NL	(bit 3)	=  New line mode:  0 = off (linefeed), 1 = on (newline)

IM	(bit 5)	=  Insert mode:  0 = off (replace), 1 = on (insert)

KA	(bit 6)	=  Numeric keypad mode:  0 = numeric, 1 = application

CA	(bit 7)	=  Cursor key mode:  0 = cursor, 1 = application

SER_INI+14:	Printer baud:

-  See SER_INI+1 (Baud Rate) above.

-  4800 bps (03H) is recommended.

SER_INI+15:	Printer bits and parity:

Bit:

7

6

5

4

3

2

1

0

0

0

0

EVP

0

DP

PE

SP

Where:

SP	(bit 0)	=  1 stopbit (0) or 2 stopbits (1)

PE	(bit 1)	=  Parity disabled (0) or enabled (1).

DP	(bit 2)	=  7 databits (0) or 8 databits (1)

EVP	(bit 4)	=  Even parity (0) or odd parity (1).

TSO_INI:		TSO mode

Bit:

7

6

5

4

3

2

1

0

OC

SC

0

Net

QM

DR

FU

DB

Where:

DB	(bit 0)	=  Debug Mode - Send message even if only one of

the coordinates detected

FU	(bit 1)	=  Finger Up detect - send '?? ??' message when

the 'key' is released.

DR	(bit 2)	=  Double resolution mode using interpolation

(optical TSO only)

QM	(bit 3)	= Query Mode - Send message only when tso report

query received from the host.

NET	(bit 4)	=  Network mode (insert net address in message)

SC 	(bit 6)	=  Single Character mode - convert coordinates to

a single character using TSOTABL.

OC 	(bit 7)	=  Old Alphasoft Compatibility mode.

Set this bit to get TSO message exactly the same as Alphasoft-030 V 2.1 or older.

TSO_INI+1	TSO repeat delay

- Delay time before repeating, if same TSO position is touched constantly.

1 = shortest delay,  255 = max delay.

TSO_INI+2	TSO repeat rate

- Repeat rate, when same TSO position is touched constantly.

1 = fastest rate, 255 slowest.

0 = no repeat.

TSO_INI+3	TSO speed

a)	on OPTICAL TSO:  Tracking speed

The rate of coordinates sent, when finger is moved.

1 = fastest, 255 slowest rate.

0 = no tracking, new coordinate sent only after finger has

been removed from the screen.

b)	on MEMBRANE TSO:  Scann rate

Membrane switch matrics scann rate, ms / row.

TSOX:		TSO X-resolution

- This parameter must be set equal to the physical resolution of the TSO connected to controller:

18h (24 dec)  for AP988030/A or /B TSO frame

30h (48 dec)  for AP988030/BB TSO frame

04h to 0Ch (4 to 12 dec)  for membrane TSO

TSOY:		TSO Y-resolution  (optical TSO only)

- As above. Value must be 10h (16) or 1Eh (30).

MINBRK:	Min. scanns key closed until accepted (membrane TSO only)

- Selects, how many consecutive scanns a contact must be closed until accepted. Must be less than or equal to parameter GRPSIZ. The contact matrix is scanned GRPSIZ times and then, if at least MINBRK of the scanns indicate contact closed, the key is accepted.

GRPSIZ:

a)  on optical TSO:	TSO Group size

08h	for AP988030/A or AP988030/B

10h	for AP988030/BB

b)  on membrane TSO:	Scann count

- Number of times the membrane matrix is scanned before checking and sending the coordinates.

KEYPAD:		Key conversion table

for keypad keys, function keys and special editing keys.

- In the table, 16 bytes are reserved for each key: 8 bytes for normal mode and 8 bytes for the application mode. If less than 8 bytes are needed, the rest is filled with nulls (00h), which are not sent to the host computer.

CURPAD:	Key conversion table for cursor keys, as above.

CODESC:	Scann codes for the keys that are used as cursor keys is SETUP.

CURPA52:	Key conversion table for cursor keys in VT52 mode.

KEYPA52:	Key conversion table for keypad keys in VT52 mode.

FKEYINI:	Top-row function key strings (F6 to F20)

- The order of the keys in the table is F6, F7, F8 etc.

- Each string is terminated by character ST (hex 9C)

FKEY100:		Top-row function key strings in the VT100 emulation mode

REPTABL:	Characters with national variations.

- These characters are handled, when generating the national replacement character sets (NRC-sets).

REP1:		Start of the first national replacement character set (Finnish).

-  Other tables are in the order indicated by language parameter (SER_INI+5).

-  Length of each table is 12 bytes

- Each entry in the table is the character code in the DEC SUPPLEMENTAL CHARACTER SET (if bit 7 is zero) or in the USASCII character set (if bit 7 is set). The fiqure of the specified character is copied to the national set in place of the ascii character. If entry in the table is Space (20h), that character is same as in ASCII set and is thus not copied.

SET20:		Character generator for the USASCII character set.

128 characters, 16 bytes per character.

SET22:		Char. gen. for the DEC SUPPLEMENTAL character set.

128 characters, 16 bytes per character.

GRA2: 		Char. gen. for the DEC SPECIAL character set (semigraphics).

Contains only characters in range 60h to 7Fh.

CHRIBM:	Char. gen. for the IBM PC character set

(IBM PC codes 80h to FFh)

DBUG_CH:	Debug Character  (Optical TSO only)

- Character sent to host in single character mode, if incomplete coordinate pair is detected and debug mode is enabled.

FING_UP:	Finger Up -character

- Character code sent to host computer, when 'key' is released in single character mode (if finger-up detect mode is enabled).

TSOTABL:	TSO single character conversion table.

- The table contains one byte for each TSO crossover point. How the characters are handled, depends on the value as follows:

00		Not sent to host. No click generated.

01 to 7F		Sent to host directly.

80 to DF	Converted to string using ALTTAB *

E0		Caps Lock key *

E1		Setup key *

FD		Ctrl key *

FE		ALT key *

FF		SHIFT key *

* Membrane TSO only

SHIFTBL:	Shift conversion table  (membrane TSO only)

-  For converting other than alphabetic keys.

-  Contains one byte for each character in range 20H to 3Fh.

ALTLEN:		Length of each entry in the ALTTAB.  Normally 04h.

ALTTAB:		ALT key / special key conversion table. (membrane TSO only)

- 4 bytes reserved for each key. If less than 4 bytes needed, fill rest which nulls (00h). If more than 4 bytes are needed, change the ALTLEN above.

-  First entry for code 80h, next 81h etc.

TESTMSG:	Text displayed at power-on, if jumper J10.7 is closed.

First byte is the length of the text.