5.  TECHNICAL INFORMATION

5.1  EPROM VERSIONS

The AP988200 controller board can be equipped with wide variety of EPROM versions, depending on the hardware configuration and optional features required. It is important that correct EPROM version is installed for each hardware configuration.

The EPROM is identified by code written on the EPROM label, for example:

AP400MX

(1)	AP200,  EL-display ¾		       ¾¾   Keyboard = IBM XT 	   (3)

(2)	Resolution 640.400 ¾¾¾¾	   ¾¾¾   Touch screen = membrane

(1)   The first 1-3 letters indicate AP988200 EPROM and defines the display type:

AP		=  EL-display

P		=  Plasma display

CRT		=  CRT display (VGA monitor or SCART TV-set)

LCD		=  LCD display (TFT, no adapter needed)

(2)   Resolution of the display:

320		=  320.256

350		=  640.350

400		=  640.400

480		=  640.480

512		=  512.256

(3)   Remaining letters define display model, keyboard, touch screen, and other options that

differ from standard version:

(nothing)	=  VT220 keyboard, Alpha Point TSO, standard version

C3 or 3		=  Planar EL640.480-C3 (or CB1/CB2) display panel

LCD or L	=  LCD display, adapter needed

PG3		=  Panasonic *PG3 display panel

PG2		=  Panasonic *PG2 display panel

PD3		=  Panasonic *PD3 display panel

S		=  Sharp EL display (if different from Planar)

CT or C		=  Carroll Touch TSO

M		=  Membrane TSO

ASC or A	=  ASCII keyboard

HEX		=  Hexadecimal keyboard at membrane connector

IBM or X	=  IBM-XT keyboard

Z		=  Zoom option installed

16		=  Only 16 lines in terminal mode (standard = 24 lines)

18		=  Only 18 lines in terminal mode

In addition to the above, several customer specific options may be indicated in the EPROM ID.

Note:  If you change the EPROM in order to use different display type, you may need to change the dot clock crystal, too. Standard crystal frequencies used are 15, 16 and 24 MHz. Normally, highest frequency that display panel accepts is used. When you order the controller board for specified display panel, correct crystal and EPROM will be installed by Alpha Point.

Below is a list of crystal frequencies for some of the common displays:

Planar		EL320.256	16	MHz

EL512.256	15	MHz

EL640.200-U	15	MHz

EL640.350  *)	16	MHz

EL640.350-DA	24	MHz

EL640.400  *)	16	MHz

EL640.400-C3	24	MHz

EL640.480-A	24	MHz  

EL8358HR	16	MHz

Sharp		LJ-320U26	16	MHz

LJ-512U25	16	MHz

LJ-640U27	16 - 22	MHz

Panasonic	MD400B640PD3	16	MHz

MD400L640PG2	16	MHz

MD400B640PG3	19.66	MHz  

MD480L640PG3	24	MHz

LCD		640.400		16	MHz

640.480		24	MHz

CRT		TV, SCART	14.318	MHz  

VGA monitor	25	MHz  

*)  Old display type

  Special cable for display needed

5.2  PATCHING THE EPROM

Several default parameters on the Alphasoft-200 may be changed by patching the EPROM contents. The list below gives information about each parameter. Parameters are referred with labels.  See Appendix A for the addresses corresponding each label.

SER_INI+0:	Databits, stopbits, parity

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CkP | CkF | 0 | EVP | 0 | DB | PE | SP |

Where:

SP	(bit 0)	=  1 stopbit (0) or 2 stopbits (1)

PE	(bit 1)	=  Parity disabled (0) or enabled (1).

DP	(bit 2)	=  7 databits (0) or 8 databits (1)

EVP	(bit 4)	=  Even parity (0) or odd parity (1).

CkF	(bit 6)	=  Check Framing Errors: 0 = yes, 1 = no

CkP	(bit 7)	=  Check Parity Errors: 0 = yes, 1 = no

Example:

To set 8 bits, 1 stopbit, Even parity:  change this location to hold value 16H.

SER_INI+1:	Baud rate         (for AP988200-F, the given baud rate values are doubled)

00H	= 38400 bps		05H	=  1200 bps

01H	= 19200 bps		06H	=    600 bps

02H	=   9600 bps		0DH	=    300 bps

03H	=   4800 bps		0EH	=    150 bps

04H	=   2400 bps

SER_INI+2:	Handshake

00	= none

01	= Xon-Xoff

02	= Hardware (CTS)

03	= Both

SER_INI+3:	Duplex

00	= Full duplex

01	= Half duplex (local echo)

03	= Local mode

09	= Block mode (= Line mode)

SER_INI+4:	Status line type

00	= No status line displayed

01	= Indicator status line

02	= Host writeable status line

SER_INI+5:	Language (national replacement character set)

00	= USASCII			06	= French

01	= Finnish			07	= Norwegian

02	= German			08	= Swiss

03	= Danish			09	= Italian

04	= British			0A	= Dutch

05	= Spanish			0B	= Swedish

SER_INI+6:	Screen saver

00	= Screen saver disabled

>00	= Screen saver time in minutes (1 to 255)

note: in order to screen saver time to be displayed correctly on SETUP, the time should be set to one of: 00, 05, 0F, 1E or 3C (off, 5, 15, 30 or 60 minutes)

SER_INI+7:	National / multinational mode

00	= Multinational mode. Use 8-bit multinational character set.

01	= National mode.  Use 7-bit NRC sets

SER_INI+8:	Cursor mode and Scrolling mode

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | CE | 0 | SC | B | T |

Where:

T 	(bit 0)	=  Cursor type:	 	0 = line,	1 = block

B	(bit 1)	=  Cursor blink:	 	0 = steady,	1 = blinking

SC 	(bit 2)	=  Scrolling: 	 	0 = jump,	1 = smooth

CE	(bit 4)	=  Cursor disable: 	0 = enabled,	1 = disabled

SER_INI+9:	Emulation

01h	= VT100

02h	= VT220, 7-bit control codes

22h	= VT220, 8-bit control codes

04h	= VT52

08h	= ADM-3   (if supported, requires special EPROM)

80h	= Monitor

SER_INI+10:	Keyboard type, language and mode:

Language:				    Mode:

00	= USA				00	= Typewriter

01	= Finnish			80	= Data Processing

02	= French

03	= UK

04	= German			    Type:

05	= Swiss German			00	= VT220

06	= Dutch			40	= ASCII

07	= Italian

08	= Norwegian

09	= Danish

0A	= Swiss French

0B	= Swedish

SER_INI+11:	Net address:

00	= Wild card address, receive all messages

01 - 4F	= Address of the terminal

SER_INI+12:	Screen mode, Blink rate and Zoom init

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | ZV | ZH | SM | BL1 | BL0 |

Where:

BL1,BL0	= Blink rate:	00  =  Fast

01  =  Normal

10  =  Slow

11  =  Very slow

SM		= Screen Mode:	0  =  Normal

1  =  Reverse

ZV, ZH		= Zoom status after reset,  Vertical and Horizontal, respectively

(requires special EPROM).

0  =  Normal

1  =  Zoomed by factor 2

SER_INI+13:	Keyboard  status after power-on:

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CA | KA | IM | 0 | NL | 0 | CL | 0 |

(Continues on next page)

Where:

CL	(bit 1)	=  Caps Lock status after reset:  0 = off, 1 = on

NL	(bit 3)	=  New line mode:  0 = off (linefeed), 1 = on (newline)

IM	(bit 5)	=  Insert mode:  0 = off (replace), 1 = on (insert)

KA	(bit 6)	=  Numeric keypad mode:  0 = numeric, 1 = application

CA	(bit 7)	=  Cursor key mode:  0 = cursor, 1 = application

SER_INI+14:	Printer baud:

-  See SER_INI+1 (Baud Rate) above.

-  4800 bps (03H) is recommended.

SER_INI+15:	Printer bits and parity:

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | EVP | 0 | DP | PE | SP |

Where:

SP	(bit 0)	=  1 stopbit (0) or 2 stopbits (1)

PE	(bit 1)	=  Parity disabled (0) or enabled (1).

DP	(bit 2)	=  7 databits (0) or 8 databits (1)

EVP	(bit 4)	=  Even parity (0) or odd parity (1).

TSO_INI:		TSO mode   (for Alpha Point TSO or Membrane TSO)

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OC | SC | 0 | Net | QM | DR | FU | DB |

Where:

DB	(bit 0)	=  Debug Mode - Send message even if only one of

the coordinates detected

FU	(bit 1)	=  Finger Up detect - send '?? ??' message when

the 'key' is released.

DR	(bit 2)	=  Double resolution mode using interpolation

(optical TSO only)

QM	(bit 3)	= Query Mode - Send message only when tso report

query received from the host.

Net	(bit 4)	=  Network mode (insert net address in message)

SC 	(bit 6)	=  Single Character mode - convert coordinates to

a single character using TSOTABL.

OC 	(bit 7)	=  Old Alphasoft Compatibility mode.

Set this bit to get TSO message exactly the same as Alphasoft-030 V 2.1 or older.

TSO_INI+1	TSO repeat delay   (for Alpha Point TSO or Membrane TSO)

- Delay time before repeating, if same TSO position is touched constantly.

1 = shortest delay,  255 = max delay.

TSO_INI+2	TSO repeat rate

- Repeat rate, when same TSO position is touched constantly.

1 = fastest rate, 255 slowest.

0 = no repeat.

TSO_INI+3	TSO speed

a)	on Alpha Point Optical TSO:  Tracking speed

The rate of coordinates sent, when finger is moved.

1 = fastest, 255 slowest rate.

0 = no tracking, new coordinate sent only after finger has been

removed from the screen.

b)	on Membrane TSO:  Scan rate

Membrane switch matrix scan rate, ms / row.

TSOX:		TSO X-resolution

- This parameter must be set equal to the physical resolution of the TSO connected to controller:

18h (24 dec)  for AP988040, AP988030/A or /B TSO frame

30h (48 dec)  for AP988030/BB TSO frame

04h to 0Ch (4 to 12 dec)  for membrane TSO

TSOY:		TSO Y-resolution  (optical TSO only)

- As above. Value must be 10h (16) or 1Eh (30).

MINBRK:	Min. scans key closed until accepted (membrane TSO only)

- Selects, how many consecutive scans a contact must be closed until accepted. Must be less than or equal to parameter GRPSIZ. The contact matrix is scanned GRPSIZ times and then, if at least MINBRK of the scans indicate contact closed, the key is accepted.

GRPSIZ:

a)  on optical TSO:	TSO Group size

08h	for AP988040, AP988030/A or AP988030/B

10h	for AP988030/BB

b)  on membrane TSO:	Scan count

- Number of times the membrane matrix is scanned before checking and sending the coordinates.

KEYPAD:		Key conversion table

for keypad keys, function keys and special editing keys.

- In the table, 16 bytes are reserved for each key: 8 bytes for normal mode and 8 bytes for the application mode. If less than 8 bytes are needed, the rest is filled with nulls (00h), which are not sent to the host computer.

CURPAD:	Key conversion table for cursor keys, as above.

CODESC:	Scan codes for the keys that are used as cursor keys is SETUP.

CURPA52:	Key conversion table for cursor keys in VT52 mode.

KEYPA52:	Key conversion table for keypad keys in VT52 mode.

FKEYINI:	Top-row function key strings (F6 to F20)

- The order of the keys in the table is F6, F7, F8 etc.

- Each string is terminated by character ST (hex 9C)

FKEY100:		Top-row function key strings in the VT100 emulation mode

REPTABL:	Characters with national variations.

- These characters are handled, when generating the national replacement character sets (NRC-sets).

REP1:		Start of the first national replacement character set (Finnish).

-	Tables for other languages (German, Danish etc.) are in the order indicated by language parameter (SER_INI+5).

- 	Length of each table is 12 bytes

-	Each entry in the table is the character code in the DEC SUPPLEMENTAL CHARACTER SET (if bit 7 is zero) or in the USASCII character set (if bit 7 is set). The figure of the specified character is copied to the national set in place of the ASCII character. If entry in the table is Space (20h), that character is same as in ASCII set and is thus not copied.

CHARACTER SETS:

The character sets are stored as 8 to 16 bytes / character, depending on the font size. Each pixel on screen is represented by one bit. First byte is the topmost row of character pixels. Most significant bit is the leftmost pixel of character.

SET20:		Character generator for the USASCII character set.

128 characters, 16 bytes per character.

SET22:		Char. gen. for the DEC SUPPLEMENTAL character set.

128 characters, 16 bytes per character.

GRA2: 		Char. gen. for the DEC SPECIAL character set (semigraphics).

Contains only characters in range 60h to 7Fh (other characters are ASCII).

CHRIBM:	Char. gen. for the IBM PC character set

(IBM PC codes 80h to FFh)

DBUG_CH:	Debug Character  (Optical TSO only)

- Character sent to host in single character mode, if incomplete coordinate pair is detected and debug mode is enabled.

FING_UP:	Finger Up -character

- Character code sent to host computer, when 'key' is released in single character mode (if finger-up detect mode is enabled).

TSOTABL:	TSO single character conversion table.

- The table contains one byte for each TSO crossover point. How the characters are handled, depends on the value as follows:

00		Not sent to host. No click generated.

01 to 7F		Sent to host directly.

80 to DF	Converted to string using ALTTAB *

E0		Caps Lock key *

E1		Setup key *

FD		Ctrl key *

FE		ALT key *

FF		SHIFT key *

* Membrane TSO only

SHIFTBL:	Shift conversion table  (membrane TSO only)

-  For converting other than alphabetic keys.

-  Contains one byte for each character in range 20H to 3Fh.

ALTLEN:		Length of each entry in the ALTTAB.  Normally 04h.

ALTTAB:		ALT key / special key conversion table. (membrane TSO only)

- 4 bytes reserved for each key. If less than 4 bytes needed, fill rest which nulls (00h). If more than 4 bytes are needed, change the ALTLEN above.

-  First entry for code 80h, next 81h etc.

TSOEND:		String of characters appended to end of TSO message.  (Optical TSO only)

First byte contains number of characters  (normally 2, maximum 6)

Following bytes contain the characters sent  (normally CR, LF).

TESTMSG:	Text displayed at power-on, if jumper J10.7 is closed.

First byte is the length of the text.