3.  USING THE TERMINAL

3.1.  SCREEN FORMAT

The AP988200 uses character based display format, in contrast to the pixel graphics display. All drawing to the display is performed in the units of characters. The characters displayed are selected from one of the character sets available, including ASCII set, multinational extension sets, semigraphics, etc. You may also download your own character sets. This feature makes it possible to display practically any graphics on the screen.

Fig. 3.1

The display screen is divided into two parts, as seen in the figure 3.1 :

1. Main display

Main display contains 24 lines, each of which can display up to 80 characters of text. All the text received from the host computer is normally displayed on the main display (unless redirected by Select Active Display command, see chapter 4.1.7). Four pages of main display are available, pages may be changed by appropriate commands from the host computer.

2. Status line

The lowermost line on the display screen is the status line. By default, it acts as indicator status line. It may also be used as host writeable status line, or disabled entirely (see chapters 3.3 and 4.1.7 for details).

The indicator status line is displayed in inverse video (light background). The status line automatically displays some useful information about the terminal state. The items are, from left to right:

§

^  		 ^			     ^			^	   ^

A				   B			              C			D	        E

(A)	Character entry mode: Insert mode or Replace mode. The mode is selected by host computer command, see chapter 4.1.3.

(B) 	Current cursor position. R = row, C = column.

(C)	Number of the page currently displayed on screen, 1 to 4.

(D)	Printer mode:

Ready 	-  Printer is ready but not currently printing

NotRdy	-  The printer is off-line

none 	-  Printer has not been on-line since the terminal was powered on

Auto		-  Auto Print mode (print each line at Line Feed)

Contrl	-  Printer Controller mode (all input is directed to printer)

PrtScr	-  Print Screen operation currently executing.

The mode is changed by using setup, by host computer command or by pressing the PRINT SCREEN key (F2). If nothing is displayed in this field, no printer operations have been tried since power-on.

(E)	At the far right in the status line is the serial error indicator. Nrmally, nothing is displayed here. Letter P indicates that a parity error has occurred on the last character received. Letter F indicates a framing error. Either error may be caused by a data transfer error due to noise on the serial line. Wrong baud, parity or databits setting may also cause the errors.

3.2  THE KEYBOARD

Many different types of keyboards can be used with the AP988200. However, a VT220-compatible keyboard is recommended. This chapter describes the use of each key on the VT220 keyboard. If IBM-PC keyboard is used, it emulates VT220 keyboard as closely as possible. However, some of the keys described here may not be available and some have different symbols engraved on keycaps.

VT220 keyboards are available with several different national key-layouts. Currently, the following national layouts are supported:

North America (USASCII)	Germany/Austria

United Kingdom			Italy

Denmark			Norway

Holland				Sweden

Finland				Switzerland (German)

France/Belgium			Switzerland (French)

Figure 3.2 shows the 105-key (USASCII) keyboard which consists of the following sections:

Main keypad

Editing keypad

Numeric keypad

Top-row function keys

Four indicator lights

Two audible indicators

Each of the sections is described in the following pages.

Fig. 3.2:  North American keyboard

3.2.1 MAIN KEYPAD

Fig. 3.3.: Main keypad

This part of keyboard (figure 3.3) operates like a standard typewriter keyboard. In addition to normal alphanumeric characters this keypad has the following special function keys:

Tab key sends a horizontal tab, which normally moves the cursor to the next tab stop.

Ctrl key pressed together with another key sends a control code to the host system. The interpretation of the control code is application dependent. Ctrl-key does not send any code when pressed alone.

Lock key pressed once makes the alphabetic keys send uppercase characters. Pressing Lock again makes the alphabetic keys send lowercase characters. The Lock key acts like the Shift-key for alphabetic part of the main keypad. Pressing Lock key several times toggles the Lock on and off. The Lock indicator light above the top-row function keys shows the state of the Lock-key. When the indicator light is on, the uppercase codes will be sent. Otherwise, the lowercase codes will be sent.

Shift key alone does not generate any keycode. It needs to be pressed together with another key. For alphabetic keys the Shift key is the uppercase selector. For other keys (like numeric keys in the top-row of the main keypad) Shift selects the upper character engraved in the keycaps. For some of the function keys (like F2/Print Screen) using the Shift key with them generates a predefined control function.

Return key sends a Carriage Return (CR) code, or CR + LF codes, if Line Feed mode is set. In some cases Return moves the cursor to the next line when editing text. In other cases Return signals the end of an operation. The interpretation is application dependent.

Õ (Delete) key sends a Delete (DEL) character. Normally, pressing the delete key erases one character to the left of the cursor. Typing Shift-Delete sends a Cancel (CAN) character.

Compose Character key lets you create characters that do not exist as standard keys on your keyboard. You start the compose sequence by pressing the Compose character key first, followed by two keys that you want to merge into one character according to the compose rules. If the compose sequence is incorrect, the Bell tone is generated and no character is composed. See chapter 3.2.7.  (On IBM-PC keyboard, ALT key acts as compose character key)

3.2.2 EDITING KEYPAD

Fig.3.4:  Editing keypad

This keypad (see figure 3.4) includes four cursor control keys (cursor right, left, up and down) with which you can move the visible cursor on the screen to any of the four directions.

The rest of the keys in this keypad have their editing functions engraved in their keycaps. Each key sends a special control string. The actual effect of these keys is application dependent. You should refer to the application software manuals of your host computer for the explanation of these keys.

3.2.3 NUMERIC KEYPAD

Fig. 3.5. Numeric keypad

This keypad (figure 3.5) lets you enter numeric data as you would with a standard calculator keyboard. Some of the keys ( PF1, PF2, PF3 and PF4) may have functions assigned to them by the application software. (On IBM-PC keyboard, Num Lock must be ON in order to use numeric keypad. If Num Lock is off, these keys act as cursor keys.)

3.2.4 TOP-ROW FUNCTION KEYS

Fig. 3.6:  Top-row function keys

Top-row function keys are shown in figure 3.6. The first five function keys (F1 - F5) have the following fixed meanings assigned to them:

F1 = Hold Screen freezes the screen display and stops any new characters from being displayed. The Hold Screen indicator light indicates the frozen state. Pressing the F1 again releases the hold and turns off the Hold Screen indicator light. (On IBM-PC keyboard, use Scroll Lock key instead.)

F2 = Print Screen works only if you have a local printer connected to the printer port of your AP988200. Pressing F2 sends the text on the screen to the printer ("hardcopy" of the screen).

Pressing Ctrl/F2 toggles the Auto Print mode on/off. In this mode the current display line is copied to printer when the cursor moves to the next line. The Auto Print mode indicator is shown on the status line ( = 25th line of the display screen).

F3 = SETUP mode toggle. Press F3 once and you see the setup-menu. Fill in the desired values in the fields of the setup-menu and, finally, press the F3 once more. This saves all the current setup values in the nonvolatile memory of the AP988200. See chapter 3.3 for details.

F4 = ESC. Sends an ESC control character. Note: This is a non-standard use of the F4 function key. The VT220 does not have an ESC-key. On VT220, F4 is used to control modem signals. On AP988200, only the CTS ( = Clear To Send) signal is provided for modem hardware handshake.

F5 = Break.  Sends a break on the host line.

F6 to F20 are function keys used by application programs. Each key sends a special control sequence. These keys are also used as programmable soft function keys, when pressed simultaneously with the shift key.

3.2.5 INDICATOR LIGHTS

Fig. 3.7:  The keyboard indicator lights

Hold Screen indicator, when lit, indicates that the screen output has been freezed by using F1 function key

Lock indicator shows the caps lock state selected by Lock key on the main keypad.

Compose indicator turns on when the Compose Character key or one of the diacritical keys is pressed. It is turned off by a successful completion of the compose-sequence or by an erroneous sequence. If the compose sequence was "illegal" the Bell tone is sounded.

Wait indicator turns on if the keyboard is prevented (locked) from sending information. The keyboard can be unlocked by a command from the host line.

3.2.6 AUDIBLE INDICATORS

The VT220-keyboard can generate both a key-click and a bell sounds. The key-click occurs every time you press a key on the keyboard, except when you press Shift- or Ctrl-keys. (These two keys do not generate any keycodes on their own. They modify keycodes sent by other keys.)

The Bell-sound is generated by the keyboard in following cases:

- When the terminal receives a BEL control character from the host computer

- After a compose character error

The durations and frequencies of the two audible indicators are:

Keyclick	:	approximately 2 milliseconds/2 kHz

Bell:		approximately 125 milliseconds/2 kHz

If  VT220-keyboard is not used on your system configuration, the bell sound is generated by on-board beeper on the AP988200 controller board. The on-board beeper also generates the 'key-click' sound for the Touch Screen Overlay.

3.2.7	COMPOSING CHARACTERS

In AP988200 you can use so called composite characters by using two-stroke or three-stroke compose sequences which enable you to create characters that you cannot produce with normal keystrokes.

The two-stroke compose sequence is initiated by entering one of the following diacritical marks: acute accent ('), grave accent (`), umlaut (¨), circumflex accent (^), tilde mark (~) or ring mark (o). Create the two-stroke compose sequence as follows:

1. Find the character you want to create in the first column of table 3.1.

2. Type the two characters in the third column of table 3.1 corresponding to the first column character you want to create. When you press the first character (the diacritical mark), the compose indicator turns on. After pressing the second key, the composite character is sent.

Example:

To create an È ( E grave), type ` followed by E.

Note: two-stroke compose sequences are not available with the North American (USASCII) keyboard.

A three-stroke compose sequence is created as follows:

1. Find the character you want to create in the first column of table 3.1.

2. Press the Compose Character-key first. The Compose-indicator light in the keyboard turns on, indicating the compose-mode.

3. Type the two characters in the second column of table 3.1 corresponding to the first column character you want to create. The composite characters marked with an asterisk (*) are order sensitive, and must be entered in the order given. Others may be entered in any order.

Example:

To create an - (A umlaut), press Compose Character, and then type " or ¨ followed by A  (or A followed by " or ¨ ).

If you succeeded in creating a "legal" compose sequence, the Compose-indicator turns off and the composite character will be transmitted to the application. If your sequence is invalid, it is aborted and the keyboard bell sounds.

You can abort a compose sequence by using the Delete-key. No character is sent to the application if a compose sequence is aborted.

Note: all the composite characters shown in table 3.1 are available only if the MULTINATIONAL mode is selected (see sections 1.3.3 and 3.3). In national mode only characters included in the selected NRC set can be created, other characters cause a bell sound to be generated.

The table 3.1 below lists all the compose sequences.  For 3-stroke sequence, press the compose-key first, then the two characters given in the table. The order of the characters entered is not significant expect for those composite characters marked with an asterisk (*). Note:  (space) means the space key.

TABLE 3.1:     C O M P O S I T E   C H A R A C T E R S

COMPOSITE CHARACTER		3-STROKE		2-STROKE

SEQUENCE		SEQUENCE

__________________________________________________________________________

"	(quotation mark)		"(space)			¨(space)

#	(number sign)			++

'	(apostrophe)			' (space)			' (space)

@	(commercial at)			aa or AA

[	(opening bracket)		((

\	(backslash)			// or /<

]	(closing bracket)			))

^	(circumflex accent)		^(space)			^(space)

'	(single quote)			' (space)			' (space)

{	(opening brace)			(-

|	(vertical line)			/^

}	(closing brace)			)-

~	(tilde)				~(space)		~(space)

¡	(inverted !)			!!

¢	(cent sign)			c/ or /c

£	(pound sign)			l- or L-

¥	(yen sign)			y- or Y-

¶	(section sign)			so or SO

¤	(currency sign)			xo or XO

©	(copyright sign)			co or CO

ª	(feminine ordinal indicator)	a- or A-

«	(angle quotation mark left)	<<

°	(degree sign)			0^ or (space) #		°(space)

±	(plus/minus sign)		+-

2	(superscript 2)			2^

3	(superscript 3)			3^

µ	(micro sign) *			/u or /U

?	(paragraph sign)			p! or P!

·	(middle dot)			.^

1	(superscript 1)			1^

º	(masculine ordinal indicator)	o- or O-

»	(angle quotation mark right)	>>

1/4	(fraction one-quarter) *		1 4

1/2	(fraction one-half) *		1 2

¿	(inverted question mark)		??

À	(A grave)			A`			` A

Á	(A acute)			A'			' A

Â	(A circumflex)			A^			^A

Ã	(A tilde)			A~			~A

-	(A umlaut)			A"  or  A¨		¨A

Å	(A ring)				A* or  A°		°A

Æ	(A E ligature) *			AE

Ç	(C cedilla)			C ,

È	(E grave)			E`			` E

É	(E acute)			E'			' E

Ê	(E circumflex)			E^			^E

Ë	(E umlaut)			E" or  E¨		¨E

Ì	(I grave)			I`			` I

Í	(I acute)			I'			' I

Î	(I circumflex)			I^			^I

Ï	(I umlaut)			I" or  I¨			¨I

Ñ	(N tilde)			N~			~N

Ò	(O grave)			O`			` O

Ó	(O acute)			O'			' O

Ô	(O circumflex)			O^			^O

Õ	(O tilde)			O~			~O

Ö	(O umlaut)			O" or  O¨		¨O

(O E ligature) *			O E

(O slash)			O/

Ù	(U grave)			U`			` U

Ú	(U acute)			U'			' U

Û	(U circumflex)			U^			^U

Ü	(U umlaut)			U" or  U¨		¨U

Y	(Y umlaut)			Y" or  Y¨		¨Y

(German small sharp s)		ss

à	(a grave)			a`			` a

á	(a acute)			a'			' a

â	(a circumflex)			a^			^a

ã	(a tilde)				a~			~a

ä	(a umlaut)			a" or a¨			¨a

å	(a ring)				a* or a o			o a

æ	(a e ligature) *			a e

ç	(c cedilla)			c ,

è	(e grave)			e`			` e

é	(e acute)			e'			' e

ê	(e circumflex)			e^			^e

ë	(e umlaut)			e" or e¨			¨e

ì	(i grave)				i`			` i

í	(i acute)				i'			' i

î	(i circumflex)			i^			^i

ï	(i umlaut)			i" or i¨			¨i

ñ	(n tilde)				n~			~n

ò	(o grave)			o`			` o

ó	(o acute)			o'			' o

ô	(o circumflex)			o^			^o

õ	(o tilde)				o~			~o

ö	(o umlaut)			o" or o¨			¨o

(o e ligature) *			o e

(o slash)			o/

ù	(u grave)			u`			` u

ú	(u acute)			u'			' u

û	(u circumflex)			u^			^u

ü	(u umlaut)			u" or u¨			¨u

(y umlaut)			y" or y¨			¨y

* Order sensitive

3.3.  SETUP MODE

Setup mode is entered by pressing the SETUP key (F3). To exit setup mode, press SETUP key again. When the setup mode is entered, the terminal is turned into local mode and the setup screen is displayed.

AP988200 V 1.48

S  E  T  U  P

BAUD RATE   DATA BITS   STOP BITS   PARITY      HANDSHAKE    DUPLEX

9600          8           1        none       XON/XOFF      FULL

EMULATION   LANGUAGE    NAT.MODE    KEYBOARD    KB MODE      RET KEY

VT-220 7b   Finnish     Multinat.   Finnish     Typewrit       CR

CURSOR      CUR.MODE    SCROLL      STATLINE    SCREEN      SCR SAVER

Block        Blink       Jump       Indicator   reverse      30 min

PRINTER     PR BAUD     PR BITS     PR PARITY               NET ADDR

normal        4800        8+1         none                    (00)

Cursor keys = Select item,  Space / Enter = Change item,   F3 = exit

Ctrl-C = Cancel,  Ctrl-D = Default values,  Ctrl-P = Power-up values

Fig 3.8. The SETUP screen

To select an item to change, use Left, Right, Up and Down arrow keys, or Carriage Return to go to the beginning of the next line.

To change item, use Space (previous item) or numeric keypad Enter-key (next item). Old values (the values that were active when setup mode was entered) can be restored by pressing Ctrl-C. To restore values that were active when power was switched on, press Ctrl-P. To read default values from the EPROM, press Ctrl-D.

To exit setup mode, press SETUP (F3) or Esc ( Ctrl-[ or F4 ). The parameters are automatically saved into the non-volatile RAM, the screen is restored and terminal is set on-line. The new setup values take effect, when you exit the setup.

AP988200 uses display page 4 for the setup screen. Therefore, the contents of the page 4 are lost when entering SETUP mode.

On the following pages, each setup item is explained.

TABLE 3.2:     S E T U P   I T E M S

Item		Explanation			Available values

Baud		Host line baud rate		150 to 38400 bps

Data bits		Number of data bits		7 or 8, with no-check option

Stop bits		Number of stop bits for transmit	1 or 2

Parity		Parity bit			Even, Odd, no-check, or None

Handshake	Handshake protocol		XON/XOFF , CTS or None

Duplex		Local echo, Online mode		Full, Half, BlkMod or Local

Emulation	Terminal emulation level		VT52, VT100, VT220 7b, VT220 8b  								or Monitor

Language		National replacement char set	(12 sets available)

Nat.Mode		National mode			National or Multinational

Keyboard		Keyboard language		(12 options available)

Kb Mode		Keyboard mode			Typewriter or Data processing

CR Key		Return key function		CR or CR+LF

Cursor		Cursor type			Line or Block, or cursor off

Cur mode		Cursor mode			Blinking or Steady

Scroll		Soft scroll mode			Soft or Jump

Statline		Statusline type			Indicator, User or Off

Screen		Screen mode			Normal or Reverse

Scr Sav		Screen saver time		5, 15, 30 or 60 min, or off

Printer		Printing mode			Off, Autoprint or Printer controller

Pr Baud		Printer baud rate		150 to 38400 bps

Pr Bits		Printer bits			7+1, 7+2, 8+1 or 8+2

Pr Parity		Printer parity			Even, Odd or None

Net Addr		Network address			00 to 63

BAUD:

Selects the data transfer rate for the host serial interface.

Note:  Since the maximum transfer rate allowed by the RS232C standard is 20 kBaud, the RS485 interface is recommended if 38400 bps transfer rate is required.

DATA BITS:

Selects the number of data bits in character, excluding parity and start/stop bits. Number of data bits must be set equal to the setting on the host computer. If the setting is wrong,  false characters may be displayed, or framing/parity errors may occur.

When a framing error occurs, an F indicator is lit on the far right end of the status line. If option 7 or 8 is selected, the characters with framing error are not displayed. If 7 or 8 with n/ck (no check) -option is selected, all the characters are displayed regardless of the error status.

STOP BITS:

Number of stop bits for characters. This is only significant for the transmitted characters. 2 stop bits gives the host computer more time to receive characters, but slows down the transfer speed for appr. 10%.

PARITY:

Selects the usage of the parity check bit on received/transmitted characters.

If parity is set to EVEN or ODD, an even or odd parity bit is added to transmitted characters, and received characters with parity error are not displayed on the screen. Instead, a P indicator is lit at the right end of the status line.

If EVEN N/CK or ODD N/CK is selected, a parity bit is added to transmitted characters, but there is no parity check for received characters.

If NONE option is selected, the parity bit is not added to transmitted characters, and there is no parity check for received characters.

HANDSHAKE:

Handshaking protocol used for host communication: XON/XOFF or hardware CTS handshake. See chapter 3.4 for details. The CTS handshake option affects only the CTS output. Hardware handshake input option and the connector pins used for handshake are configured by on-board jumpers.

DUPLEX:

If HALF duplex is selected, the characters entered from the keyboard are also locally displayed on screen, and the host computer should not echo them back.

If FULL duplex is selected, the characters are only sent to the host computer which may then echo them back to the terminal.

In LOCAL mode, the characters are only displayed on screen and not sent to the host computer. The characters received from host are not displayed.

BLK MODE is the block (line) mode.  Characters entered from the keyboard are displayed on screen, but not sent to the host computer. When Return-key is pressed, the whole line of text is sent to host. Characters received from the host are displayed normally.

EMULATION:

VT-220:  if this option is selected, all the VT220 (ANSI) commands listed in the chapter 4.1 are allowed. You may also select, if the commands generated by cursor- and function keys are sent to host in 8-bit format (VT-220 8b) or as 7-bit extensions (VT-220 7b).

VT-100:  this mode is a subset of the VT220 mode, and should be selected, if strict VT100 compatibility is required. In VT100 mode, some of the commands (marked with an asterisk on chapter 4.1) are ignored, terminal-id report is changed to VT100 id, and some keys on the keyboard have different meaning.

VT-52:  this is a non-ANSI mode that provides compatibility with Digital VT52 terminal. In this mode, a special command set is used (see chapter 4.2), and some keys (e.g. keypad and cursor keys) send different strings (see chapter 4.3 ).

MONITOR mode may be used for test purposes. In this mode, none of the control characters or escape sequences are interpreted as commands, but instead displayed on the screen. Keyboard acts as in VT220 mode.

LANGUAGE:

Selects one of the 12 national replacement character (NRC) sets used to display text on screen, if NATIONAL mode is selected from the NAT MODE field. If character set is changed from one language to another, all national characters on screen are replaced by those of the new set. (Only affects the 7-bit national replacement characters, does not affect the 8-bit characters on the DEC multinational character set.)

NRC set can also be selected by host computer command. See chapter 4.1.6.

NAT.MODE:

Selects NATIONAL or MULTINATIONAL mode. In national mode, the terminal sends 7-bit characters from the national replacement character set selected by LANGUAGE field. In multinational mode, the terminal sends 8-bit characters from USASCII set and DEC multinational character set.

Unlike the VT220, the AP988200 allows 8-bit characters to be received and displayed even if the national mode has been selected. Also, the display language and keyboard language can be selected independently.

KEYBOARD:

Selects the keyboard language (national keyboard layout). You should select the keyboard layout that matches the keyboard currently connected to the terminal.

Unlike on VT220, the keyboard selected does not affect the NRC set used. The NRC set is selected from LANGUAGE field.

KB MODE:

If TYPEWRITER mode is selected, the characters marked on the left side of the keycaps are used. If DATA PROCESSING mode is selected, the characters marked on the right side of the keycaps are used. This option is ignored, if USA keyboard is in use.

CR KEY:

Selects the function of RETURN [¿] key and the effect of any LF control characters received. This field has the same effect as NEW LINE MODE command.

If CR is selected, the return key sends only a carriage return, and any received LF control characters are interpreted as LF only.

If CR+LF is selected, the return key send two characters, CR and LF. Also, any received LF control characters are interpreted as new line (CR+LF).

CURSOR:

Selects  block cursor, line cursor or cursor off.

CUR.TYP:

Cursor type: blinking or steady. The cursor type and mode may also be set by host computer commands, but the status is saved in the non-volatile memory only when set from the SETUP.

SCROLL:

Selects how the screen is scrolled up or down, when the top or bottom of the screen is reached. If JUMP scroll is selected, the screen scrolls line by line, allowing fast display of text. If SMOOTH scroll is selected, the screen is scrolled pixel by pixel, at rate of about 3 lines / sec. This slows down the display, but allows text to be read during scrolling.

STATLINE:

Selects the type of the status line. If INDICATOR is selected, some information about the current state of the terminal is displayed in the status line in inverse video (see chapter 3.1).  If USER is selected, the host computer can write to the status line (by first issuing a command that redirects the output to the status line). The status line can be turned off by selecting OFF.

SCREEN:

Selects the screen mode: normal or reverse video.

SCR SAV:

The SCREEN SAVER time. The screen saver blanks the screen after the defined time has passed since the last keypress or last character received from the host. The screen is automatically turned on when a key is pressed or a character is received from the host. If the screen saver is set OFF, the screen will not blank.

PRINTER:

Printing mode. If AUTOPR (Autoprint) is selected, every Line Feed causes the current line to be printed before moving cursor to next line.

If CONTROLL (Printer Controller Mode) is selected, the terminal sends all the received characters directly to the printer without displaying them.

If NORMAL mode is selected, the printer operations are performed only by host computer command or by pressing F2 key.

The printer mode may also be changed by host computer commands or by Ctrl-F2 key.

PR BAUD:

Selects the data transfer rate for the printer interface.

Note:  Baud rate 4800, with 8+1 bits and no parity is recommended for the printer interface. If any other settings are selected, the keyboard is not read during the printer operations.

PR BITS:

Selects the number of data and stop bits for the printer interface.

PR PARITY:

Selects the parity check option for the printer interface:  EVEN,  ODD  or  none.

NET ADDR:

Sets the net address used for multipoint communications. If multiple terminals are connected to the same serial line (e.g. by using RS485), this allows the host computer to selectively write on any of the terminals and to identify the terminal that sends the characters. (See chapter 4.1.15).

If address is set to 00, the terminal receives all the messages.

3.4.  HOST COMMUNICATIONS

3.4.1. TRANSMIT AND RECEIVE BUFFERS

If only displayable characters are received (without scrolling), the AP988200 is capable of displaying all characters in real time up to the speed of 19200 bps. At 38400 bps, the actual display speed is slightly lower than the speed characters are received. Scrolling reduces the speed, especially if smooth scroll is selected. Some commands, such as Insert or Delete Line, change Line Attributes, may also be slow to execute.

The data received from the host computer is buffered with a 1 k byte receive buffer. After the commands have been interpreted, the displayable characters are put into another buffer of about 1000 characters and written to display during the vertical blanking time. The host can thus continue to send data at full speed even if slow commands are executed. Approximately one full screen of text may be stored in the buffers.

The data flow between the host and the terminal is controlled either by XON-XOFF protocol or hardware CTS handshake, selectable by on-board jumpers and the SETUP screen.

When the free buffer space decreases below 400 bytes, the terminal sends an XOFF or  turns the CTS line high. If the host does not stop sending the characters, and the buffer goes full, an XOFF is send at each character received. Characters lost because of buffer full condition are displayed on screen as reverse question mark.

After sending the XOFF, the terminal reads characters from the buffer. When most of the characters are read, and there are only 100 unread bytes left in the buffer, the terminal sends an XON to the host or turns CTS line low again.

There is also a 256 byte transmit buffer for characters to be sent to the host computer. Host may stop the terminal from sending characters with XON-XOFF protocol or hardware handshake, as above. Characters received from the keyboard or from Touch Screen Overlay are then stored in the transmit buffer, until transmission is re-enabled by the host computer.

The host may also lock the keyboard and/or disable the Touch Screen Overlay, in which case the characters from these devices are discarded, not stored in the transmit buffer.

3.4.2. RS485 INTERFACE

The optional RS485 interface allows up to 32 terminals to be connected on a single party line, using a single twisted pair cable (bus configuration). An add-on board may be installed on the AP988200, to allow full duplex communications via two twisted pair cables and to deliver compatibility with the RS422 interface. The receiver(s) of the data transmitted through the party line is selected by using network addresses. See chapter 4.1.15 for details.

Cable length on either configuration may be up to 1500 metres. However, if multiple terminals are connected to the cable, the maximum data transfer distance is reduced. If more terminals or longer distances are required, repeaters may be used.

When the single twisted pair cable (bus configuration) is used, any terminal or host computer connected to the party line can send data to any other unit. However, only one terminal can send data at any given time. A handshaking mechanism is therefore needed in order to avoid collision. The host computer is normally responsible of distributing the resources.

When the two twisted pair cable configuration is used (with the add-on board), the host computer may send data to the terminals while simultaneously receiving data from a terminal. This makes it easier to implement the communications software on the host computer. In this case, one terminal can not send data to another terminal. Logically, this is a star configuration network.

The RS485 interface with two twisted pair cable may also be used in point-to-point communications, just to allow longer distances and higher baud rates. In this case, the interface can be used just as the normal RS232C interface.

When long cables are used, terminating resistors must be used on both ends of the cable. For multipoint communications, additional biasing resistors must be used to keep the bus in legal state while none of the terminals are transmitting. An application note is available for more details.

See chapter 6.2 for jumper settings for the RS485 interface.