_____________________________________________________________________________________

1.   INITIALIZE 								I

I res , col <CR>

Initializes the graphics mode to default state. Clears the display memory, moves pen position to (0,0), sets origin and viewport to (0,0), sets line type to solid and line width to 1 pixel.

This command is performed automatically, when entering the graphics mode from terminal mode (but not when entering from mixed mode).

Optional parameter res can be used with the 640.480 display panels. It selects the display resolution for graphics mode. Accepted values are:

res	resolution mode

400	640.400 display resolution

480	640.480 display resolution

After entering the graphics mode, the display is set to 640.400 mode. This mode gives higher screen refresh rate and compatibility with software created for 640.400 displays.

640.480 mode allows the whole screen to be used for graphics.

Optional parameter col selects the number of colors. This option is only available with full color displays, not with red-green EL displays.  Increasing the number of colors reduces the virtual screen size.

col	colors available		Virtual screen size (HxV)

4	4 colors			2048 x 1024

16	16 colors		1024 x 1024

Note: in the 16 color mode, some of the operations (such as XCOPY) are slower than in 4 color mode or on monochrome display.

If second parameter is not given, the color mode does not change. If no parameters are given, neither display resolution nor color mode is not changed.

Examples:

I			Initializes the graphics mode, does not change display resolution

I480			Initializes the graphics mode and selects 640.480 display resolution.

I400,16			Initializes, and selects 640.480 resolution with 16 colors.

_____________________________________________________________________________________

_____________________________________________________________________________________

21.   SET INK  (COLOR)							SI

SI color <CR>

Selects the color for all subsequent drawing operations. Parameter color is the color number. The range of colors available depends on display type in use. In the following table, the colors are listed for red-green EL-display in 4-color mode, and full color display in 4-color and 16-color mode:

Full color		Red-Green

Parameter	16 colors	4 colors		4 colors

1	Red	Red	Red

2	Green	Green	Green

3	Orange	White	Yellow

4	Blue

5	Magenta

6	Cyan

7	White

8	Grey

9	Bright Red

10	Bright Green

11	Bright Yellow

12	Bright Blue

13	Bright Magenta

14	Bright Cyan

15	Bright White

The color mode is based on bit planes. If, for example, red color is selected, only red bit plane is written or erased, and green bit plane is not affected. Writing red over green produces yellow.

On monochrome displays, this command is ignored.

_____________________________________________________________________________________