4.  TERMINAL MODE

This section describes the commands and functions available in the Terminal Mode and Mixed Mode. Terminal Mode is a text only mode, Mixed Mode allows HiRes graphics to be mixed with text.

The AP988300 is in Terminal Mode after power-on or reset or after exiting the SETUP mode. The terminal mode can also be selected with command ESC F, the mixed mode can be selected with command ESC X. You can exit terminal mode or mixed mode and enter Graphics Mode with command ESC G. (For more information about operating modes, see chapter 1.2.)

4.1.  ANSI (VT-100) COMMANDS

This chapter describes the ANSI X3.64 (VT-100) commands supported by the Alphasoft.-300.  These commands are available, if the VT100 emulation has been selected in the EMULATION field on the SETUP.

4.1.1.  CONTROL FUNCTIONS

The control functions are used to make the terminal  perform special  actions such as editing data, query terminal status etc.  There are single-character and multiple-character control functions.

CONTROL CHARACTERS

The single-character control functions are the ASCII control characters in range 00 to 1F (hex). The control characters are listed in the table 4.1.

CONTROL SEQUENCES

There are three basic types of multiple-character control functions:  escape sequences, control sequences and device control strings.

An escape sequence uses two or more bytes to define a control function. Escape sequences do not include variable parameters. The format of an escape sequence is:

ESC I F

where ESC (hex 1B) introduces the sequence, I is zero or more intermediate characters in range 20 to 2F hex, and F is the final character in range 30 to 7E hex.

The control sequence uses two or more bytes to define a control function. Control sequences usually include variable parameters. The general format for a control sequence is:

CSI P...P I...I F

Where CSI is the Control Sequence Introducer, a 2-character sequence Esc [  (hex 1B 5B).

P...P  is parameter string, which may contain zero or more numeric parameters, separated by semicolons (;). Each parameter is a positive integer of up to four digits. If the first character in the parameter string is in range 3C to 3F ('<' to '?'), it indicates that private parameters follow.

I...I  are zero or more intermediate characters in range 20 to 2F hex.

F  is the final character in range 40 to 7E hex.

The command is executed after the final character has been received. If the sequence contains an error, it is ignored and an error character (checkerboard) is displayed.

The device control strings (DCS), like control sequences, use two or more bytes to define specific control functions. However, it also includes a data string. The format is:

DCS P...P I...I F Data_string ST

DCS is the device control string introducer, a 2-character sequence ESC P (hex 1B 50).

P...P  is parameter string, as in the control strings above.

I...I  are zero or more intermediate characters in range 20 to 2F hex.

F  is the final character in range 40 to 7E hex. The intermediate and final characters define the control function. All the characters after final character are interpreted as the data string.

Data_string contains the data definitions. It may contain any number of bytes, depending on the function it is defining.

ST is the string terminator, which indicates the end of the string. It is either an C1 control character 9C hex or equivalent 7-bit sequence  ESC \ (hex 1B 5C).

Any undefined device control strings are ignored. None of the characters between the DCS and ST are displayed.

FORMAT OF COMMAND DESCRIPTIONS:

On the following pages, each control sequence is explained in detail using following format:

__________________________________________________________________________________

Nr	Name									Mnem

ASCII				HEX

Description

__________________________________________________________________________________

Where Nr is the number of the command (for reference only), name is the name of the command, mnem is the ANSI mnemonic for the command, ASCII is the command sequence in the ASCII format, HEX is the sequence in hexadecimal format.  After these is the description of the command.

The hex code for CSI is shown as 7-bit code extension (hex 1B 5B). The future versions of the Alphasoft-300 may also support 8-bit environment, where the sequence may be replaced with the 8-bit code (hex 9B).

Pn, Pn1, Pn2 etc. represent  numeric parameters. Those are used directly as numeric value.

Ps, Ps1, Ps2 etc. represents selective parameters. Those are numeric values that select the action of the command.

Table 4.1:   CONTROL CHARACTERS:

hex  	ascii 	key 		function

00	NUL 	Ctrl-@	      	No effect

01	SOH	Ctrl-A		Start of header (network command)

05	ENQ	Ctrl-E	Send answerback message

07	BEL	Ctrl-G		Sound bell

08	BS	Ctrl-H 		Backspace

09	HT 	Ctrl-I  or  TAB  	Tabulation

0A	LF 	Ctrl-J     		Line feed

0B	VT 	Ctrl-K  		Treated as LF

0C	FF 	Ctrl-L       		Treated as LF

0D	CR 	Ctrl-M  or Return  	Carriage return

0E 	SO 	Ctrl-N      	 	Shift Out, select G1 char set

0F	SI 	Ctrl-O       	Shift In,  select G0 char set

11	DC1	Ctrl-Q       	XON  (restart transmission)

12	DC2	Ctrl-R		Enable transmission (network command)

13 	DC3 	Ctrl-S       		XOFF (halt transmission)

18 	CAN	Ctrl-X       	Cancel escape sequence

1A	SUB	Ctrl-Z       		Cancel escape sequence

1B	ESC 	ESC          		Start escape sequence

On the following pages, the commands are sorted in functional groups as follows:

Title:					page:

Cursor control				4-4

Insert & Delete				4-8

Tabulation				4-10

Character and line attributes		4-11

Soft character sets			4-14

Screen control commands		4-15

Set mode					4-17

Reset and test commands		4-22

Report commands			4-23

Printer commands			4-28

Set line parameters			4-Error: Reference source not found

Touch Screen commands		4-Error: Reference source not found

Multipoint communications		4-Error: Reference source not found

C U R S O R  C O N T R O L

__________________________________________________________________________________

1.  CURSOR UP   								CUU

ESC [ Pn A   		1B 5B Pn 41

This sequence moves the cursor Pn (any decimal number) lines up. If the cursor is attempted to   move above the top margin, the cursor will stop at the top margin. No scroll down will occur. If the parameter Pn is not given, the default is 1.

__________________________________________________________________________________

2.  CURSOR DOWN  								CUD

ESC [ Pn B   			1B 5B Pn 42

This sequence moves the cursor Pn lines down. The cursor will not move bellow the bottom   margin. No scroll up will occur. If Pn is not given, the default is 1.

__________________________________________________________________________________

3.  CURSOR FORWARD  							CUF

ESC [ Pn C   		1B 5B Pn 43

This sequence moves the cursor Pn columns to the right. The cursor will stop at the rightmost   column. If Pn  is not given, the default is 1.

__________________________________________________________________________________

4.  CURSOR BACKWARD  							CUB

ESC [ Pn D   		1B 5B Pn 44

This sequence moves the cursor Pn of columns to the left. The cursor will stop at the leftmost   column. If Pn is not given, default is 1.

__________________________________________________________________________________

5.  CURSOR POSITION  							CUP

ESC [ Pn1 ; Pn2 H  		1B 5B Pn1 3B Pn2 48

This sequence positions the cursor at the Pn1 line in the Pn2 column. Top left corner of the screen is line 1, column 1. Line and column numbers must be separated by a semi-colon. Parameters not given default to 1. Parameters outside range are truncated to largest allowed value.

See also command 50 (Cursor position report) on page 4-23.

__________________________________________________________________________________

6.  CURSOR HOME 								CUP

ESC [ H   			1B 5B 48

This function moves the cursor to the Home position (top left corner of the screen or scrolling region, depending on the setting of Origin Mode).

__________________________________________________________________________________

__________________________________________________________________________________

7.  INDEX  									IND

ESC D   			1B 44

This sequence moves the cursor one line down in the same column position. If the cursor is in the last   line, a scroll up will occur.

__________________________________________________________________________________

8. NEXT LINE									NEL

ESC E   			1B 45

This sequence moves the cursor to the first (leftmost) position on the next line down. If the cursor is in   the bottom margin, a scroll up will occur.

__________________________________________________________________________________

9. REVERSE INDEX  								RI

ESC M   			1B 4D

This sequence moves the cursor one line up in the same column. If the cursor is in the topmost line, a  scroll down will occur.

__________________________________________________________________________________

10.  SAVE CURSOR  								SC

ESC 7   			1B 37

This sequence causes the cursor position and many of the terminal settings to be saved for later use. The following settings are saved:

- cursor position (row, col)

- cursor on/off status

- current selection of character sets

- character attributes

- newline enable flag

- wrap around flag

- insert / replace mode flag

- keyboard normal / application mode flags

__________________________________________________________________________________

11.  RESTORE CURSOR							RC

ESC 8   			1B 38

This sequence restores the cursor position, attributes and character set saved previously with the SC command. If  no SC command is performed after power-on, the power-on default values are used.

__________________________________________________________________________________

__________________________________________________________________________________

12a.  CURSOR ON/OFF  					DECTCEM

To set: 		ESC [ ? 2 5 h   	1B 5B 3F 32 35 68

To reset:		ESC [ ? 2 5 l   	1B 5B 3F 32 35 6C

When set, the cursor is displayed normally.  When reset, cursor is not displayed.

Programming tip: Turning the cursor off increases the text writing speed by 10-50%, depending on the display type, scrolling mode etc. This may be useful, when lots of text has to be written on screen rapidly at high baud rates.

__________________________________________________________________________________

12b.  CURSOR OFF/ON  							-

To set: 		ESC [ ? 3 5 h   	1B 5B 3F 33 35 68

To reset:		ESC [ ? 3 5 l   	1B 5B 3F 33 35 6C

When set, the cursor is not displayed.  When reset, cursor is displayed normally.

Note: This command syntax is retained for compatibility with old terminals. For compatibility with other terminals, it is recommended to use command 12a instead.

__________________________________________________________________________________

13. CURSOR BLINK / STEADY							-

To set: 		ESC [ ? 3 4 h   	1B 5B 3F 33 34 68

To reset:		ESC [ ? 3 4 l   	1B 5B 3F 33 34 6C

When set, selects the blinking cursor. When reset, selects non-blinking cursor.

__________________________________________________________________________________

14. CURSOR LINE / BLOCK 							-

To set:		ESC [ ? 3 3 h   	1B 5B 3F 33 33 68

To reset:		ESC [ ? 3 3 l   	1B 5B 3F 33 33 6C

When set,  selects block type cursor. When reset, selects underline type cursor.

__________________________________________________________________________________

I N S E R T   &   D E L E T E

__________________________________________________________________________________

15. DELETE CHARACTER							DCH

ESC [ Pn P  			1B 5B Pn 50

Deletes Pn characters on a single line, starting with the character at the cursor position. Any   characters remaining on the line to the right of the last deleted character will be moved Pn spaces  left, to fill the gap created by the deletion.  If Pn is not given, it defaults to 1.

__________________________________________________________________________________

16. DELETE LINE								DL

ESC [ Pn M  			1B 5B Pn 4D

Deletes Pn complete lines from the display, starting with the line which contains the cursor. Remaining lines below cursor to the bottom margin move up to fill the gap, and Pn blank  lines are added to the bottom of screen or scrolling region. Cursor moves to the beginning of the first line after deleted block. If Pn is not given, it defaults to 1.

__________________________________________________________________________________

17. INSERT LINE								IL

ESC [ Pn L  			1B 5B Pn 4C

Creates a blank area of Pn lines by moving all lines from the cursor line to bottom margin down by Pn   lines.  Cursor is moved to the beginning of the first blank line created.

__________________________________________________________________________________

18. INSERT / REPLACEMENT MODE						IRM

Insert mode:	 	ESC [ 4 h 		1B 5B 43 68

Replace mode:		ESC [ 4 l		1B 5B 43 6C

In insert mode each new character received is inserted at the cursor position and all characters from the cursor position to the end of line are shifted one place to the right.

In replace mode the character received replaces the character at current cursor position.

__________________________________________________________________________________

19. ERASE IN DISPLAY							ED

ESC [ Ps  J			1B 5B Ps 4A

This sequence erases certain parts of the display screen according to the parameter Ps and the   cursor location. If Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the screen.

1	Erases from the start of the screen to the cursor.

2	Erases the entire screen.

Programming tip:  Use parameter value 2 for fastest possible screen erase.

__________________________________________________________________________________

__________________________________________________________________________________

20. ERASE IN LINE								EL

ESC [ Ps K			1B 5B Ps 4B

This sequence erases certain parts of the display screen according to mode and the cursor location. If Ps is not given, it defaults to 0.

Ps	Resulting action

0	Erases from the cursor to the end of the line

1	Erases from the start of the line to the cursor

2	Erases the entire line

__________________________________________________________________________________

21. HOME AND CLEAR 							-

ESC [ Ps X 			1B 5B Ps 58

The screen is cleared and the cursor is moved to the Home position. The parameter Ps is optional and has currently no effect.

Note:   This is not a standard VT100 command.  For future compatibility, use of this command is not recommended.

__________________________________________________________________________________

T A B U L A T I O N

__________________________________________________________________________________

22. SET TAB STOP								HTS

ESC H   			1B 48

This sequence sets one horizontal tab stop at the current cursor location.

__________________________________________________________________________________

23. CLEAR TAB STOPS							TBC

ESC [ Ps g 			1B 5B Ps 67

This sequence clears tab stops as specified by the parameter Ps:

Ps	Resulting action

0	Clear tab stop at current cursor column

3	Clear all tab stops

If Ps is not given, it defaults to 0.

__________________________________________________________________________________

24. TABULATION FORWARD							HT

HT   				09

The cursor is moved to the right, to the next tab stop on the same line. Does not wrap around to the  next line.

__________________________________________________________________________________

25. TABULATION BACKWARD

ESC B   			1B 42

The cursor is moved left to the first tab stop backwards, on the same line. If the cursor is in the first   column of the line, it will not move.

__________________________________________________________________________________

C H A R A C T E R   A N D   L I N E   A T T R I B U T E S

__________________________________________________________________________________

26. SELECT GRAPHIC RENDITION						SGR

ESC [ Ps m 			1B 5B Ps 6D

This sequence selects the character video attributes and color. Ps represents one or more parameters, separated by semi-colons. Each parameter selects an attribute:

Ps	Result

0	Attributes off  (normal text)

1	Bold characters

4	Underlined characters

7	Reverse video

31	Red  *)

32	Green  *)

33	Yellow  *)

*)  Requires color display

All characters following the sequence are displayed with the attributes selected, until the next SGR sequence. If no parameters are given, Ps defaults to 0, turning all attributes off.

Example:

ESC [ 0 ; 1 ; 4 m

The above sequence clears all previous attributes, then sets bold and underline attributes.

__________________________________________________________________________________

27. DOUBLE HEIGHT LINE - TOP							DHLT

ESC # 3   			1B 23 33

This sequence causes the cursor line to become the top half of a double-width, double-height line. If the line was single-width, all characters to the right of the center of the screen are lost. If the cursor is located to the left of the screen center, it will remain in the same character position. If the cursor is to  the right of the screen center, it is moved to the rightmost column of the line.

__________________________________________________________________________________

28. DOUBLE HEIGHT LINE - BOTTOM						DHLB

ESC # 4   			1B 23 34

This sequence causes the line to become the bottom half of a double-width, double-height line. The ESC # 3 and ESC # 4 sequences are to be used in conjunction with two equal lines of text   to produce one double-width, double-height line. Double-width limits a line to 40 characters.

__________________________________________________________________________________

__________________________________________________________________________________

29. SINGLE-WIDTH LINE							SWL

ESC # 5   			1B 23 35

This sequence causes the cursor line to become single-width, single-height (the default character size).

__________________________________________________________________________________

30. DOUBLE-WIDTH LINE							DWL

ESC # 6   			1B 23 36

This sequence causes the cursor line to become a double-width line. Double-width limits a line to 40 characters. If the line was previously a single-width line, then all characters to the right of the center of the screen are lost.

__________________________________________________________________________________

31. SELECT CHARACTER SET							SCS

To select G0 set:	ESC ( Dscs 		1B 28 Dscs

To select G1 set:	ESC ) Dscs		1B 29 Dscs

These sequences are used to designate the G0 and G1 character generator to the hard character sets. For each character generator, the set is selected by parameter Dscs. The following character sets are available:

Dscs		Character set

B		ASCII character set

0		DEC special graphics set  (semigraphic)

1		IBM PC extended character set    *)

<		DEC supplemental character set  (multinational)   *)

National replacement sets:   **)

A		British

4		Dutch

5 or C	Finnish

R		French

K		German

Y		Italian

6 or E	Norwegian

Z		Spanish

7 or H	Swedish

=		Swiss

*)	IBM character set is not a standard VT100 feature.

DEC supplemental set is a VT220 feature not found on the standard VT100.

**)	National sets are VT220 feature not found on the standard VT100. Only one national replacement set can be designated at a time. However, unlike VT220, AP988300 allows national replacement set to be changed at any time. Unlike the AP988200, the national characters already on screen are not affected. Thus, multiple national character sets can be displayed on screen simultaneously (Note, however, that you can do this easier by using 8-bit character codes with IBM or DEC multinational character set).

In addition, any previously downloaded soft character sets can be selected, by using the name defined in the Dscs field of the soft font download command.  Se "soft character sets" on page 4-14.

Once a character set has been designated using the above commands, it must be mapped to active set using the "Mapping the character sets" commands described below.

__________________________________________________________________________________

32. MAPPING THE CHARACTER SETS

After you have designated a character set as G0 or G1, you must map the set into the in-use table as GL or GR. GL defines the character set to be used for characters in range 20 to 7F (hex), GR defines the character set to be used for characters in range 80 to FF (hex).  By default, the G0 set is mapped to GL and G1 set to GR. You can change the GL mapping using the locking-shift control functions below:

Sequence

Name		ASCII	Hex	Function

LS0		  SI		 OF	Map G0 into GL

LS1		  SO		 0E	Map G1 into GL

On current Alphasoft-300 version, the G1 set is always mapped to GR (characters 80 to FF hex). This allows 8-bit character sets to be used. Note, however, that the 8-bit character set is a VT220 feature not found on standard VT100.

Unlike on the VT220, the characters in range 80 to 9F hex can also be used. This gives access to the full IBM-PC character set. If compatibility with VT220 is needed, these characters should not be used.

__________________________________________________________________________________

S O F T   C H A R A C T E R   S E T S

In addition to the build-in character sets, you may design your own character sets and download them to the terminal. You can download full character sets, or replace just one or few characters on the existing character sets.

You may use VT220 or VT320 character sets on the AP988300, but because the character cell sizes are different on these terminals, the characters may appear different than in the original environment.

The character cell size varies depending on the display panel used as follows:

640.400	640.350	640.200	512.256	320.256

Cell width		8	8	8	7	8

Cell height		16	14	8	10	10

Body width		7	7	7	6	7

Body height		11	9	7	7	7

Ascender height		3	3	2	2	2

Descender height		2	2	1	2	2

Spacing after character	1	1	1	1	1

Spacing above character	2	1	0	1	1

Spacing below character	3	2	0	2	1

While downloading, each character is divided into sixtels. A sixtel is a 6-bit binary value that represents a vertical column of 6 pixels on the screen. Eight adjacent sixtels horizontally forms a group. The character downloaded contains up to three groups. See the picture below.

The 8 by 16 pixel character cell shown above consists of 3 groups. On the last group, only the 4 least significant bits are used. The least significant bit of the first sixtel is the upper left corner of the character.

__________________________________________________________________________________

33. DOWNLOAD SOFT CHARACTERS				      DECDLD

DCS Pfn ; Pcn  ; Pe ; Pcms ; Pw ; Pt  { Dscs Sxbp1 ; ... ; Sxbpn ST

The soft characters are downloaded using the above device control string, where

DCS	=	Device control string introducer ESC P (hex 1B 50)

Pfn	=	Font buffer number:     0 or 1

Pcn	=	Starting character, 0 to 95 (ASCII-code - 20h)

Pe	=	Erase control:

0  =  Erase all characters in the character set before downloading

1  =  Erase only characters in locations being reloaded

Pcms	=	Character matrix size:

0  =  device default  (8 x 16)

1  =  illegal

2  =  5 x 10 pixel cell  (VT220)

3  =  6 x 10 pixel cell  (VT220)

4  =  7 x 10 pixel cell  (VT220)

Pw	=	Font width:

0 = device default  (80 columns)

Pt	=	Text or full cell

0 = device default  (text)

1 = text  - characters are auto-centered to the cell

2 = full cell  - all pixels addressable

Dscs	=	Character set name, in the format

I F

where

I	= 0, 1 or 2 intermediate characters in range 20 to 2F hex,

F	= the final character in range 30 to 7E hex.

The character set name is used when designating the character set with SCS command.

Sxbp1 ; ... ; Sxbpn

=	The sixtel bit patterns for individual characters, separated by semi-colons (hex 3B). 1 to 96 patterns can be sent in one DECDLD string. Each sixtel bit pattern is in the following format:

S...S/S...S/S...S

Where the first S...S represents the sixtels in Group A of the character (eight bytes typ.),  / (hex 25) advances the sixtel pattern to Group B, etc.

Each sixtel is coded as ASCII character, whose code is the bit pattern of the vertical row of six pixels + 3Fh. The uppermost bit is the least significant bit.

ST	= The string terminator (hex  9C). You can also use ESC \ (hex 1B 5C).

S C R E E N   C O N T R O L   C O M M A N D S

__________________________________________________________________________________

34. SET SCROLLING REGION							SCAR

ESC [ Pn1 ; Pn2  r 		1B 5B Pn1 3B Pn2 72

This sequence sets the top and bottom margins for the scrolling region. The parameter Pn1 refers to the top line and Pn2  to the bottom line of the scrolling region. The numbers are line numbers, ranging from 1 to 24. The minimum permitted size of the region is two lines, in which  case Pn1 is the uppermost line of the region, and Pn2  is Pn1 + 1.

After the scrolling region has been set, the scrolling of screen is limited within these margins. Cursor can not be moved outside these margins by means of relative cursor movement commands.

Upon setting the scrolling region, the cursor will move to the home position, depending on the setting of Origin Mode. If Origin Mode is set, the cursor will move to the upper left corner of the scrolling region. If origin mode is reset, the cursor moves to line 1, column 1, regardless of where the scroll area is.

__________________________________________________________________________________

35. SET LEFT AND RIGHT MARGINS						-

ESC [ Pn1 ; Pn2  v 		1B 5B Pn1 3B Pn2 76

This sequence sets the left and right margins. It is not possible to move the cursor outside these margins by means of CURSOR MOVEMENT commands, including TAB, LINE FEED, RETURN. The minimum size of the vertical region between the margins is 2 columns. The default margin columns are 1 to 80, i.e. the complete  screen.

Note:  This is an extended command not found on standard VT100 terminal.

__________________________________________________________________________________

36. ORIGIN MODE								OM

To set:			ESC [ ? 6 h   		1B 5B 3F 36 68

To reset:		ESC [ ? 6 l  		1B 5B 3F 36 6C

When set, the origin (Home position) is at the upper left character position within the scrolling region.  When reset, the origin is at the upper left corner of the screen.

Once margins have been defined and the origin mode set,  the line and column numbers are relative to those margin settings. The cursor is not allowed to be positioned outside those margins.

The cursor is moved to the new Home position when this sequence is received. Lines and columns are numbered consequentially, with the origin (Home) being line 1, column 1 upper left position.

__________________________________________________________________________________

S E T   M O D E

__________________________________________________________________________________

37. WRAP AROUND								WRP

To enable:		ESC [ ? 7 h   		1B 5B 3F 37 68

To disable:		ESC [ ? 7 l  		1B 5B 3F 37 6C

When enabled, any displayable character received when the cursor is at the right margin causes the cursor to advance to the beginning of the next line, performing a scroll up if required.

When disabled, any displayable character received when the cursor is at the right margin overwrites any previous character at this position. The cursor will not move to the next line until an explicit carriage return / line feed is received.

__________________________________________________________________________________

38. SEND / RECEIVE MODE							SRM

To set:			ESC [ 1 2 h   		1B 5B 31 32 68

To reset:		ESC [ 1 2 l   		1B 5B 31 32 6C

When set, the characters sent to the host are not echoed to the screen locally. The host must echo the characters back to the terminal, if they are to appear on the screen (Full duplex).

When reset, the characters sent to the host are also echoed locally on the screen (Half duplex). This command has exactly the same effect as selecting Full / Half Duplex on the Setup screen.

__________________________________________________________________________________

39. NEW LINE - LINE FEED MODE						LNM

To set:			ESC [ 2 0 h   		1B 5B 32 30 68

To reset:  		ESC [ 2 0 l   		1B 5B 32 30 6C

When set, the terminal is in new line mode. A line feed (LF, hex 0A) received from the host computer is interpreted as a carriage return and a line feed, positioning the cursor in the first column of the next line.  The RETURN key causes CR and LF characters to be sent to the serial line.

When reset, the terminal is in line feed mode. A line feed (LF, hex 0A) is interpreted as a line feed only, positioning the cursor at the same column on the next line.  Pressing the RETURN key causes a CR character to be sent only. The default setting for New Line / Line Feed mode can be selected from the SETUP field 'CR KEY'.

__________________________________________________________________________________

40. KEYPAD APPLICATION MODE				DECKPAM

ESC =  			1B 3D

Set numeric keypad to application mode. Control sequences are transmitted instead of numeric key codes. See chapter 4.4 for sequences sent.

__________________________________________________________________________________

__________________________________________________________________________________

41. KEYPAD NUMERIC MODE					DECKPAM

ESC > 				1B 3E

Set numeric keypad to numeric mode. ASCII-codes marked on key-caps for numeric keys are transmitted. __________________________________________________________________________________

42. CURSOR KEY MODE							CKM

To set:			ESC [ ? 1 h 		1B 5B 3F 31 68

To reset:		ESC [ ? 1 l 		1B 5B 3F 31 6C

This sequence determines the escape sequences transmitted by the cursor arrow keys. When reset, the normal cursor control sequences are sent. When set, special sequences are sent. See the table below for the sequences sent:

Cursor key		When set	When reset

Up Arrow		ESC O A	  ESC [ A

Down Arrow		ESC O B	  ESC [ B

Right Arrow		ESC O C	  ESC [ C

Left Arrow		ESC O D	  ESC [ D

On the AP988300, these sequences may be changed by patching the cursor key conversion table on the EPROM. See chapter 7 and Appendix A.

__________________________________________________________________________________

43. KEYBOARD LOCK MODE							KLM

To set:			ESC [ 2 h 		1B 5B 32 68

To reset:	  	ESC [ 2 l 		1B 5B 32 6C

When set, the keyboard is locked. Wait indicator on the VT220 keyboard is lit. No key codes are transmitted to the host. The only key combination that is effective is the SETUP key. Since the keyboard is locked, the keyboard unlock command has to come from the host. However, entering the SETUP mode also causes the keyboard to be unlocked.

__________________________________________________________________________________

44. SCREEN MODE				     DECSCNM

To set:	CSI ? 5 h 	1B 5B 3F 35 68

To reset:	CSI ? 5 l 	1B 5B 3F 35 6C

When set, the whole screen is displayed in inverse video.

When reset, the screen is displayed in normal video.

__________________________________________________________________________________

__________________________________________________________________________________

45. AUTO REPEAT MODE			     DECARM

To set:	CSI ? 8 h 	1B 5B 3F 38 68

To reset:	CSI ? 8 l 	1B 5B 3F 38 6C

When set, the key repeat on the VT220 or IBM-PC keyboard is ON.

When reset, the key repeat is OFF.

After power-on, the auto repeat mode is set  (unless changed by patching the EPROM).

__________________________________________________________________________________

46. SPACE COMPRESSION MODE						-

To set:			ESC [ ? 1 3 h 	1B 5B 3F 31 33 68

To reset:	  	ESC [ ? 1 3 l 		1B 5B 3F 31 33 6C

When set, only displayable characters are sent to the host, when REMOTE SEND COMMAND is received.  When reset, also space characters are sent to the host.

__________________________________________________________________________________

47. ANSI / VT52 MODE					DECANM

ESC [ ? 2 l 			1B 5B 3F 32 6C

When this command is received, the terminal exits the ANSI (VT100) mode and enters the VT52 mode. In the VT52 mode the terminal executes special VT52 command set, listed in the chapter 4.2. Also, some of the function keys and special keys send VT52 strings instead of VT100 strings (see chapter 4.4).

__________________________________________________________________________________

48. NATIONAL REPLACEMENT CHARACTER SET MODE	 		DECNRCM

To set : 	CSI ? 4 2 h   		1B 5B 3F 34 32 68

To reset: 	CSI ? 4 2 l  		1B 5B 3F 34 32 6C

When the national mode is set, the terminal sends 7-bit characters from the national replacement character set currently active (selected by LANGUAGE field on the SETUP screen) and uses national replacement character set to display the 7-bit characters on the screen. National mode restricts the characters terminal can send to those available in the selected NRC set.

When reset, the terminal is in multinational mode, and uses 8-bit characters from USASCII set and DEC supplemental character set.

Setting this mode resets all the character sets to their default (power-on) states.

The default state after power-on is defined by the LANGUAGE field on the setup screen. If USASCII is selected, the national mode is reset. If any other language is selected, the national mode is set.

Note: This is VT220 command not found on VT100 terminal. Unlike VT220, AP988300 can receive and display 8-bit characters even if the national mode is selected.

__________________________________________________________________________________

R E S E T   A N D   T E S T   C O M M A N D S

__________________________________________________________________________________

49. RESET TERMINAL							RIS

ESC c  			1B 63

This command resets the terminal, clearing the screen, sets the character sets to default, erases the attributes to default and puts cursor to Home position.

Note: since all the buffers are cleared, the characters received immediately after this command may be lost. Therefore the host should wait for at least 0.5 seconds before sending any further characters.

__________________________________________________________________________________

50. SCREEN ALIGNMENT DISPLAY					DECALN

ESC # 8  			1B 23 38

This sequence fills the entire display screen with upper case E's for screen testing.

__________________________________________________________________________________

R E P O R T   C O M M A N D S

__________________________________________________________________________________

51.  CURSOR POSITION REPORT							DSR

ESC [ 6 n   			1B 5B 36 6E

When this sequence is received, the cursor position report (CPR) is sent to the Host in the format

ESC [ Pl ; Pc  R   		1B 5B Pl 3B Pc 52

Where the current line and column numbers are represented by Pl  and Pc, respectively.

__________________________________________________________________________________

52. DEVICE STATUS REPORT							DSR

ESC [ Ps  n   			1B 5B Ps 6E

Both the device status request from the host and the device status report from the AP988300 use the  same format. The parameter Ps indicates the status:

Ps		Result

0		Response from the terminal: all is ready, no errors detected.

3		Response from the terminal: error detected.

5		Request from the Host.

__________________________________________________________________________________

53. DEVICE ATTRIBUTES REPORT						DA

ESC [ Pn  c			1B 5B Pn 63

By sending the sequence above, the host request the device attributes report from the terminal.   For report request, the parameter Pn must either zero or omitted (no parameter). The AP988300 responds by sending the sequence below, identifying itself as VT100 with printer installed:

ESC [ l ; 2 c  			1B 5B 31 3B 32 63

__________________________________________________________________________________

__________________________________________________________________________________

54. SECONDARY DEVICE ATTRIBUTES REPORT					DA

ESC [ > Pn  c		1B 5B 3E Pn 63

With this sequence, the host requests the terminal's  identification code, firmware version and hardware options. Parameter Pn must be zero, or it may be omitted. The terminal responds with the following sequence:

ESC [ > Pp ; Pv ; Po c  		1B 5B 3E Pp 3B Pv 3B Po 63

Where:

Pp	= Identification for the terminal. For AP988300 V4.*, the id code is 14.

Pv	= Firmware version  (the two digits to the right of decimal point)

Po	= Hardware options

Example:

ESC [ > 1 4 ; 3 0 ; 0 c	= AP988300, V4.30, no options

Note:  this is VT220 command not found on standard VT100 terminal.

__________________________________________________________________________________

55. PRINTER STATUS REPORT						DSR

ESC [ ? 1 5 n			1B 5B 3F 31 35 6E

With  this sequence, the host asks for the status of the printer connected to the terminal's printer port. The status is checked by testing the DTR or CTS handshake line on the printer port (depends on the settings of the on-board jumpers). The terminal responds with one of the following reports:

Sequence	Meaning

ESC [ ? 1 0 n	Printer ready.

ESC [ ? 1 1 n	Printer not ready.

ESC [ ? 1 3 n	No printer. Printer has not been ready since the last power-up or reset.

__________________________________________________________________________________

56. LOCATOR DEVICE STATUS REPORT						DSR

ESC [ ? 5 5 n			1B 5B 3F 35 35 6E

With this sequence, the host asks for the status of the locator device. The terminal responds with one of the following reports:

Sequence		Meaning

ESC [ ? 5 3 n		No locator device or locator device disabled  (J10 pos. 5)

ESC [ ? 5 0 n		Locator device ready

__________________________________________________________________________________

__________________________________________________________________________________

57. LOCATOR DEVICE ID						DSR

ESC [ ? 5 6 n			1B 5B 3F 35 36 6E

With this sequence, the host asks what kind of locator device is connected to the locator device port. The terminal responds with one of the following reports:

Sequence		Meaning

ESC [ ? 5 7 ; 0 n	Can not identify the locator device

ESC [ ? 5 7 ; 1 n	Locator device is Digital's mouse

ESC [ ? 5 7 ; 2 n	Locator device is Digital's tablet

ESC [ ? 5 7 ; 4 n	Locator device is Alpha Point optical Touch Screen Overlay

ESC [ ? 5 7 ; 8 n	Locator device is Membrane Switch Touch Screen Overlay

ESC [ ? 5 7 ; 14 n	Locator device is external optical Touch Screen Overlay

This report does not indicate the presence of locator device. Before using the locator device, the application should check the locator device status.

__________________________________________________________________________________

58. RESET / BATTERY STATUS REPORT						-

ESC [ > Ps  r		1B 5B 3E Pn 72

With this sequence, the host computer requests the reset / battery status report. Parameter Ps selects, if the status flags should be cleared after sending the report:

Ps 		result

0		Status flags are cleared after sending report

1		Status flags are left unchanged.

The terminal responds by sending the following report sequence:

ESC [ <Pres ; Pbat  r		1B 5B 3C Pres 3B Pbat 72

Where Pres indicates the cause of last terminal reset, as follows:

Pres		meaning

0		No reset occured since the flag was cleared

1		Reset command executed

2		Power-down reset *

5		Watchdog bite

7		Stack error

8		Undefined instruction trap

Pbat indicates the battery status:

Pbat		meaning

0		Battery OK

1		Battery OK, but battery disable jumper (J10, pos. 7) set

2		Battery dead or not installed

*  Power-down detect requires voltage sensing circuitry (R1, R2) to be installed.

Note:  This is AP988300 enhanced command not found on standard VT100 terminal.

__________________________________________________________________________________

__________________________________________________________________________________

59. REMOTE SEND COMMAND							-

ESC 5				1B 35

After receiving this command, the terminal sends the contents of the screen to the host computer. The screen contents are sent as a continuous string of ASCII characters. No CR:s or LF:s are sent between lines.  If  SPACE COMPRESSION MODE (see page 4-20) is set, the spaces are not sent.

__________________________________________________________________________________

P R I N T E R   C O M M A N D S

The following commands are used to control the interaction between the terminal and the printer.  See also: Printer Status Report on page 4-24.

__________________________________________________________________________________

60. PRINT LINE								MC

ESC [ ? 1 i   			1B 5B 3F 31 69

Media Copy: Send the line containing the cursor  to the printer (text only).

__________________________________________________________________________________

61. PRINT PAGE								MC

ESC [ i   			1B 5B 69

Media Copy: Send the contents of the screen to the printer (text only). If Print Extent mode is set, the whole screen is printed, if reset, only the scrolling region is printed.

__________________________________________________________________________________

62. PRINT PAGE GRAPHICS							 -

ESC [ ? i   			1B 5B 3F 69

Send the contents of the screen to the printer in graphics format. This allows all the graphic characters, attributes etc. to be displayed on the paper copy. An Epson FX-80 compatible printer is required.

See also the Hard Copy function explained in the chapter 5 (graphics command No. 26.)

Note:  This is not a standard VT100 command.

__________________________________________________________________________________

63. ENTER AUTO-PRINT MODE							MC

ESC [ ? 5 i  			1B 5B 3F 35 69

Media Copy: enter auto print mode. In the auto-print mode, the contents of cursor line is sent to the printer before a Line Feed is performed by a LF control character or line wrap operation.

__________________________________________________________________________________

64. EXIT AUTO-PRINT MODE							MC

ESC [ ? 4 i  			1B 5B 3F 34 69

This command terminates the Auto-print Mode.

__________________________________________________________________________________

__________________________________________________________________________________

65. ENTER PRINTER CONTROLLER MODE						MC

ESC [ 5 i   			1B 5B 35 69

Media Copy: enter printer controller mode. In printer controller mode the terminal will send all received characters to the printer without displaying them. None of the data is interpreted as a terminal command except the Exit Printer Controller Mode command.

It is recommended to check the printer status using Printer Status Report (see page 4-24) before sending large blocks of data to printer, otherwise the system may halt for long period while waiting for printer.

__________________________________________________________________________________

66. EXIT PRINTER CONTROLLER MODE						MC

ESC [ 4 i     			1B 5B 34 69

This command is detected by the terminal when in the Printer Controller Mode and causes the   controller to display all subsequent data instead of printing it. No character of this command is passed to the printer.

__________________________________________________________________________________

67. PRINT FROM FEED MODE						DECPFF

To enable:		ESC [ 1 8 h   		1B 5B 31 38 68

To disable:		ESC [ 1 8 l   		1B 5B 31 38 6C

When enabled, the terminal adds a Form Feed character (hex 0C) to the end of the data sent  to the printer after each PRINT PAGE command.  When disabled, no form feed is added.

__________________________________________________________________________________

68. PRINT EXTENT							DECPEX

To set:			ESC [ 1 9 h  		1B 5B 31 39 68

To reset:		ESC [ 1 9 l  		1B 5B 31 39 6C

When this function is set, any subsequent PRINT PAGE command will print the entire screen contents.

When reset, the PRINT PAGE command prints the lines within the scrolling region specified by the SCAR command (page 4-15).

__________________________________________________________________________________

S E T   L I N E   P A R A M E T E R S

__________________________________________________________________________________

69. SET LINE PARAMETERS							-

ESC [ baud ; parity ; databits ; stopbits; handshake ; duplex e

This command allows host computer to set the serial line communications parameters of the AP988300.  All the 6 parameters must be given. The parameters are:

baud: 			  stopbits:

0  =	no change 		0  =	no change

1  =		19200 bps               	1  =	(invalid)

2  = 		9600 bps          	2  =	1 bit

3  =		4800 bps            	3  =	(invalid)

4  =		2400 bps           	4  =	2 bits

5  =		1200 bps

6  =		600 bps

7  =		300 bps                 	  handshake:

19 =		19200 bps		0  =	no change

38 =		38400 bps  		1  =	XON/XOFF

76 =		76800 bps *)  	2  =	Hardware (CTS)

3  =	Both

parity: 			4  =	none

0  =	no change

1  =	no parity

2  =	odd parity                       	  duplex:

3  =	even parity  		0  =	no change

1  =	full duplex

databits: 	     		2  =	half duplex

0  =	no change

1  =	8 bits

2  =	7 bits

*)	76800 bps baud rate is available on the High-Speed version of AP988300 only.

Example:

ESC [ 5 ; 0 ; 0 ; 0 ; 1 ; 0 e

Selects 1200 bps, xon-xoff protocol without changing other parameters.

Warning:  After changing the line parameters, the line parameters of the host computer must be changed accordingly. Otherwise it may be impossible to communicate between the host and the AP988300.

Note:  This is Alpha Point enhanced command not found on standard VT-100 terminal.

__________________________________________________________________________________

T O U C H   S C R E E N   C O M M A N D S

__________________________________________________________________________________

70.  SET TSO MODE   					-

To set:			CSI > Ps h   		1B 5B 3E Ps 68

To reset:		CSI > Ps l   		1B 5B 3E Ps 6C

This sequence selects the TSO operating mode. Parameter list Ps may contain one or more of the following parameters:

Ps	Mode, when set

14	Report only on query

15	TSO enabled

16	Debug mode  (send also incomplete coordinate pairs)

17	Exit report mode  (send ?? ?? message, when the finger is removed)

18	Sound click on report

20	Double resolution mode  (using interpolation)

21	NET mode  (insert net address in the message)

22	Single character mode  (get character code from tsotable in EPROM)

Multiple modes may be set or reset by giving multiple parameters, separated by semi-colons. If the single character mode is set, the double resolution mode and net mode are ignored.

If external Touch Screen device (other than Alpha Point TSO) is used, only the following mode options are available:

14	Report only on query

18	Sound click on report

21	NET mode  (insert net address at end of message)

22	Convert control characters (add 30H to all characters below 80h)

After power-on or reset, all modes are reset, except TSO enable, which depends on TSO enable jumper on-board, and click mode, which is set by default.

TSO modes can also be set by using Graphics Mode command SET TSO (see chapter 5). Default TSO mode can be set by patching the EPROM location TSOMODE.

See also:  Chapter 1.3  (The Touch Screen Overlay option).

__________________________________________________________________________________

__________________________________________________________________________________

71.  TSO TIMING						-

CSI > Pn1 ; Pn2 ; Pn3 T   		1B 5B 3E Pn1 3B Pn2 3B Pn3 54

Set TSO timing, according to the parameters:

Param	Explanation		Allowed values	 Reset value

Pn1		Repeat delay,	1 to 255.  			9

Pn2		Repeat rate, 	1 to 255,  0 = no repeat.	 	3

Pn3		Tracking speed	1 to 255,  0 = no tracking mode		1

Parameters Pn2 and Pn3 are optional. If parameter Pn3, or both Pn2 and Pn3 are omitted, these values are not changed. The table above gives parameters for the optical TSO. For membrane TSO, Pn3 sets the scan rate.

The reset values may be changed by patching the EPROM locations TSODEL, TSORAT and TSOSPD.  See chapter 7.2 for more information.

This command has no effect on external touch screen device connected to keyboard interface. For controlling external TSO, see command 72.

__________________________________________________________________________________

72.  SEND COMMAND TO EXTERNAL TSO				-

DCS Pn t  c1 c2 ... cn ST  		1B 50 Pn 74 c1 c2 ... cn 1B 5C

This device control string is used to send commands to external locator device connected to the keyboard interface of the terminal. All characters after the final character (t) are sent directly to the external device, until the String Terminator (ST) is reached.

DCS	=	Device control string introducer:  ESC P  (hex 1B 50).

Pn		=	Selects the I/O port for the locator device.

0  = 	Device default  (VT220 keyboard port)

1  =	VT220 Keyboard port

2  =	IBM-PC keyboard port

t		=	The final character. All characters after this are sent to the external device.

c1 c2 ... cn

=	Characters sent to external device. May be any value except ST.

ST		= The string terminator (hex  9C). You can also use ESC \ (hex 1B 5C).

See your locator device manual for interpreting the command characters c1 ... cn.

Programming tip:  When this command is entered from another terminal, you may give any number of commands to the locator device from your keyboard and examine the results immediately on screen. Enter Esc \  to end the host-to-device session.

Note:  This is not a standard VT100 command. Use of this command is dependent on the system configuration.

__________________________________________________________________________________

M U L T I P O I N T   C O M M U N I C A T I O N S

__________________________________________________________________________________

73.  SELECT NET TARGET  					-

SOH a a ... a STX   		01 a a ... a 02

This sequence is used to select receiver(s) of any following messages on the multi-point communication systems (e.g. terminal network using the optional RS485 interface).

After this command, only the terminals listed in the string will receive the characters transmitted through the serial line, until next SOH ... STX header is received. The net address for each terminal is selected from the NET ADDR field on the SETUP screen.

Any number of terminals may be selected by entering any number of address bytes (a) in the header string. Each address is a single byte coded as net address + 30H. Addresses may vary in the range 1 to 79, thus, each address byte in the string may be in range 31H to 7EH (ASCII character '1' to '~'). A special address zero (30h, ASCII '0') is used as broadcast address. After that, all the terminals will receive the messages (the default state).

If the first character after SOH is '+', the following addresses are appended to the list of active terminals. Otherwise, the list replaces the old list.

Examples:

SOH 2 STX			- Selects terminal No. 2

SOH 1 5 A STX		- Selects terminals 1, 5 and 17

SOH + 7 8 STX		- Adds terminals 7 and 8 to list of the receivers.

__________________________________________________________________________________

74.  ENABLE TRANSMISSION					-

DC2 a  			12 a

This sequence enables transmission for given terminal on the network (selective XON). Only the selected terminal is able to send anything on the serial line.

Parameter a, which selects the transmitting terminal, is a single byte coded as net address + 30h, and may vary in range 30 to 7F hex, '0' to '~' ASCII. Address '0' selects all the terminals. Any value below 30h disables all the terminals from sending. A space (20H) is recommended.

If 'Report Only On Query' mode is selected using the TSO MODE command (parameter 14), the terminal immediately sends the last TSO coordinate report. No further coordinate reports are sent, until the next DC2 sequence is received. This may be used to poll the TSO messages from each terminal.

__________________________________________________________________________________

4.2.  VT52 COMMANDS

These commands are available in the Terminal Mode and Mixed Mode, when VT52 emulation has been selected in the EMULATION field in SETUP, or if DECANM command has been received in the VT100 mode.

The control characters recognized in the VT52 mode are the same as in the VT100 mode. The escape sequences recognized are listed below.

__________________________________________________________________________________

1. CURSOR ADDRESSING							CUA

ESC Y row col    		1B 59 row col

Moves the cursor to the location specified by row and col, where row and col must be two ASCII characters representing the line and column numbers respectively. The character code for row and col is calculated by adding 1Fh to the line or column number. Row or column 1 is thus represented by space (20h), etc.

__________________________________________________________________________________

2. CURSOR BACKWARD							CUB

ESC D				1B 44

Moves cursor one column to the left.

__________________________________________________________________________________

3. CURSOR DOWN								CUD

ESC B				1B 42

Moves cursor down one line.

__________________________________________________________________________________

4. CURSOR FORWARD								CUF

ESC C				1B 43

Moves cursor one column to the right.

__________________________________________________________________________________

5. CURSOR HOME								CUH

ESC H				1B 48

Moves cursor to home position (upper left corner of the screen).

__________________________________________________________________________________

__________________________________________________________________________________

6. CURSOR UP									CUU

ESC A				1B 41

Moves cursor up one line.

__________________________________________________________________________________

7. REVERSE LINE FEED							RVLF

ESC I				1B 49

Moves cursor up one line. If at the top of the screen, scroll down is performed.

__________________________________________________________________________________

8. CLEAR SCREEN								CRS

ESC J				1B 4A

Erases from cursor to end of screen.

__________________________________________________________________________________

9. ERASE LINE									EL

ESC K				1B 4B

Erases from cursor to end of line.

__________________________________________________________________________________

10. SPECIAL GRAPHICS							GRF

ESC F				1B 46

Selects special graphics character set. All subsequently received data will be mapped to the special graphics characters.

__________________________________________________________________________________

11. EXIT SPECIAL GRAPHICS							ASCII

ESC G				1B 47

Returns to ASCII character set, if special graphics set was selected using the above command.

__________________________________________________________________________________

12. AUTO PRINT MODE								APM

To set:		ESC ^			1B 5E

To reset:		ESC _			1B 5F

When set, the current line is printed (transmitted out the printer port) before each line feed operation.

__________________________________________________________________________________

__________________________________________________________________________________

13. CHANGE MODE								CM

ESC <				1B 3C

Exits the VT52 mode and returns terminal to ANSI (VT100) mode.

__________________________________________________________________________________

14. KEYBOARD APPLICATION MODE						KAM

ESC =				1B 3D

Sets the numeric keypad to application mode. In application mode the keypad sends special escape sequences instead of the characters marked on the key-caps. These escape sequences are different from those in VT100 mode. See chapter 4.4.

__________________________________________________________________________________

15. RESET KEYBOARD APPLICATION MODE					RKAM

ESC >				1B 3E

Returns numeric keypad to normal mode, where each key sends the values marked on the key-caps.

__________________________________________________________________________________

16. TERMINAL IDENTIFICATION						TID

ESC Z				1B 5A

The host request terminal to identify itself. In VT52 mode the terminal responds with the sequence:

ESC / Z			1B 2F 5A

__________________________________________________________________________________

4.3  ADM3 COMMANDS

These commands are available in the terminal mode and mixed mode, when the ADM3 option has been selected on the EMULATION field of the SETUP screen.

The ADM3 emulation of the AP988300 is based on the Lear Siegler ADM3 series of terminals (ADM3, ADM3A, ADM5 etc.). This emulation mode may also be used with software developed for other terminals based on ADM3 (ADDS, Ampex, Hazeltine, Qume, Tele Video etc.). However, this is not an exact emulation of any of the above terminals.

__________________________________________________________________________________

1. CURSOR UP

Ctrl-K					0B

Moves cursor down one line in the same column. If cursor is on line 24, it is not moved.

__________________________________________________________________________________

2. CURSOR DOWN  (LINE FEED)

Ctrl-J					1A

Moves cursor down one line in the same column. If cursor is on line 24, the screen is scrolled up.

__________________________________________________________________________________

3. CURSOR LEFT

Ctrl-H  or  BS				08

Moves cursor one column to the left.

__________________________________________________________________________________

4. CURSOR RIGHT

Ctrl-L					0C

Moves cursor one column to the right. If cursor is in the last column of line, it is moved to the first column of next line.

__________________________________________________________________________________

5. CARRIAGE RETURN

Ctrl-M  or  ENTER			0D

Moves cursor to the first column of next line.

__________________________________________________________________________________

__________________________________________________________________________________

6. CURSOR HOME

Ctrl-^					1E

Moves cursor to home position (upper left corner of the screen).

__________________________________________________________________________________

7. CURSOR ADDRESSING

ESC = row col    			1B 3D row col

Moves the cursor to the location specified by row and col, where row and col must be two ASCII characters representing the line and column numbers, respectively. The character code for row and col is calculated by adding 1Fh to the line or column number. Row or column 1 is thus represented by space (20h), row or column 2 is "!" (21h), etc.

__________________________________________________________________________________

8. CURSOR ADDRESSING, ROW

ESC [ row     				1B 5B row

Moves the cursor to new row in the same column. Parameter row is a single ASCII character that specifies the destination row.  The character code for row is calculated by adding 1Fh to the row number. Row 1 is thus represented by space (20h), and row 24 is ASCII character "7"  (37h).

__________________________________________________________________________________

9. CURSOR ADDRESSING, COLUMN

ESC ] col     				1B 5D col

Moves the cursor to new column on the same row. Parameter col is a single ASCII character that specifies the destination column.  The character code for col is calculated by adding 1Fh to the column number. Column 1 is thus represented by space (20h), and column 80 is ASCII character "o"  (6Fh).

__________________________________________________________________________________

10. TAB

Ctrl-I  or  TAB				09

Moves cursor to the next tab stop. If there are no more TAB stops on the line, the cursor remains at the right edge of the screen. By default, TAB stops are set to columns 1, 9, 17, 25 etc.

__________________________________________________________________________________

__________________________________________________________________________________

11. BACK TAB

ESC I					1B 49

Moves cursor to the next tab stop. If there are no more TAB stops on the line, the cursor is moved to column 1.

__________________________________________________________________________________

12. CLEAR SCREEN

ESC +					1B 2B

or	Ctrl-Z					1A

Clears the whole screen to spaces. Does not move the cursor.

__________________________________________________________________________________

13. ERASE END OF PAGE

ESC Y					1B 59

Erases all characters from cursor position to the end of page.

__________________________________________________________________________________

14. ERASE END OF LINE

ESC T					1B 54

Erases all characters from cursor to end of line.

__________________________________________________________________________________

15. SET VIDEO ATTRIBUTES

ESC G attr				1B 47 attr

Inserts a video attribute control character at the current cursor location. The cursor is moved to next character position. All the characters from the cursor to next attribute control character or to the end of line are immediately changed to the attribute selected. Any new characters entered will also appear with the attribute specified by the attribute control character.

The attribute control character appears on the screen as a space with specified attribute. If the attribute control character is deleted or overwritten, all the characters from cursor to the next attribute control character are changed to the attribute specified by previous attribute control character on the same line.

The parameter attr specifies the attributes as follows:

attr	effect

ascii 	hex

0		30	normal

2		32	bold

4		34	reverse

6		36	bold reverse

8		38	underline

:		3A	bold underline

<		3C	reverse underline

>		3E	bold reverse underline

__________________________________________________________________________________

16. GRAPHICS CHARACTER SET

ESC $					1B 24

Selects the graphics character set (line drawing symbols). The graphics characters are listed in the Appendix B: "ADM3 Graphics Character Set".

__________________________________________________________________________________

17. ASCII CHARACTER SET

ESC %					1B 25

Selects the normal ASCII character set.

__________________________________________________________________________________

18. ENTER GRAPHICS MODE  (Alphasoft)

ESC g					1B 67

Enters the High Resolution Graphics mode. If the current mode is Terminal Mode, the screen is cleared. If the current mode is Mixed Mode, the screen is not cleared.

In the graphics mode, a special Alphasoft command set is recognized, see chapter 5. You may re-enter ADM3 terminal mode with command ESC F or mixed mode with ESC X. For more information about different operating modes, see chapter 1.2.

__________________________________________________________________________________

19. ENTER ANSI (VT100) MODE

ESC U					1B 55

Enters the VT100 emulation mode, just as if the VT100 option were selected from the SETUP screen. The screen is cleared. You may re-enter the ADM3 mode with command:

ESC u					1B 75

Note:  these are not standard ADM3 or VT100 commands.

__________________________________________________________________________________

__________________________________________________________________________________

20. LOCK KEYBOARD

ESC #					1B 23

After receiving this command, no data received from the keyboard is sent to the host nor displayed on the screen. The keyboard can be unlocked by a command from host computer or by entering the SETUP mode.

__________________________________________________________________________________

21. UNLOCK KEYBOARD

ESC "					1B 22

After receiving this command, the keyboard operates normally.

__________________________________________________________________________________

22. PRINT SCREEN

ESC P					1B 50

Sends the contents of the screen from HOME through cursor position to the printer. After all the data has been printed, the terminal sends an ACK control character to the host computer.

__________________________________________________________________________________

23. TRANSPARENT PRINT MODE

ESC `					1B 60

Sends all data received from host, including escape sequences and control codes, directly to the printer. No data is displayed on screen. The transparent printing mode is exit with command:

ESC a					1B 61

__________________________________________________________________________________

24. COPY PRINT MODE

ESC @					1B 40

The data received from the host is both sent to the printer and displayed on the screen. The copy print mode is exit with command:

ESC A					1B 41

__________________________________________________________________________________

### 4.4.  TRANSMITTED CODES

This chapter describes the codes transmitted to host computer, when a key is pressed on keyboard. VT220 type keyboard is assumed. On IBM-PC-keyboard, some keys may have different markings on keycaps and some keys may not be available. Most keys, however, work equally on both keyboards. (See end of this chapter for main differences.)

Main keypad Function Keys:

Key:			Code transmitted:

Õ			DEL character (hex 7F)

Tab			HT character  (hex 09)

¿  Return	CR character only or CR and LF, depending on the state

of line feed / new line mode (LNM).

Ctrl			Does not send code. Used in conjunction with other keys

to generate control characters.

Lock			Does not send code. Used to toggle caps lock on / off.

ò Shift		Does not send code. Used in conjunction with other keys

to generate capital letters or special symbols.

Space bar		SP character (hex 20).

Compose		Does not send a code. Used in special compose character

character		sequences (see chapter 3.1.7).

Editing Keys:	(VT100 mode only. These are VT220 keys not on standard VT100)

Key			Code transmitted

Find			CSI 1 ~

Insert Here	CSI 2 ~

Remove		CSI 3 ~

Select		CSI 4 ~

Prev Screen	CSI 5 ~

Next Screen	CSI 6 ~

Cursor Control Keys:

ANSI mode *)		VT52 mode	ADM3 mode

Cursor key mode:

Key			Normal	Application

ñ	(up)		CSI A	SS3 A		ESC A	VT	(Ctrl-K)

ò	(down)	CSI B	SS3 B		ESC B	LF	(Ctrl-J)

ð	(right)	CSI C	SS3 C		ESC C	FF	(Ctrl-L)

ï	(left)	CSI D	SS3 D		ESC D	BS	(Ctrl-H)

*)	ANSI mode applies to VT220 and VT100 modes.

VT52 and ADM3 modes are ANSI-incompatible modes.

Auxiliary Keypad Keys:

ANSI mode			 VT52 mode

Keypad mode:		Keypad mode:

Key			Numeric       Application		Numeric	Application

0			  0	SS3 p		0	ESC ? p

1			  1	SS3 q		1	ESC ? q

2			  2	SS3 r		2	ESC ? r

3			  3	SS3 s		3	ESC ? s

4			  4	SS3 t		4	ESC ? t

5			  5	SS3 u		5	ESC ? u

6			  6	SS3 v		6	ESC ? v

7			  7	SS3 w		7	ESC ? w

8			  8	SS3 x		8	ESC ? x

9			  9	SS3 y		9	ESC ? y

-			 -  (minus)	SS3 m		-	ESC ? m

,			 ,  (comma)	SS3 l.		,	ESC ? l

.			 .  (period)	SS3 n		.	ESC ? n

Enter		 CR *)	SS3 M		CR *) 	ESC ? M

or CR LF			or CR LF

PF1			 SS3 P	SS3 P		ESC P	ESC P

PF2			 SS3 Q	SS3 Q		ESC Q	ESC Q

PF3			 SS3 R	SS3 R		ESC R	ESC R

PF4			 SS3 S	SS3 S		ESC S	ESC S

*) In numeric keypad mode, ENTER generates the same code as RETURN. You can change the code generated by RETURN with the line feed / new line mode. When reset, only CR is transmitted. When set, CR and LF are transmitted.

Note:  SS3  (single shift 3)  is the escape sequence  ESC  O  (hex 1B 4F).

In ADM3 mode, the  keypad  acts as numeric keypad.

Top Row Function Keys:

Code generated:

ADM3, VT52 and

Legend          	Name		VT220 mode **	VT100 modes

Hold Screen	(F1)*		-	-

Print Screen	(F2)*		-	-

Set-Up		(F3)*		-	-

Data/Talk	(F4)		ESC	ESC

Break		(F5)*		(break signal)	(break signal)

F6			F6		CSI 1 7 ~	-

F7			F7		CSI 1 8 ~	-

F8			F8		CSI 1 9 ~	-

F9			F9		CSI 2 0 ~	-

F10			F10		CSI 2 1 ~	-

F11			F11		CSI 2 3 ~	ESC

F12			F12		CSI 2 4 ~	BS

F13			F13		CSI 2 5 ~	LF

F14			F14		CSI 2 6 ~	-

Help			(F15)		CSI 2 8 ~	-

Do			(F16)		CSI 2 9 ~	-

F17			F17		CSI 3 1 ~	-

F18			F18		CSI 3 2 ~	-

F19			F19		CSI 3 3 ~	-

F20			F20		CSI 3 4 ~	-

*   F1 through F5 are local function keys and do not generate codes., except the F4 key, which sends an ESC  (the Data/Talk function is currently not used)

**   VT220 mode is currently not supported

Alphanumeric Keys:

Each of the alphanumeric keys send the codes marked on the key-caps. The code for each key varies among the different national versions of the keyboard.

Many of the national keyboard versions have different symbols marked on the left and right side of the key-cap. Which symbol is sent, depends on the Typewriter / Data Processing mode selection. See chapter 3.2 (SETUP MODE).

The national layouts are defined for the VT220 keyboard. On IBM-PC keyboards, the key layout for national characters may not match with this definition.

IBM-PC keyboard specific keys

Some of the keys listed above are not available on IBM-PC type keyboard. However, IBM-PC keyboard contains some special keys not listed above, and some of the keys have different markings on keycaps.

Key			Function

Esc			ESC character (hex 1B)

PrtScr		Print Screen function (same as F2 on VT220 keyboard)

Scroll Lock	Freezes display (same as F1, Hold Screen on VT220 keyboard)

¬  Back space	DEL character (hex 7F)  (Same as Õ on VT220 keyboard)

¿   Enter		Same as Return on VT220 keyboard  (main keypad key)

\			Backslash (hex 5C)  *)

Alt  (left)		Acts as Compose Character key

Alt Gr 		When pressed together with an alphabetic key, transmits the symbol

engraved on the right side of the keycap (not on USA keyboard).  *)

Num Lock	Toggles the numeric keypad between mumeric mode and cursor key

mode. Num Lock light indicates the current state.

*)  These keys are available on IBM-AT keyboard only

  