5.  GRAPHICS COMMANDS

SUMMARY OF GRAPHICS COMMANDS:

No.     Command	Name	 Explanation	page
__________________________________________________________________________________________

1. 	I	INITIALIZE        	- initialize graphics mode   	5-3

2. 	C	CLEAR	- clear screen    	5-3

3. 	D	DOT	- draw, erase or complement a dot  	5-4

4. 	L	LINE TYPE	- select line type   	5-5

5. 	M	MOVE	- move graphics cursor or draw a line   	5-6

6. 	B	BOX	- draw a box   	5-7

7. 	R	RECTANGLE	- draw a rectangle filled with pattern   	5-8

8. 	A	ARC	- draw an arc or a circle   	5-10

9. 	BZ	BEZIER	- draw a Bézier curve 	5-12

10. 	P	PAINT	- paint or erase area within boundaries  	5-13

11. 	T	TEXT	- write alphanumeric text on screen   	5-14

12. 	K	TEXT	- write text, simplified syntax  	5-15

13.	Z	ZIGZAG	- draw polyline etc.	5-16

14.	X	XCOPY	- pixel block copy	5-18

15. 	V	VIEWPORT	- move viewport  within logical screen    	5-19

16. 	O	ORIGIN	- set origin point for graphic commands	5-19

17. 	SA	SET  ATTRIBUTES	- set  visual text attributes	5-21

18. 	SB	SET  BAUD	- set  serial line parameters	5-21

19. 	SC	SET  CLICK	- set  Bell sound length	5-22

20. 	SF	SET  FONT	- select  text font	5-22

21. 	SI	SET  INK	- select  color  (color displays only)	5-22

22. 	SL	SET  LANGUAGE	- select  character set	5-24

23. 	ST	SET  TSO	- set  Touch Screen Overlay parameters	5-25

24. 	SS	SET  SCREEN	- set  screen saver (blank screen)	5-Error: Reference source not found

25. 	SZ	SET  ZIGZAG	- set  default parameters for zigzag command	5-Error: Reference source not found

26. 	H	HARD COPY	- print a paper copy 	5-Error: Reference source not found

27. 	#	NET  TARGET	- select receiver(s) of commands	5-Error: Reference source not found

28.	:...;	DEFINE  MACRO	- define or delete macro	5-Error: Reference source not found

29. 	@	CALL  MACRO	- call macro	5-Error: Reference source not found

30.	?:	REPORT	- query report for macro buffer space	5-Error: Reference source not found

31.	?@	REPORT	- query  macro ID report	5-Error: Reference source not found

32. 	>	TIMER	- start timer  (delayed macro)	5-Error: Reference source not found

__________________________________________________________________________________________

High resolution graphics commands can be entered, when the graphics mode  has been  selected (command ESC G).  While in Graphics Mode, the cursor is not displayed.

Each graphics command contains one or two letters, followed by parameters (if any) and is terminated by a carriage return (CR, hex 0D). Parameters are decimal values (in ASCII), and are separated by commas. Some commands also use string parameters. Some of the parameters are optional. If an optional parameter is omitted (by entering fewer parameters), default value is used. If a parameter is skipped by entering just a comma, value 0 is assumed.

Commands are case-sensitive. Most commands are upper case letters. Lower case letters are typically commands using relative coordinates.

Commands containing syntax errors or illegal parameters are not executed. LF and SPACE characters within commands are ignored.  Leading zeros are ignored in all numeric parameters.

The command is executed after the CR character has been received. Before that, the command can be canceled by control character CAN (hex 18, key Ctrl-X) or SUB (hex 1A, key Ctrl-Z).

CONTROL CHARACTERS RECOGNIZED IN GRAPHIS MODE:

Hex	ASCII	Key		 Function
______________________________________________________________________________

01 	SOH	Ctrl-A       	- Start of header *

02 	STX	Ctrl-B		- Start of text *

07	BEL	Ctrl-G		- Sound bell

0D	CR	Return		- Command terminator

11	DC1	Ctrl-Q		- XON  (restart transmission)

12	DC2	Ctrl-R		- Network XON *

13	DC3	Ctrl-S		- XOFF  (halt transmission)

18	CAN	Ctrl-X		- Cancel command

1A	SUB	Ctrl-Z		- Cancel command

1B	ESC	ESC		- Start escape sequence

______________________________________________________________________________

* see Multi-point Communications on chapter 4.1

ESCAPE SEQUENCES:

Esc F	- Exit graphics mode and enter terminal mode

Esc X	- Enter mixed mode  (see chapter 1.2)

Esc [	- Start VT100 escape sequence (if applicable)

_____________________________________________________________________________________

1.   INITIALIZE 								I

I res <CR>

Initializes the graphics mode to default state. Clears the display memory, moves pen position to (0,0), sets origin and viewport to (0,0), sets line type to solid and line width to 1 pixel.

This command is performed automatically, when entering the graphics mode from terminal mode (but not when entering from mixed mode).

Optional parameter res can be used with the 640.480 display panels. It selects the display resolution for graphics mode. Accepted values are:

400	selects 640.400 display resolution

480	selects 640.480 display resolution

After entering the graphics mode, the display is set to 640.400 mode. This mode gives higher screen refresh rate and compatibility with software created for 640.400 displays.

640.480 mode allows the whole screen to be used for graphics.

If no parameter is given, the display resolution is not changed.

Examples:

I			Initializes the graphics mode, does not change display resolution

I480			Initializes the graphics mode and selects 640.480 display resolution.

_____________________________________________________________________________________

2.   CLEAR									C

C <CR>

Clears the whole display memory of 2048 by 2048 pixels (all images).

(To clear only part of display memory, use RECTANGLE command.)

_____________________________________________________________________________________

_____________________________________________________________________________________

3.   DOT 										D

D xx , yy , z <CR>

Draws a dot at the specified location (xx,yy), where 0,0 is the lower left corner of the logical display area (if not changed by Origin command). Pen position is moved where the dot was drawn.  Parameter z selects the drawing mode as follows:

z = 0	ignore  (move only)

z = 1	complement  dot  (XOR)

z = 2	erase dot   (AND)

z = 3	set dot   (OR)

The third parameter may be omitted, in which case the drawing mode defaults to 1 (complement).

Examples:

D33,444,3 		- Draws a dot to x=33, y=444

D1000,1200,2		- Erases the dot at 1000,1200

D0,6			- Complements (XOR) the dot at 0,6

D10,100,0		- Moves the graphic cursor to 10,100

_____________________________________________________________________________________

3b.   DOT RELATIVE								d

d xx , yy , z <CR>

Draws a dot at location relative to current pen position. Parameters xx and yy define the offset.  Negative values can be used for directions left and down. Pen position is moved to the position where the dot was drawn.

Optional parameter z defines the drawing mode, see the D command above.

Examples:

d20,30 			- Complements a dot at 20 pixels right, 30 pixels up from current pen

d-100,0,3		- Draws a dot at 100 pixels left from current pen position

_____________________________________________________________________________________

_____________________________________________________________________________________

4.   LINE TYPE									L

L t , w <CR>

Selects the type and width of line drawn by MOVE, ZIGZAG and BOX commands. The selected line type and width will remain active until next LINE TYPE command.

Parameter t  selects one of the eight line types available:

t = 0	solid line

t = 1	dashed line  (4:4)

t = 2	dotted line  (1:1)

t = 3	dot-dashed line

t = 4	long dashes  (7:1)

t = 5	medium dashes  (3:1)

t = 6	short dashes  (2:2)

t = 7	coarse dotted  (1:3)

Optional parameter w selects line width in range 1 to 255. If w is omitted, the line width is not changed.  Default line width after entering graphics mode is 1.

On line commands, the line width increases towards higher coordinates. On Box and Zigzag-Circle commands, the line width increases towards the center of the box or circle.

Note:  For 640.200 display, the width for horizontal lines is set to half of the given value, in order to retain the aspect ratio. That is, if you set line width to 4, the width of vertical lines will be 4, but the width of a horizontal line is 2.

Examples:

L3 		- Selects dot-dashed line type

L0,8		- Selects solid line type and line width of 8 pixels

_____________________________________________________________________________________

_____________________________________________________________________________________

5.   MOVE   (or DRAW A LINE)						M

M xx , yy , z <CR>

Moves the "pen" from current position to a point specified by xx,yy, where 0,0 is the lower left corner of the logical display area (if not changed by Origin command). Parameter z selects the drawing mode:

z = 0   move only

z = 1   complement   (XOR)

z = 2   erase   (AND)

z = 3   set dot   (OR)

When z=0, only the pen position is moved to the new position. When z>0, a line is drawn from old position to xx,yy.  If parameter z is not given, it defaults to 0.

Programming tip:  If you draw a picture using XOR-mode (z=1), and then draw the same picture again,  the picture is erased without affecting any old data on the screen.

See also:

Command 4   (LINE TYPE):	Selecting type and width of line.

Command 13  (ZIGZAG):		Draw polyline

Examples:

M133,611,3 		- Draws a line from old position to 133,611

M666,555,2 		- Erases a line to 666,555

M155,4,1  		- Draws a line to 155,4 by complementing each dot

M0,0   			- Moves the graphic cursor to the origin

_____________________________________________________________________________________

5b.   MOVE RELATIVE							m

m xx , yy , z <CR>

Moves the "pen" or draws a line using relative coordinates. The operation of this command is identical to the M command above, except that coordinates xx and yy are relative to current pen position rather than absolute coordinates. Negative values can be used for directions left and down.

Examples:

m50,20	 		- Moves pen position 50 pixels right, 20 pixels up.

m120,-85,3 		- Draws  a line 120 pixels right, 85 pixels down.

_____________________________________________________________________________________

_____________________________________________________________________________________

6.   BOX									B

B x1 , y1 , x2 , y2 , z <CR>

Draws a BOX with left lower corner defined by coordinates x1,y1 and right upper corner by x2,y2. The pen position is not moved.

Optional parameter z defines the drawing mode (1 to 3), see below. Default value for z is 1.

z = 1   complement   (XOR)

z = 2   erase  (AND)

z = 3   set  dot  (OR)

The box is drawn using the line type and width selected by L command. Line width grows towards the center of the box, so the outer corners of the box are always those defined in the command.

Examples:

B0,0,140,290,3 			- Draws a box with lower left corner at origin

B0,0,140,290,2 			- Erases the box drawn above

_____________________________________________________________________________________

6b.   BOX RELATIVE								b

b dx , dy , z <CR>

Draws a BOX using relative coordinates. One corner is the current pen position, other corner is defined by  parameters dx and dy, relative to the pen position.  Thus, parameter dx is the width and dy is the height of the box. Negative numbers may be used to draw box to left and down from current pen position. The pen position is not changed. Parameter z is the drawing mode, as above.

Examples:

b100,75			- Draws a box 100 pixels right, 75 pixels up, using XOR mode.

b-150,-120,3		- Draws a box 150 pixels left,  120 pixels down, using OR mode.

_____________________________________________________________________________________

_____________________________________________________________________________________

7.   RECTANGLE FILL								R

R x1 , y1 , x2 , y2 , p , z <CR>

Draws a RECTANGLE filled with a pattern. Coordinate pairs (x1,y1) and (x2,y2) define the two corners of the rectangle.

Optional parameter p defines the pattern to be used. Default value is 0 (solid pattern). There are 16 patterns available:

You may also use any ASCII character as pattern, by entering the ASCII code of the character (32 to 127) as the parameter p. Semigraphic characters may also be used, by values 128 to 160.

Optional parameter z  defines the drawing mode  (default = 0):

z = 0	Replace each pixel				z = 4	Reverse replace

z = 1	Complement pixels set on pattern  (XOR)    	z = 5	Reverse complement

z = 2	Erase pixels set on pattern  (-AND)		z = 6	Reverse erase

z = 3	Set  pixels (OR)				z = 7	Reverse set

Patterns can be mixed multiple ways to get hundreds of new patterns:  Draw second rectangle on top of the first one by varying patterns and/or shifting position by one or two pixels. Drawing mode 1 (XOR) usually gives best results.

Very fast filling is achieved, when using solid pattern ( p = 0 ). This feature can be used to quickly erase, fill or complement specified rectangular area anywhere in the screen memory.

Examples:

R50,10,100,60			- Draws a rectangle with solid pattern and replace mode

R80,30,120,60,3			- Draws a rectangle with pseudo-grey pattern

R10,10,100,100,65,1		- Draws a rectangle using letter 'A' as fill pattern

R0,400,639,799,0,2		- Erases the 'second' image area of the total screen memory.

_____________________________________________________________________________________

_____________________________________________________________________________________

7b.   RECTANGLE RELATIVE						r

r dx , dy ,  p , z <CR>

Draws a RECTANGLE using relative coordinates. One corner is the current pen position, other corner is defined by  parameters dx and dy, relative to the pen position.  Thus, parameter dx is the width and dy is the height of the rectangle. Negative numbers may be used to draw box to left and down from current pen position. The pen position is not moved.

Parameter p is the fill pattern, as on the R command above. Parameter z is the drawing mode, as above.

Examples:

r200,150			- Draws a rectangle with solid pattern, using replace mode.

r-250,100,14		- Draws a rectangle with "brick wall" pattern, replace mode

r-99,-37,0,2		- Erases a rectangular area down and left from current pen position

r400,200,15,1		- Draws a rectangle with pattern 15, using XOR mode.

_____________________________________________________________________________________

_____________________________________________________________________________________

8.   ARC,  CIRCLE								A

A x1 , y1 , x2 , y2 , cx , cy , d , z <CR>

Draws an ARC between specified coordinates. CIRCLEs may also be drawn, by giving the start point equal to the end point. Pen position is moved to the end of the arc. The ARC command has eight parameters. The first four parameters must always be given, the center point and parameters d and z are optional.

x1 , y1  	=  starting point of the arc or circle

x2 , y2 	=  ending point of the arc  ( for circle:  x2=x1, y2=y1 )

cx , cy	=  center point of the arc or circle. If cx and cy are omitted, the mid point of

(x1,y1) and (x2,y2) is used, which causes a half-circle to be drawn.

d      	=  direction for drawing the arc:

0  -  the arc is drawn clockwise

1  -  the arc is drawn counter clockwise   (the default)

z      	=  drawing mode:

1   -  complement  each pixel  (XOR)  (the default)

2   -  dot off  (AND)

3   -  set dot  (OR)

If the given ending point does not match with the arc defined by starting point and center, a line is   drawn from the periphery of the circle to the defined ending point. Usually, this should be avoided, so care must be taken to calculate the correct center point.

Note:  On 640.200 EL display panels the aspect ratio of each pixel is 1:2.  The ARC command automatically corrects the aspect ratio for arcs and circles. However, this may cause some of the pixels not to be drawn, if complement mode (z=1) is used. In this case, use modes 2 and 3 instead.

See also:  The Circle sub-command in the Zigzag command (page 5-16).

Examples:

A20,80,120,80,70,80,1,3

- Draws an ARC from point 20,80 to 120,80 counter-clockwise 				   around center-point 70,80.

A120,80,120,80,170,80

- Draws a CIRCLE at center point 170,80 with radius of 50 pixels,

using complement  mode.

_____________________________________________________________________________________

_____________________________________________________________________________________

8b.   ARC RELATIVE								a

a x1 , y1 , cx , cy , d , z <CR>

Draws an ARC from current pen position to specified relative coordinates. Coordinate values are relative to current pen position. Pen position is moved to the end of the arc. The ARC RELATIVE command has six parameters. The first two parameters must always be given, the center point and parameters d and z are optional.

x1 , y1  	=  end point of the arc, relative to current pen position

cx , cy	=  center point of the arc or circle. If cx and cy are omitted, the mid point of

current pen position and (x1,y1) is used, which causes a half-circle to be drawn.

d      	=  direction for drawing the arc:

0  -  the arc is drawn clockwise

1  -  the arc is drawn counter clockwise   (the default)

z      	=  drawing mode:

1   -  complement  each pixel  (XOR)  (the default)

2   -  dot off  (AND)

3   -  set dot  (OR)

Except for the entry of the coordinates, the ARC RELATIVE command works exactly as the ARC command.

See also:	ARC command on previous page

The Circle sub-command in the ZIGZAG command (page 5-16).

BEZIER command, approximating ellipses  (page 5-12).

Examples:

a100,0,50,0,0,3

- Draws a semi-circle of radius 50 to the right of current pen position,

clockwise, using "OR" drawing mode..

a0,0,50,0

- Draws a CIRCLE of radius 50 pixels to the right of current pen position,

using  default (XOR) drawing mode.

_____________________________________________________________________________________

_____________________________________________________________________________________

9.   BEZIER   									BZ

BZ x1 , y1  , x2 , y2 , x3 , y3 , z <CR>

bz  x1 , y1  , x2 , y2 , x3 , y3 , z <CR>

Draws a Bézier curve from current pen position to point (x3,y3). Pen position is moved to the end of curve. Command with capital letters uses absolute coordinates (relative to Origin), command with lower case letters uses coordinates relative to current pen position. Minimum of 6 parameters must be entered, the 7th parameter (z) is optional.

The parameters for the command are:

x1 , y1  	=  first control point

x2 , y2 	=  second control point

x3 , y3 	=  end point of curve

z 	=  drawing mode

Accepted values for drawing mode (z) are:

z = 0 	  replace mode  (the default)

z = 1 	  complement   (XOR)

z = 2 	  erase  (AND)

z = 3 	  set  dot  (OR)

Starting point of the curve is always the current pen position. Use Move command to set the start position, if necessary.  The end point must lay within the logical display area (x and y coordinates must be in range 0 to 2047). However, the control points (x1, y1, x2, y2) may be set outside the display area, if needed to get the desired shape. Use negative values if necessary.

Note: Line width setting (L command) has currently no effect on Bézier curves. Line type may be set, but the resulting line type is generally not what expected, especially for short curves.

Bézier curve is very flexible tool for creating all kind of curved shapes. You can easily approximate other curves, such as sine wave, parabola, ellipse etc.

Examples:

Approximating sine wave:

M0,200

BZ320,600,320,-200,639,200

Approximating an ellipse with 4 bézier curves:

bz0,25,50,50,100,50

bz50,0,100,-25,100,-50

bz0,-25,-50,-50,-100,-50

bz-50,0,-100,25,-100,50

_____________________________________________________________________________________

_____________________________________________________________________________________

10.   PAINT   (AREA FILL)							P

P xx , yy , p <CR>

This command paints or erases any form or shape of a closed area on the screen. The painting procedure is accomplished by defining a location ( xx,yy ) inside the area surrounded by a line. The painting algorithm then automatically updates all the pixels within the borders.

Accepted values for parameter p are:

p = 2	for ERASING the area

p = 3	for PAINTING the area

The speed of paint procedure depends on the complexity of the shape.  Filling large, complex areas may take up to several seconds.

Warning:  If the area is not totally closed, i.e. the boundary has holes in it, the paint algorithm may escape  from the boundaries and paint the whole graphics space!

Example:

A100,60,200,60,150,60,0,3 		- draws a half-circle

M100,60,3

P150,70,3				- fills the half-circle

P150,70,2				- erases the filled half-circle

Pattern fill can be done by combining paint command with XCOPY (X) and RECTANGLE (R) commands. Use the following procedure:

1. Prepare a buffer area somewhere in the virtual display area

2. Copy the figure from drawing area to buffer area using X command

3. Paint the figure in the buffer area using P command

4. Draw a rectangle (R) on the buffer area using selected pattern and ERASE drawing mode

5. Copy (X) the figure from buffer area to drawing area using OR drawing mode.

Example:

M200,200					- Draw a pie with slice

ZC100U99D99F70

X199,99,301,301,1199,99			- Copy to buffer area

P1250,200,3				- Paint in buffer area

R1199,99,1301,301,15,2			- Draw rectangle, erase mode

X1199,99,1301,301,199,99,3		- Copy back, OR mode

_____________________________________________________________________________________

_____________________________________________________________________________________

11.   TEXT 									T

T xx , yy , s , r , z , text... <CR>

Writes alphanumeric text starting from the position xx,yy. Coordinate pair xx,yy defines the lower left corner of the first character of the text. The maximum length of the text string is 80 characters.  The command is terminated by CR, as usual.  The pen position is moved to the end of the text.

Parameter  s  defines the character size (zooming factor) in range 0...15. The smallest size (s=0) is 8x8 or 8x16 pixels, depending on the font selected (see command SF). For larger characters, the character is zoomed by factor s+1.

Parameter  r  defines direction of the text.

16 directions  are available, see the diagram on the right.

Value 2 gives the normal text orientation, i.e. left to right.

If you add  8  to the value of r, the text is written using

slanted characters. If you add 16 to the value of r, the text

is written vertically rather than horizontally, in relation to

character orientation.

Parameter  z   is the drawing mode:

z = 0	Replace

z = 1	Complement  each pixel that is set in the pattern  (XOR)

z = 2	Erase each pixel that is set in the pattern  (-AND)

z = 3	Set  on each pixel that is set in the pattern  (OR)

See also: 	Set Attributes (SA) command on page 5-21

Set Font (SF) command on page 5-22.

Set Language (SL) command on page 5-24

Examples:

T100,100,4,2,3,Dimension

- Writes string "Dimension" to the screen starting from 100,100. Text is enlarged by factor of 5. Direction  is normal text, from left to right, horizontally. Text is printed normally, dot on.

T200,200,2,12,1,ALPHA

- Writes "ALPHA" vertically (top of the characters is to the left) using slanted characters. Characters are XORed with background image.

_____________________________________________________________________________________

_____________________________________________________________________________________

11b.   TEXT RELATIVE							t

t s , r , z , text... <CR>

Writes alphanumeric text at current pen position. Parameters s, r and z are as on the T command above.

Example:

t5,2,3,Hello		- Writes text "Hello" horizontally at current cursor position, zoomed

by factor 6 (5+1), from left to right (2), using replace mode (3).

_____________________________________________________________________________________

12.   TEXT,  SIMPLIFIED SYNTAX						K

K text... <CR>

Writes alphanumeric text using previously set parameters. Text start position is the current pen position. Other parameters (size, direction, drawing mode) are set by previous T command. The pen position moves to the end of text.

With this command, you can easily add more text at end of previously written text, e.g. after changing visual attributes using SA command.

Example:

KAlpha

SA1

K Point 			- Writes text "Alpha Point" at current pen position. Text "Point" is written

using boldface characters.

_____________________________________________________________________________________

_____________________________________________________________________________________

13.   ZIGZAG									Z

Z string <CR>

Draws a graphics shape (polyline) using a string of sub-commands. The maximum length of the string is 80 characters. Z-command actually provides a mini-language that gives you shortcuts from other graphic commands, significantly reducing the amount of data needed to transfer between the host and the terminal.

Summary of ZigZag sub-commands:

Command	Meaning	Command	Meaning

U n	draw or move up		C n	draw a circle of radius n

D n	draw or move down		P	Paint area within boundaries

L n	draw or move left		S n	set scale factor to n

R n	draw or move right		= n	set drawing mode to n

E n	draw or move up and right		T n	set line type to n

F n	draw or move down and right		W n	set line width to n

G n	draw or move down and left		I n	set ink (color) to n

H n	draw or move up and left

N	draw and return -prefix

&	combine directions -postfix

The basic sub-commands within string are the direction codes. These allow moving the pen or drawing a line to one of eight directions. See the diagram below:

H	U	E

L	+	R

G	D	F

A capital letter causes line to be drawn to the specified direction. Lower case letter causes the pen to be moved without drawing. The letter alone causes a move of one pixel. If the letter is followed by a numeric value, that specifies the number of pixels to move.

Any of the eight direction commands can be preceded by a N prefix. It tells controller to draw a line and then return pen to its original position. The direction command must immediately follow the N prefix.

The & postfix allows lines to be drawn to other than the eight directions above. If a direction command (with its parameter) is followed by &, the line is not drawn until after the next direction command. The resulting line is the vector sum of the two lines.

Example: to draw a line 10 pixels up and 20 pixels right from the current pen position, use command

ZU10&R20

Sub-command 'C' draws a circle at current pen position. Command character C must be immediately followed by a numeric parameter, which defines the radius of the circle. The pen position is not moved.

Sub-command  'P' is the PAINT command.  It fills a closed area defined by current pen position.  If the drawing mode currently active is 2, the area is erased. You must be sure that the pen is inside the area before using the P sub-command.

There are also several attribute command that modify the way lines are drawn. The attribute commands S, =, T, W and I are effective up to the end of current Zigzag string. At end of the Zigzag string, the global values are restored.

Sub-command  'S' sets the scaling factor. The number after S is the number by which future drawing commands will be scaled. For example, S4 causes the length of lines in the following commands to be enlarged by factor 4.  The scale factor must be in range 1 to 255.  At start of each Zigzag command, the scaling factor is set to the global value defined by SZ command.

Sub-command  '=' sets the drawing mode. Character '=' must be followed by a number in range 0 to 3 (as the z-parameter in the MOVE command). All lines following the =n are drawn using the drawing mode specified, until next =-command or end of the string. Default drawing mode (at beginning of each Z command)  is set by Set Zigzag (SZ) command. The four drawing modes are:

0	Move only

1	Complement  (XOR)

2	Erase  (AND)

3	Draw  (OR)

Sub-command  'T' sets the line type, just as the LINE TYPE (L) command. T must be followed by a singe number in range 0 to 7. The global line type, set by the L command, is restored at end of string.

Sub-command 'W' sets the line width. W must be followed by numeric value in range 1 to 255. At end of the zigzag sting, the global line width set by L command is restored.

Spaces within the string are ignored, if they appear between the sub-commands. However, there must not be spaces between a command and prefix or command and postfix.

Examples:

ZE8F8G8H8rP					- draws a filled diamond shape  (A)

ZS20 UNENH S15 UNENH S10 UNENHU			- draws a branch shape  (B)

(A)					(B)

_____________________________________________________________________________________

_____________________________________________________________________________________

14.   XCOPY									X

X x1 , y1 , x2 , y2 , xd , yd , z , s <CR>

Pixel Block Copy.  Copies a rectangular block of pixels from one part of virtual display area to other part. One of four drawing modes can be used.

Coordinate pairs x1,y1 and x2,y2 define the two opposite corners of the block to be copied. The two corners may be entered in any order.

Coordinate pair xd,yd defines the lower left corner of the copy destination. The source and destination blocks may overlap.

Optional parameter z is the drawing mode:

z = 0	Replace all pixels

z = 1	Complement  each pixel that is set in the block  (XOR)

z = 2	Erase each pixel that is set in the block  (-AND)

z = 3	Set  each pixel of destination that is set in the block  (OR)

If z parameter is not entered, it defaults to 0 (replace mode). In replace mode, all pixels in destination area are replaced by the pixels of the source. In all other drawing modes, only those pixels that are set in the source block are changed in the destination block. The result is the logical operation between source pixel and original value of destination pixel.

Optional parameter s defines scaling factor. If s = 0 or is not entered, the block is copied in original size. Values 1 to 15 cause the block to be enlarged by factor s+1 (2 to 16). Note: if this option is used, and source and destination blocks overlap (the source X coordinates are completely inside the destination block), the copy operation may not be performed correctly.

Examples:

X100,50,300,150,500,200			- Copy using replace mode

X0,0,639,16,0,384,1			- Copy using XOR mode

X0,0,100,100,200,50,3,1			- Copy using OR mode, enlarge by factor 2

- The following set of commands draws a filled circle on top of brick wall pattern. The circle is first drawn outside of display area and then copied on top of the pattern using OR mode.

R100,100,300,200,14

M950,100

ZC45P

X900,50,1000,150,150,140,3

For more examples, see PAINT command.

_____________________________________________________________________________________

_____________________________________________________________________________________

15.   MOVE VIEWPORT 							V

V xx , yy , w , hh  <CR>

Moves the display viewport within the 2048 x 2048 pixel graphics space and optionally defines split screen windows.

Parameter pair xx,yy defines the lower left corner of the window. Values for xx and yy are absolute coordinates, not affected by the Origin command. If too big values are given for xx or yy, the value is automatically truncated to the largest possible value.

Vertical smooth scrolling can be achieved by stepping yy in increments of 1. However, the xx coordinate can be changed only at 16 pixel increments (the value is rounded downwards).

Optional parameter w selects the window that is affected by the command. Possible values are:

0	=	Upper window (the default)

1	=	Lower window

The two windows can be scrolled and panned independently of each other.

Optional parameter hh can be used to split the screen into two horizontal windows. The value is the Y-coordinate of the split position, i.e. the height of the lower window. Value 0 causes only the upper window to be displayed. After entering the Graphics Mode, only the upper window is displayed and both windows are set to coordinates (0,0).

Up to 15 complete screens can be displayed on 640.400 display panel, or up to 30 screens on 640.200 display panel. Simple animation effects may be produced by drawing slightly different pictures on each screen and then quickly changing the viewport to show each picture in turn.

Examples:

V640,400			- Moves (upper) window to coordinates X=640, Y=400

V0,500,0,100		- Splits screen into two windows and moves upper window to 0,500.

V1280,950,1		- Moves lower window to coordinates X=1280, Y=950

V0,0,0,0			- Resets viewports to default state: only one window, at 0,0.

_____________________________________________________________________________________

16.   ORIGIN									O

O xx , yy  <CR>

Moves the origin point to  xx,yy.  After receiving this command, all the graphics coordinates are relative to this origin point (except commands V and O, which always use absolute coordinates). This allows easy repositioning of the figures on the screen.

After entering the graphics mode or after init (I) command, the origin is set to (0,0).

_____________________________________________________________________________________

_____________________________________________________________________________________

17.   SET ATTRIBUTES							SA

SA n <CR>

Set attributes for text commands. Parameter n is a single decimal number to select one or more of the text attributes:

0  =  Normal text

1  =  Boldface

2  =  Underlined

4  =  Inverse

Multiple attributes may be selected by adding values together.

Example:

SA5			- Sets bold and inverse attributes

_____________________________________________________________________________________

18.   SET BAUD etc.								SB

SB baud , parity , bits , stopb , hshake , duplex  <CR>

Sets the serial port parameters.

baud	- Baud rate: 300 to 19200 bps  (parameter values 7 to 1, respectively)

parity	- Parity:  3 = even, 2 = odd,  1 = none

bits	- Number of data bits:  2 = 7 bits,  1 = 8 bits

stopb	- Number of stop bits:  2 = 1 stop bit,  4 = 2 stop bits

hshake	- Handshake protocol:   1 = Xon-Xoff,  2 = Hardware, 3 = both, 4 = none

duplex	- 1 = Full duplex,  2 = half duplex

If any of the parameters are zero or omitted, that value is not changed. See terminal mode command SET LINE PARAMETERS for more information about the parameter values.

Example:

SB4,,,,2			- Sets baud rate to 2400 bps, handshake to hardware (CTS), and 					  does not change other settings.

_____________________________________________________________________________________

_____________________________________________________________________________________

19.   SET CLICK								SC

SC beltime , tsoclick<CR>

Parameter beltime sets the BEL sound time in units of 10 ms. The default value after power-on is 20, i.e. the bell sound time is 200 ms. If bell time is set to 0, no bell sound is generated.

Parameter tsoclick enables (1) or disables (0) the Touch Screen click sound.

_____________________________________________________________________________________

20.   SET FONT									SF

SF h , w <CR>

Selects the font used by text commands (T and K).

Parameter  h selects the font size (height):

8	=  8x8 font      (or 6x8   on the EL512.256 display)

16	=  8x16 font    (or 8x14 on the EL640.350 display)

The actual font depends on the display used. Some displays may only have one font size available.

Note:  Some of the character sets may not be available for 8x8 font size.

Optional parameter w selects the character spacing (width) in pixels. It does not have effect on the size or shape of the characters, only the space between characters. Default spacing is 8 (or 6 for 6x8 font). When terminal mode is entered, character spacing is reset to that default.

_____________________________________________________________________________________

21.   SET INK  (COLOR)							SI

SI color <CR>

Selects the color for all subsequent drawing operations. Parameter color is the color number. The range of colors available depend on display type in use. For red-green EL-display in 4-color mode, the possible values are:

1  =  Red

2  =  Green

3  =  Yellow

The color mode is based on bit planes. If, for example, red color is selected, only red bit plane is written or erased, and green bit plane is not affected. Writing red over green produces yellow.

On monochrome displays, this command is ignored.

_____________________________________________________________________________________

_____________________________________________________________________________________

22.   SET LANGUAGE								SL

SL g0 , g1 , natmod <CR>

Selects the language (national replacement character set) used by text commands (T and K). By default, the character set defined by the language field of the SETUP screen is in use. The character set may also be set by using Terminal Mode commands; it is not changed when entering Graphics Mode.

SL command allows you to use multiple character sets on the same screen. You can also use semigraphic and special characters and soft fonts.

Parameter g0 selects the G0 character set, which is normally mapped to GL (characters 20h to 7Fh).  The following character sets are available:

0	=  USASCII			9	=  Italian

1	=  Finnish / Swedish		10	=  Dutch

2	=  German			  (11-15   reserved)

3	=  Danish			16	=  Semigraphics

4	=  British			17	=  DEC multinational set

5	=  Spanish			18	=  IBM-PC extended set

6	=  French			19	=  ADM3 semigraphics

7	=  Norwegian			20	=  Soft Font buffer 0

8	=  Swiss			21	=  Soft Font buffer 1

Parameter g1 selects the G1 character set, which is normally mapped to GR (characters 80h to FFh).  The following character sets are available:

0	=  USASCII			4	=  IBM-PC extended set

1	=  National			5	=  ADM3 semigraphics

2	=  Semigraphics			6	=  Soft Font buffer 0

3	=  DEC multinational		7	=  Soft Font buffer 1

Optional parameter  natmod  sets or resets the national mode.

0	= National mode off

1	= National mode on

If national mode is ON, the keyboard sends 7-bit national characters from the national set selected by g0 parameter. Only those characters included in the selected national set can be sent.

If national mode is OFF, the keyboard sends 8-bit characters from the DEC Multinational character set.

Examples:

SL16			- Select semigraphic character set

SL0,4			- Select ASCII set to GL, IBM-PC set to GR

SL6,4,1			- Select French national set  + IBM set and turn National Mode ON

_____________________________________________________________________________________

_____________________________________________________________________________________

23.   SET TSO									ST

ST mode , delay , rate , speed <CR>

Sets the Touch Screen Overlay operating mode and timing. (See chapter 1.3 for more information.)  Parameter  mode  sets the TSO operating mode:

0	=  Normal

1	=  Debug mode  (incomplete coordinate pairs also sent)

2	=  Finger-up detect

4	=  Double resolution mode

8	=  Query mode  (send report only on query)  (E)

16	=  Network mode  (use network handshake, insert net address

at the beginning of each message)  (E)

32	=  (reserved)

64	=  Single character mode (convert coordinate-pair to a single character

using TSOTABL).  (E: convert control characters)

128	=  Use TSO message format compatible with old Alphasoft

version (V2.1 or older).

256	=  Disable TSO operation

Multiple modes may be selected by adding values together. (Note: Only options marked with (E) are available with external TSO interface.  See chapter 4.1 for external TSO commands.)

Parameter  delay  sets the delay before repeating the TSO message, when the finger is hold on the same position on the screen. Value for delay must be in range 1 to 255. 1 gives the shortest delay. The default value may be set by patching the EPROM, normal value is around 8.

Parameter  rate  sets the repeat rate.  1 is the fastest, 255 the slowest rate. If rate is set to 0, TSO repeat is disabled. Normal value is 2, but may be changed by patching the EPROM.

Parameter  speed  has different meaning on optical and membrane TSO:

a) on OPTICAL TSO:

speed  is the tracking speed, that is, the rate on which coordinates are sent when the finger is moved around the screen. 1 gives fastest rate, 255 the slowest. If speed is set to 0, new coordinates are sent only after finger is removed and re-inserted. Default value after power-on is 1 (but this can also be changed by patching the EPROM).

b) on MEMBRANE TSO:

speed  is the scanning rate for the switch matrix, ms / row.  Default value is normally set to 2.

Parameters  delay, rate and speed  are optional.  If any of these are not given, these settings are not changed.  These three parameters have no effect on external TSO interface.

Examples:

ST256			- Disable TSO operation.

ST6,3,1			- Enable TSO, select finger-up detect and double resolution mode

and set shorter times for repeat delay and repeat rate.

_____________________________________________________________________________________

_____________________________________________________________________________________

24.   SET SCREEN SAVER 							SS

SS time <CR>

Sets Screen Saver time, i.e. the time after which the screen is blanked, if no characters are received from keyboard, touch screen or host computer. Parameter time is the screen saver time in minutes, 1 to 255.  If time is set to 0, screen saver is disabled.

Note:  If you set screen saver time to any other value than the options available on Setup screen, the Screen Saver field in the Setup id displayed as blank.

If SS command is given without parameters, the screen saver time is not changed, but screen is blanked immediately.  Any character received from the host, keyboard or touch screen overlay will restore the display.

_____________________________________________________________________________________

25.   SET ZIGZAG								SZ

SZ s , a , m <CR>

Sets the default parameters for the Zigzag command.

Parameter  s  sets the default scaling factor in range 0 to 255. Line length of each Zigzag direction command is multiplied by this factor, thus changing the size of the picture. If the scaling factor is set to zero, the zigzag command won't draw anything. The scaling factor may also be changed locally within the Zigzag command string by the S sub-command. After entering the Graphics Mode, the Zigzag scaling factor is set to 1.

Parameter  a  sets drawing angle for the zigzag direction commands. This allows the whole picture to be rotated in steps of 45 degrees. Accepted values are 0 to 7, which cause the picture to be rotated 0, 45, 90 etc. degrees counter-clockwise. Note, however, that if your picture contains diagonal lines, you should avoid using odd values (rotation of 45 degrees etc.), since that may cause the picture to be distorted. If the parameter a is not given, the rotation angle will not be changed. Initial value for the rotation angle is 0.

Parameter  m  sets the default drawing mode, which is active at the start of each Zigzag command. Value must be in range 0 to 3, as the z parameter in the Move command. If m is not given, the value will not change. Default mode after Init command or reset is 1 (XOR mode).

_____________________________________________________________________________________

_____________________________________________________________________________________

26.   HARD COPY								H

H n , p <CR>

Prints a paper copy of the display memory through the serial printer interface (K4), if connected.  Display memory is copied pixel by pixel to an Epson FX-80 compatible printer (the printer must support the Esc * commands).

Parameter n defines the printed area and print density:

n = 0	Prints the currently displayed screen area (640 x 400 pixels on 640.400 display)

n = 1	Prints the graphics area from 0,0 to 1280 x 800 (this was the whole graphics area on

the AP988030 display controller).

n = 2	Prints 570 x 400 pixels from the currently displayed screen area to give 1:1 dot ratio

on Epson FX-80 printer.

n = 3	Prints the currently displayed screen area (640 x 400) using 1:1 dot ratio.

Note: this option requires  wide paper (> 22,5 cm) to be used.

Optional parameter p defines single or dual pass mode:

p = 1	Single pass (the default)

p = 2	Dual pass

Dual pass mode gives better print quality, but is slower.

While the printing is in progress, a message is shown on the bottom of the screen. If printer is not ready, a bell sound is generated and the hard copy operation exit.

_____________________________________________________________________________________

26b.   HARD COPY TO HOST						?H

?H n , p <CR>

Sends a hard copy to host port. The data sent to host is exactly the same as the data for printer on the HARD COPY command. Host computer can then store the data into a file and print locally. The printer must be FX80 compatible.

For explanation of the parameters, see the HARD COPY command above.

_____________________________________________________________________________________

_____________________________________________________________________________________

27.   NET TARGET								#

# a1 , a2 , a3 , ... , a8 <CR>

Selects the net target(s). This command is used in systems where several AP988300 terminals are connected on the same serial line (e.g. by using RS485 or Current Loop  interface).

After receiving this command, only the selected terminal(s) will execute any subsequent commands, until next # command is received. For inactive terminals, the only command recognized is the # command.

Parameters a1 to a8 are the addresses of the terminals. Each address must be in range 1 to 99. Up to 8 terminals can be selected active on a single command. All other terminals will be turned inactive. However, # command with parameter 0 selects all terminals (broadcast).

The address of the terminal is selected from the NET ADDR field in the SETUP mode. Default address may also be patched in the EPROM.  If address of the terminal is set to 00, the terminal will ignore any # command, and thus executes all commands.

Example:

#0

T100,100,2,2,0,This terminal is inactive

#3,4,12

C

T100,100,2,2,0,This terminal is active

B50,50,600,200

After the above commands, the terminals 3, 4 and 12 show the text "This terminal is active" and a box around it. In all the other terminals remains the text "This terminal is inactive".

Note:  You can also use the SOH ... STX sequence to select the net target. It works both in graphics mode and terminal mode.  See 'Multipoint Communications' in chapter 4.1 for more information.

See also:  Enable Transmission (DC2) in chapter 4.1,  'Multipoint Communications'.

_____________________________________________________________________________________

_____________________________________________________________________________________

28.   DEFINE MACRO								: ... ;

: n <CR>

commands

commands

;  <CR>

Macro is a group of standard graphics commands, which can be executed simply by calling it by name (see CALL MACRO command on the next page).  Executing a macro is much faster than sending equivalent commands through serial line.

The macro definition starts with a colon (:), followed by the name of the macro. The macro name n  is a single ASCII character. Most ASCII characters are accepted, but it is recommended to use only numbers 0 to 9 and letters A to z. A capital letter and lower case letter are interpreted as two different names. Characters in range A0h to FFh can also be used as macro name.

After that follows the macro text, which may contain any Graphics Mode commands using normal syntax. Each command is terminated with Carriage Return, as usually (Line Feeds are not needed). All displayable characters are accepted, except semi-colon (;), which is the terminator character. A line containing just semi-colon (;) ends the macro definition.

When a macro is defined, any old macro with the same name is deleted. If ROM macro with same name exists, the new definition overrides the ROM macro.

The macro can be deleted with command

: n ; <CR>

where n  is the name of the macro.  To delete all macros in the RAM, use command

: * ; <CR>

When a user defined macro is deleted, any ROM macro with same name becomes available again.

The macros are stored in the battery backed CMOS RAM, and thus remain in the memory even if the power is turned off.

The number of macros is limited only by available different macro names and by free macro space. The macro space available may vary from version to version. Currently, 4095 bytes are reserved for macros. Macro space may be expanded by installing larger RAM on-board (consult factory). See also the MACRO BUFFER REPORT (?:) command on page 5-Error: Reference source not foundError: Reference source not found.

Example:

:T						- Define a macro

B0,0,200,50

T20,20,1,2,3,Testing

;

@T						- Call macro

:T;						- Delete the macro

_____________________________________________________________________________________

_____________________________________________________________________________________

29.   CALL MACRO								@

@ n <CR>

Executes a macro, which may be defined by user (see previous page) or stored in the ROM. A macro may contain any number of standard graphics commands, which are then executed exactly as if received from the host, except without the delay caused by data transfer.

The single character n immediately following @ is the name of the macro.

Macro call command scans first the downloaded RAM macros and, if not found, the ROM. Thus, any macro in ROM may be re-defined by user. If the specified macro is not found, the command is ignored.

The graphic generated by a macro call may be positioned at different locations on the display by changing the Origin point (command O) or, if the macro contains only relative coordinate commands, by moving the pen position.  If the macro contains only Zigzag commands (Z), the generated picture can also be re-sized and rotated using SET ZIGZAG (SZ) command before calling the macro.

A macro may call another macro. A RAM-macro may call ROM-macro or vice-versa. However, macro should not call itself, since this would cause an infinite loop or stack overflow.  If same macro needs to be executed repeatedly, use the TIMER command (>), see page 5-Error: Reference source not found.

The ROM macros are stored in the EPROM starting from label MACROS.  Consult factory for user specific EPROMs.  By default, only a few demo macros are included.

Examples:

@D				- Executes the Demo macro (pie chart etc.)

@G				- Starts Blinking demo

@1				- Start blinking

@0				- Stop blinking

The examples above call ROM macros, which are normally included in the Alphasoft EPROM. However, these macros may not be available on all EPROM versions.

_____________________________________________________________________________________

_____________________________________________________________________________________

30.   MACRO BUFFER REPORT						?:

? : <CR>

Asks the macro buffer status report. After receiving this command, the terminal sends the report to host computer in format

= free_space , total_space <CR> <LF>

Where free_space is the number of bytes free in the macro storage area, total_space is the total size of macro storage area. Both values are displayed as decimal numbers, up to 5 digits, without leading zeros.

Example of report sent to host:

=3580,4095

_____________________________________________________________________________________

31.   MACRO ID REPORT							?@

? @ src <CR>

Asks the macro ID report, i.e. names of the macros currently in the terminal memory. Optional parameter src selects the macro source:

0	= RAM macros  (the default)

1	= ROM macros

After receiving this command, the terminal sends the report to host computer in format

= macro_list <CR> <LF>

Where macro_list  is a string of characters, each character being a name of a macro. If no macros have been defined, the macro_list contains no characters.

Example of report query (ask RAM macro names):

?@

Example of report sent to host:

=ABChjk

See also:	DEFINE MACRO  (:...;)   on page 5-Error: Reference source not found

CALL MACRO     (@)    on page 5-Error: Reference source not found

_____________________________________________________________________________________

_____________________________________________________________________________________

32.   TIMER									>

> delay , id <CR>

Starts a macro after specified time delay. The timer command can be used to generate repeated events, such as blinking, or creating automatically rolling demos etc.

Parameter delay defines the time after which the macro is started, in units of 10 ms. Thus, value of 100 generates delay of 1 second. Largest possible value is 65535, which generates delay of over 10 minutes. If delay is set to 0, the macro is not executed and any previous timer operation is canceled.

Parameter id is the name of the macro (one letter, without the @ prefix). If the specified macro does not exist, the timer starts, but no macro is executed after delay.

Only one timer operation can be active at a time. If new TIMER command is entered while a timer is active, the old timer command is discarded.

Examples:

>500,A			- Starts macro "A" after 5 seconds

:1			- Define two macros to create blinking effect

V0,0			   by switching between two screens

>50,2

;

:2

V0,400

>50,1

;

>1,1			- Start the blinking macro after 10 milliseconds

>0,0			- Stop blinking

_____________________________________________________________________________________

  