2.  INSTALLATION

2.1  CONFIGURATION OPTIONS

The default configuration of the AP988300 controller board contains:

- Display panel interface for the display type selected

- Host serial interface, RS232C.

- Serial printer interface, RS232C

- Optical Touch Screen Overlay interface

- Membrane type Touch Screen Overlay interface (requires special EPROM)

Can also be used as parallel printer interface.

- Interfaces for both VT220 and IBM-PC compatible keyboards

- On-board beeper

- On-board DC/DC converter for RS232C voltages

- On-board Switched Mode Power Supply, to generate the 5V operating voltage

for both the AP988300 and  the display panel

- CMOS RAM with battery backup

- Alphasoft-300 EPROM for the selected display type and resolution

Optionally, the AP988300 controller board may include:

- RS485 or Current Loop for host serial interface

- ARCNET network interface, either as coaxial or RS485 version

- 12V regulator for keyboard and Touch Screen (when using operating voltage

higher than 12V)

- 5V power output connector

- Power LED indicator

- Customized Alphasoft-300 version

- Large EPROM for Kanji characters etc.

Optional accessories:

-  Display cable for the selected display type (EL, Plasma, etc.)

-  Host serial cable

-  Printer cable

-  Mechanical assembly accessories

-  Keyboard  (VT220 or IBM-PC compatible)

-  An AC power supply

(Input: 115 or 230 VAC,  Output: 12 VDC, 3 Amps).

2.2  ELECTRICAL CONNECTIONS

Fig. 2.1:  I/O connectors

When installing the AP988300 in your system, the following electrical connections are needed. (Some of the connections are optional, depending on your system requirements.)

1.	Connect the power supply to the Power Input -connector (PK1). Do not switch the power on until all the connections have been made. The operating voltage may be in range 9 to 32 volts, depending on the display type used (check your display panel data sheet for correct voltage). The 5V operating voltage is generated by an on-board Switched Mode Power Supply (SMPS).  The AP988300 uses only the 5V voltage, and also supplies 5V to the display panel. However, the VT220 keyboard and the TSO both need a 12V supply. If these peripherals are connected and operating voltage larger than 12V is used, an optional 12V regulator has to be installed on-board.

Some display panels, such as plasma panels, may require additional operating voltages. Those voltages must be connected externally to the display panel.

2.	Connect the display cable from the display connector (K9) to the display panel. Make sure that the pin1 of the cable connector is connected to the pin 1 of the connector on the PCB.

3.	Connect the host serial cable to connector K6. A flat cable with IDC connectors may be used to connect to an 9-pin D connector. The pin order is compatible with a standard 9-pin serial port connector. The AP988300 may act either as a Data Terminal Equipment (DTE) or as Data Communications Equipment (DCE), as selected by jumpers J11 and J12. You must be sure to use correct type of cable (direct or crossed wire), depending on the type of host serial port (DTE or DCE). Only three wires need to be connected: the data lines (TxD, RxD) and ground. If the hardware handshake is used, the handshake lines (CTS, RTS, DTR) may also be needed. See chapter 6 for pin descriptions.

4.	Connect the serial printer to port K4 (if required). Select the DTR handshake input pin, using jumper J14, to suit your printer. The printer must supply hardware handshake; the Xon-Xoff handshake is not supported. Printer baud rate etc. can be selected from the SETUP.

5.	Connect the VT220 compatible keyboard (if required) to connector K3 (a 4-pin telephone type connector), or an IBM-PC/XT compatible keyboard to connector K7 (a 5-pin DIN connector). A serial ASCII keyboard may also be used on connector K3 (see chapter 6.2 for jumper settings). WARNING: The VT220 keyboard uses the 12V power supply. If you are using other than 12V power supply voltage, the optional 12V regulator must be installed.

6.	Connect Touch Screen Overlay (TSO), if required. Optical TSO is connected to connector K2. Membrane TSO or switch matrix type keyboard is connected to connector K1. Only one type of TSO may be connected at a time. The standard Alphasoft-300 firmware contains support for the optical TSO. Membrane TSO requires a special version of the Alphasoft-300 EPROM.

See chapter 6.3 for the pin descriptions for each connector.

2.3  TESTING THE INSTALLATION

	After you have gone through the installation on the chapter 2.2 successfully, turn ON the power on your new AP988300 terminal.

You should see a cursor at the upper left corner of the display (unless the cursor display has been disabled from the SETUP). If the mode select jumper (J10) position 8 has been connected, an introduction screen is displayed. The screen is then cleared, when any key is pressed or a character is received from the host computer.

	If the keyboard is available, press function key F3 in the upper row of the keyboard to invoke the SETUP-mode.

On other than VT220 or IBM-PC keyboard, some other key may be used as setup-key. See your keyboard specific application note. If the keyboard does not have the setup-key, the SETUP-mode may be invoked by pressing Ctrl-Shift-@ twice.

SETUP screen should appear (see fig. 3.7 at chapter 3.2). Use cursor keys to move the highlight bar  around on the display. You now know, that both display and keyboard are working.

	If you have Touch Screen Overlay (TSO) installed, touch the screen and move your finger around. You should hear click sounds when you move your finger.

	For testing the serial port, you may perform a loop test. Connect the Transmit Data pin of the serial port directly to Receive Data pin. (On D-connector, short circuit pins 2 and 3. Or, on AP988300 board connector K6, short circuit pins 3 and 5. See chapter 6.3.)

Now enter characters from keyboard or from the TSO. The characters should appear on screen (even in Full Duplex mode).

In order to the system to work properly, correct serial port parameters need to be set. The parameters may be set from the keyboard by using the SETUP mode, see chapter 3.2.

If your system configuration does not include a keyboard, and the standard serial port parameters (see chapter 3.2)  are not suitable, the setup parameters may be set by patching the EPROM (see chapter 7.2). Some of the parameters can also be changed by sending commands from host computer (See chapter 4.1).

Use the SETUP-menu according to chapter 3.2 instructions to modify the host communications and AP988300 local parameters to match the requirements of your host hardware/software. Set the serial parameters so that they match the settings on the host computer. Also make sure that your host computer has been set up to recognize the AP988300 either as a VT100-,  VT52 or ADM3 type terminal.

If you are using a VT220 or IBM-PC compatible keyboard, make sure that the KEYBOARD field in the SETUP-menu contains the proper setting for your national keyboard. Note: the national layouts are defined according to the VT220 keyboard, and may not match exactly on all national layouts of the IBM-PC keyboards.

Once you are satisfied with the SETUP parameter settings, press the F3-key once again to change from SETUP mode into the ON-LINE mode and to save the settings in nonvolatile memory.

	You may test the terminal features by entering text and commands from the keyboard. For this purpose, you may choose HALF DUPLEX or LOCAL MODE from the DUPLEX field of the SETUP (or use loop test described above).  In half duplex mode, the commands and characters are both sent to the host computer and displayed locally. In local mode, the characters are only displayed locally. Don't forget to change to on-line mode (FULL or HALF duplex) before trying to connect to the host computer.

2.4  TROUBLESHOOTING

Before calling Alpha Point or authorized service center, please check the following items:

| Problem | Check these items |
| --- | --- |
| No picture, no response from keyboard or TSO | - Correct operating voltage connected? (See chapter 2.2) - EPROM properly in socket? |
| No picture, keyboard and TSO responding OK | - Any additional voltages required by display panel connected? - Display cable (K9) properly connected? - Correct EPROM version installed? (See chapter 7.1) - Display panel undamaged? |
| Keyboard not responding | - Correct keyboard type in use (e.g. PC/XT vs. PC/AT kbd)? - Correct EPROM version for selected keyboard? - Jumper J10 pos. 1 set correctly (open, except for ASCII kbd) - VT220 keyboard: 12 Voperating voltage available? (e.g. when using 24 V for display panel) |
| Does not send/receive characters from host | - SETUP field DUPLEX must not be set to LOCAL. - Jumper J8 set correctly, J11, J12 and J19 connected? - Perform loop test (see previous page). If loop test fails, the serial port drivers may be broken and the board needs servicing. |
| Does not send/receive characters from host. Loop test passed. | - Correct type of cable in use (direct / crossing)? - Correct handshake mode (Xon-Xoff / CTS) - Does host computer require hardware handshake lines? - Jumpers J8, J11, J12 (and J7, J19, if CTS handshake used) correctly set? (see chapter 6.2) |
| Does not send/receive characters on RS485 or current loop i/f. | - Jumpers J8, J17, J23, J25, J26 and J27 set correctly? - Current loop: 20 mA current source provided? |
| Garbage characters received from host | - Baud, parity, bits settings on SETUP correct? - Jumper J8 connected? - Cable OK? (check ground wire!) |
| RS485: garbage characters received | - Biasing resistors installed (for multipoint comm.)? - Cable OK? (correct polarity, ground connected) |
| Touch Screen not responding | - Mode select jumper J10 position 5 installed? - Correct EPROM version installed? - External TSO interface: operating voltage connected? |
| Printer not responding | - Printer baud, parity etc. correct? - Jumpers J13 and J14 set correctly? |
| Scrambled display, unsynchronized or flickering display | - Correct EPROM for selected display? (See chapter 7.1) - Correct crystal frequency for selected display? |
| Does not execute commands correctly. | - Correct emulation selected from SETUP? |

APTEST program (for IBM-PC compatibles) is available from Alpha Point for easier testing of the terminal configuration.

Rev B board only

Rev C board only