AP088300  Rev C

Errata for Users Manual  V4,4, April 1995

Changes in the Users Manual for Rev C board.

Page 1-2:

At the sentence mentioning ARCNET, add footnote:

Rev B board only.

Page 2-1:

At the paragraph Optionally, the AP988300 controller board may include:

- add footnote reference 1

- add the line with footnote reference 2

- add footnotes 1 and 2

- ARCNET network interface, either as coaxial or RS485 version1

- Large EPROM for Kanji characters etc.2

1 Rev B board only

2 Rev C board only

Page 6-3:

Add footnote reference (*)  in the jumper table at  J2, J4, J4, J5, J20, J23 and J27

Add footnote:  * These jumpers are not available on Rev C board

Change jumper J21 entry in the table to:

J21	EPROM type	Rev A,B	(3)	27512		27256

Rev C	(3)	4M		1M, 2M

Page 6-7:

At end of page, add:

Note:    ARCNET interface option is not available on Rev C boards.

Page 6-9:

At the entry for K7, IBM PC Keyboard Interface, add the following paragraph:

Hardware note:  If IBM-AT type keyboard is used, pull-up resistors of around 10K must be connected on the keyboard CLK and Data pins. On Rev C board, these resistors are already installed.

Page 6-12:

Replace the line about EPROM with these two lines:

EPROM:	max   64 K  (27256 or 27512)  on Rev B board

max  512K  (27010 to 27040)  on Rev C board