6.  HARDWARE DESCRIPTION

6.1.  ON-BOARD PERIPHERALS

6.1.1.  BEEPER

The BEL sound is normally generated by the build-in beeper of the VT220 keyboard. The AP988300 controller board also has an on-board beeper. It generates the bell sound, when the keyboard is not connected or other type of keyboard is used. The on-board beeper also generates a key-click for the Touch Screen Overlay and IBM-XT keyboard. The duration of BEL sound can be set using SET CLICK command, see chapter 5.

6.1.2.  POWER SUPPLY

The AP988300 needs only single operating voltage, in range 9 to 32 VDC. The voltage may be chosen as required by the display panel. A common value is 12 V.

The on-board SMPS (Switched Mode Power Supply) generates the 5 VDC operating voltage for both the AP988300 display controller and the display panel. The 12 V input voltage is not used internally by the AP988300.

Warning:  The VT220 keyboard and TSO frame board both use 12V operating voltage. If higher supply voltage is used and one of above devices is connected, the optional 12V regulator (IC28) must be installed on-board.

The +12V and -12V voltages for the RS232C interface are generated on-board by the RS232C interface chip.

Fig. 6.1:  AP988300 printed circuit board

6.2.  JUMPER SETTINGS

The following jumper options are available on-board. (Default setting underlined.)

Mode Select  jumper block  (J10):

o o o o o o o o

J10	o o o o o o o o

Position:	8 7 6 5 4 3 2 1

Position	explanation	Open	Closed

1.	Keyboard type	VT220 or IBM  	ASCII

2.	Debug mode	Normal	Debug

3.	(reserved)	     -	     -

4.	(reserved)	     -	     -

5.	TSO enable	Disabled	Enabled

6.	RS485 mode	Disabled	Enabled

7.	Battery back-up	Enabled	Disabled

8.	Display power-up screen	No	Yes

It the TSO (Touch Screen Overlay) is not in use, the J10 position 5 should be left open, to disable the TSO scanning. Otherwise false coordinates may be generated. TSO scanning also consumes some CPU time.

The RS485 mode (J10.6) should be enabled, if multi-drop RS485 interface is used. When an RS232C interface is used, or RS485 interface is used for point-to-point communications (RS422 compatible mode), this jumper should be left open. For more information, see "Connector K10" later in this chapter.

The Battery back-up disable jumper (J10.7) may be used, when a new EPROM is changed on the AP988300 board. Closing this jumper ensures that new SETUP defaults are read from the EPROM. After that, the jumper may be removed. The default values can also be read from EPROM by pressing Ctrl-D at the SETUP mode. Downloaded macros are kept in non-volatile RAM regartless of this jumper.

Note:  The J10 jumper block is read by software only at power-up or reset.

Other jumpers:

Pins connected:

Jumper	explanation			  size	  1 to 2		2 to 3

or open		or closed

J2	LCA load data input  *		(3)	external		CPU

J3	LCA external load input connector  *	(5)		(open)

J4	LCA load clock input  *		(3)	external		CPU

J5	LCA load mode select   *		(3)	external		CPU

J6	Watch dog enable 		(2)	disabled		enabled

J8	Host port data input select 	(3)	RS232C		Current loop or RS485

J9	Host port DTR output, force high 	(2)	no		yes

J13	Printer port TxD pin select 	(3)	DTE		DCE

J15	IBM PC keyboard reset enable  	(2)	disabled		enabled

J16	Host port TX data:  hw-Break 	(3)	enabled		disabled

J17	Current Loop Rx Data enable 	(2)	disabled		enabled

J18	Display connector Blank pin select 	(3)	GND		enabled

J19	Host port CTS input pin select 	(3)	DCD		CTS

J20	(Not used)  *		(2)		( open )

J21	EPROM type	Rev A,B	(3)	27512		27256

Rev C	(3)	4M		1M, 2M

J22	RS485 RxD terminator resistor 	(2)	no		yes

J23	RS485  TxD source  *		(3)	async port	ArcNet

J24	RS485 TxD terminator resistor 	(2)	no		yes

J25	RS485 configuration 		(3)	2-wire		4-wire

J26	RS485 (2-wire) receive enable 	(3)	always enabled	software sel.

J27	RS485 RX/TX select source  *	(3)	async port	ArcNet

* These jumpers are not available on Rev C board

1	2	3	4	5

J7	Host port CTS/DCD input select:

a)  Disabled	o----o    o    o----o					b)  CTS	o----o    o----o    o

c)  DCD	o    o----o    o----o

If hardware (CTS) handshake input is to be used, jumper J7 should be set according to option b). If the hardware handshake is not used,  J7 should be set according the option a), or connection should be made on the cable to set the handshake line always ON. The handshake output is enabled by using SETUP screen. See also J11 and J19.

J11	Host port  CTS / RTS pin select:

J12	Host port  TxD / RxD pin select:

DTE		  DCE

Jumpers J11 and J12 can be used to set		J11:	1 o---o 2		1 o   o 2

the RS232C  pin configuration either as			3 o---o 4		3 o   o 4

DTE or DCE. The default setting is DTE.		J12:	1 o---o 2		1 o   o 2

3 o---o 4		3 o   o 4

J14	Printer port handshake input select:		K4 pin 	D-sub

Select one of the four jumper positions to		J14:	1 o---o 2		6	( 8 )

choose the hardware handshake pin suitable			3 o---o 4		7	( 4 )

for your printer. (See also J13.)				5 o---o 6		2	( 6 )

7 o---o 8		4	( 7 )

6.3.  INTERFACES

PK1  -  POWER SUPPLY CONNECTOR

The operating voltage (9 to 32 VDC) for both AP988300 and the display panel is input through this connector. The voltage must be chosen according the requirements of the display panel (see chapter 6.1.2).

1	+9...32V

2	GND

PK2  -  5 VDC POWER OUTPUT

The regulated 5 V output from the on-board DC-DC converter is output from this connector (if installed). It can be used for external devices requiring the 5V operating voltage. However, this connector is not needed for the display panel, since the operating voltage for the display is output through the display connector. The maximum continuous output current is 500 mA.

1	+5V

2	GND

K1  -  MEMBRANE T.S.O. INTERFACE / NET ADDRES SELECT

Membrane switch type Touch Screen Overlay input device (TSO) is connected to this connector. You may also connect an undecoded matrix keyboard here. A special version of the Alphasoft-300 EPROM is required for these devices. This interface can also be used for Net Address select switch.

The membrane TSO is a matrix of contacts, each of which connects one of the column lines to one of row lines. The AP988300 scans the TSO matrix by supplying low level voltage to one of the 12 column lines at a time and then reading the 8 row lines.

Note: If there is need for multiple keys to be pressed simultaneously (shift or control keys, etc.), isolation diodes must be connected at each of the column lines.

Enable the TSO by connecting jumper J10 position 5.

Membrane TSO specifications:

Max. resolution (matrix size):		8 by 12

Contact resistance, max:			2  kOhm

Connector K1:  20 pin single line pin array:

oooooooooooooooooooo

1                  20

1	Row 7

2	Row 6

3	Row 5

4	Row 4

5	Row 3

6	Row 2

7	Row 1

8	Row 0

9	Column 0

10	Column 1

11	Column 2

12	Column 3

13	Column 4

14	Column 5

15	Column 6

16	Column 7

17	Column 8

18	Column 9

19	Column 10

20	Column 11

Net Address select switch can also be connected to this connector. It can be used as an option to selecting net address from the SETUP. Pin 20 is is the common output, pins 2 to 8 are used for data inputs, to select net address in range 0 to 127 (normally, only values 0 to 63 are used). Pin 8 is the least significant bit. Pin 1 is the enable input: net addres is read from the switch only if the contact on pin 1 is closed. If Touch Screen device is in use (at either connector K1 or K2), a diode must be connected to each data line prevent data content (see the drawing below).

The net address switch status is read at power-on. An open contact is interpreted as zero bit. If the contact on pin 1 is open, the value stored in the CMOS-RAM is used. That value can be set from the SETUP (see chapter 3.2). The default value for Net Address is 0 (the widlcard address), but it may be changed by patching the EPROM (see chapter 7).

If all the 12 columns are required for the membrane Touch Screen device, the net address select switch must be closed only at power-on and opened before the Touch Screen can be used.

K2  -  OPTICAL T.S.O. INTERFACE

Optical TSO (AP988030/A, AP988030/B, AP988030/BB or AP988040) is connected to this interface. The TSO is enabled by connecting the jumper J10.5. The AP988300 then performs the scanning of the TSO and sends coordinates to the host serial port.

Connector K2:  20 pin dual line pin array:

2        20

oooooooooo

oooooooooo

1        19

1	+5V		 2	+5V

3	Data 4		 4	Data 3

5	Data 2		 6	Data 1

7	Data 0		 8	Addr 2

9	Addr 1		10	Addr 0

11	Clock		12	--

13	Addr 4		14	--

15	GND		16	GND

17	+12V		18	+12V

19	tx/rx sel	20	Beeper

K3  -  VT220 KEYBOARD INTERFACE

A VT220 compatible keyboard can be connected to this interface. The interface is asynchronous serial interface, 4800 bps, RS232C compatible. Note: the VT220 keyboard needs 12V operating voltage. See chapter 6.1.2.

This connector can also be used to connect an ASCII keyboard or other external device, such as external Touch Screen controller, Bar Code Reader etc. A special EPROM is needed for such devices.

Connector K3:  4 pin modular jack  (telephone connector)

1	RxD  (input)

2	+12V

3	GND

4	TxD  (output)

K4  -  SERIAL PRINTER INTERFACE

RS232C compatible serial printer  interface. This interface is output only. Therefore, an XON-XOFF handshake is not possible, and hardware DTR handshake must be used. Use jumper J14 to select which pin is used for handshake input.

A 9-pin D-type connector may be connected here by using an ISD connector and flat cable. Pin assignment is compatible with standard 9-pin connectors. (Pin numbers for D-connector are shown in parenthesis.)

Connector K4:  10 pin dual in-line pin array

2   10

ooooo

ooooo

1   9

1 (1)	--	 2 (6)	DTR *

3 (2)	TxD **	 4 (7)	DTR *

5 (3)	TxD **	 6 (8)	DTR *

7 (4)	DTR *	 8 (9)	--

9 (5)	GND	10	--

* Use J14 to select a pin for DTR handshake line.

** Use J13 to select a pin for TxD.

K5  -  ARCNET coaxial interface adapter

Connector K1:  10 pin single line pin array:

oooooooooo

1        10

The adapter for coaxial ARCNET interface is connected to this connector. (If RS485 Arcnet is used, no adapter is needed.)

Note:    ARCNET interface option is not available on Rev C boards.

K6  -  HOST SERIAL PORT (RS232C)

An RS232C compatible serial interface for the host computer. Communication parameters for this interface are selected from the SETUP menu. Jumpers J7, J11, J12 and J19 are used to select the pin assignment. A 9-pin D-type connector may be connected here by using an ISD connector and flat cable; pin assignment is compatible with standard 9-pin D-connectors used on many computers. (D-connector pin positions are shown in parenthesis.)

Connector K6:  10 pin dual in-line pin array        	( <  = input,   >  = output )

2   10

ooooo

ooooo

1   9

DTE*	DCE*		DTE*	DCE*

1 (1)	< DCD ¤		 2 (6)	  --	  --

3 (2)	> TxD	< RxD	 4 (7)	< CTS	> RTS

5 (3)	< RxD	> TxD	 6 (8)	> RTS	< CTS

7 (4)	> DTR	> DTR 	 8 (9)	  --	  --

9 (5)	  GND	  GND	10	  --	  --

*	Use jumpers J11 and J12 to select either DTE or DCE pin order

¤ 	Use jumper J19 to select DCD input instead of CTS

Use jumper J9 to tie DTR to +12V, if required.

Typical RS232C cable connections:

In these examples, we assume a 9-pin D-connector has been connected to K6 with direct flat cable and AP988300 has been jumpered as DTE (the default). Pin numbers refer to D-connector pins.

AP988300 (DTE) connected to another		AP988300 (DTE) connected to a DCE

DTE device						device

(e.g. IBM-PC  with 25-pin D-connector):		(e.g. IBM-PC with 9-pin D-connector)

AP988300		DTE device			AP988300	    DCE device

(9-pin D-sub)	(25-pin D-sub)			(9-pin D-sub)	    (9-pin D-sub)

1 o	o 1	1 o	o 1

2 o	o 2	2 o	o 2

3 o	o 3	3 o	o 3

4 o	o 4	4 o	o 4

5 o	o 5	5 o	o 5

6 o	o 6	6 o	o 6

7 o	o 7	7 o	o 7

8 o	o 8	8 o	o 8

9 o		9 o	o 9

o 20

K7  -  IBM PC KEYBOARD INTERFACE

An IBM-PC compatible keyboard may be connected here. The interface is synchronous serial interface, using TTL levels. Standard version accepts PC/XT-type keyboard (or AT keyboard if it has a switch for XT compatible mode).  An EPROM for IBM-AT compatible keyboard is available on request.

It is possible to have both VT220 and IBM-PC keyboard connected simultaneously. You can also connect a special device (bar code reader, tracker ball, external TSO etc.) to one interface and keyboard to the other one.

Connector K7:  5 pin DIN connector

µ §

1	CLK

2	Data in/out

3	Reset

4	GND

5	+5V

Hardware note:  If IBM-AT type keyboard is used, pull-up resistors of around 10K must be connected on the keyboard CLK and Data pins. On Rev C board, these resistors are already installed.

K8  -  HOST SERIAL PORT, CURRENT LOOP   (optional)

Passive 20 mA opto isolated current loop interface. The current must be supplied externally.  Note:  Maximum voltage for the external current source is 24 V.

Use jumper J8 to select RxD from RS232C or Current  Loop.

Connector K8:  10 pin dual in-line pin array

2   10

ooooo

ooooo

1   9

1 (1)	+12V	 2 (6)	RxD (+)

3 (2)	+12V	 4 (7)	TxD (-)

5 (3)	GND	 6 (8)	TxD (+)

7 (4)	GND 	 8 (9)	GND

9 (5)	RxD (-)	10	--

K9  -  DISPLAY CONNECTOR

Interface for the Flat Panel Display. This interface is software configurable to suit most display types. Pin assignments may vary between different display types and manufacturers. Display type is selected by installing corresponding Alphasoft-300 EPROM. On some cases, you may need to change crystal (XT2) and jumper J18 position. 10 to 22 pin connectors may be assembled on the PCB. The pin assignments for Finlux / Planar / Sharp EL displays and for Panasonic Plasma display are given below.

Connector K9:

a)  EL-display: 				b)  Panasonic PG3 Plasma display:

2      16				2        20

oooooooo				oooooooooo

oooooooo				oooooooooo

1      15				1        19

1  +12V		  2  +12V		 1  +5V  	   2  +5V

3  +5V		  4  +5V			 3  ENA	 	   4  BACKG

5  Blank	  6  n/c			 5  Vsync	   6  GND

7  GND  	  8  GND			 7  Hsync	   8  GND

9  VSync	 10  GND			 9  Blank	  10  GND

11  HSync	 12  GND			11  Clock	  12  GND

13  CLK  	 14  GND			13  Data0	  14  GND

15  Data 	 16  GND			15  Data1	  16  GND

17  Data2	  18  GND

19  Data3	  20  GND

c)  Cable connection for Planar EL640.480-A

or EL7768MS display:

AP988300  (20-pin)			Display panel  (34-pin)

VDat 0	19 o	o 2	VDat 0

VDat 1	17 o	o 4	VDat 1

VDat 2	15 o	o 6	VDat 2

VDat 3	13 o	o 8	VDat 3

VClk	11 o	o 10	VDat 4

Blank	 9 o	o 12	VDat 5

HS	 7 o	o 14	VDat 6

VS	 5 o	o 16	VDat 7

Enable*	 3 o	o 18	VClk		* Enable-pin is used

+5V	 1 o	o 20	Blank		  for screen saver, to

o 22	HS		  switch display off.

GND: 8,10,	o 24	VS		  If screen saver is not

12,14,16,	o 26	Self Test	  needed, this pin may

18,20	   o	o 28	Enable*		  be left unconnected.

o 30	Low-Power

o 1,3,5,15,17,19,21  GND

K10  -  HOST SERIAL PORT,  RS485   (optional)

An RS485 compatible serial interface for host computer. (The RS485 interface is also electrically compatible with the RS422 standard.)

RS485 allows the 38400 bps transfer rate to be used at distances of up to 1500 m. Up to 32 terminals may be connected on the same party line. Single twisted pair cable (2-wire RS485) or two twisted pairs (4-wire RS485) can be used to connect the terminals and the host computer.

This connector is also used for ArcNet-type RS485 network, when the ArcNet controller is installed and the Coaxial adapter is not  in use. This is a low-cost alternative for true ARCNET.  Data transfer rate is 2.5 Mbit/s.

Set jumper J8 to position 2-3 to select RxD from RS485. Use jumpers J25, J26 and J10 position 6 to select 2-wire or 4-wire RS485 mode or RS422-compatible point-to-point mode:

J25	J26	J10.6

2-wire multi-point			1-2	2-3	closed

4-wire multi-point			2-3	1-2	closed

4-wire point-to-point (RS422)		2-3	1-2	open

On-board terminator resistors may be enabled by closing jumpers J22 and J24. Terminator resistors are needed only at the both ends of the cable. For other terminals on the same party line, leave the jumpers open. For point-to-point communications (e.g. RS422), close only jumper J22.

For multi-point communications, additional biasing resistors must be connected somewhere on the cable, to avoid illegal state on the bus when none of the terminals is transmitting. (An application note is available for more information.)

Note: RS485 and Current Loop may not co-exist on the same board.

Connector K10:  10 pin dual-in-line pin array

2   10

ooooo

ooooo

1   9

1 (1)	TxD -	 2 (6)	n/c

3 (2)	TxD +	 4 (7)	n/c

5 (3)	RxD +	 6 (8)	n/c

7 (4)	RxD -	 8 (9)	n/c

9 (5)	GND	10	n/c

For 2-wire RS485, pins 1 and 3 serve both as TxD and RxD.

6.4.  TECHNICAL SPECIFICATIONS:

AP988300		- Standard version

AP988300-EXT	- Extended temperature range

AP988300-F	- High speed version

Technology:		CMOS

CPU:			Z180,    6.144 MHz

Z180,  12.288 MHz  (AP988300-F)

EPROM:			max   64 K  (27256 or 27512)  on Rev B board

max  512K  (27010 to 27040)  on Rev C board

System RAM:		32 K static, with battery back-up. Expandable to 128 K.

Video processor:		NEC  D72020

Video RAM:		512 K Dual Port VRAM

Logic device:		XC3020 Logic Cell Array

|  | AP988300 | AP988300-EXT |  |
| --- | --- | --- | --- |
| Supply voltage: | 9 to 32 VDC | 9 to 32 VDC | 1) |
| Supply current: @ 12V | 150 mA | 150 mA | typ 2) |
| 5V output for display: cont. | 500 mA | 500 mA | max |
| Peak (3 sec) | 1000 mA | 1000 mA | max |
| Operating temperature: | 0 to 70 °C | -20 to 75 °C |  |
| Storage temperature: | -25 to 85 °C | -40 to 125 °C |  |
| Humidity: (non-condensing) | 0 to 90 % | 0 to 90 % |  |
| MTBF: (calculated) | 45000 hours | 45000 hours |  |
| Dimensions: | 228 x 110 x 24 mm | 228 x 110 x 24 mm |  |
| Weight: | 210 g | 210 g |  |

1)  Supply voltage has to be set as required by the display panel in use.

2)  Excluding display and keyboard

BOARD DIMENSIONS:

- All dimensions in millimeters,  ± 0.5 mm.

- All screw holes:  Æ 3.2 mm.

  