7.  TECHNICAL INFORMATION

7.1  EPROM VERSIONS

The AP988300 controller board can be equipped with wide variety of EPROM versions, depending on the hardware configuration and optional features required. It is important that correct EPROM version is installed for each hardware configuration.

The EPROM is identified by code written on the EPROM label, for example:

3E400M

(1)	 AP988300   ¾¾¾¾		 ¾¾   Touch screen = membrane   (4)

(2)	 EL-display   ¾¾¾¾¾	        ¾¾¾¾   Resolution 640.400   (3)

(1) 	The number 3 indicates that this is AP988300 firmware.

(2) 	The next 1-3 letters define the display type:

E		=  EL-display

L		=  LCD

P		=  Plasma display

PD		=  Panasonic plasma, PD type

CRT		=  CRT display (VGA monitor or SCART TV-set)

(3) 	Resolution of the display:

320		=  320.256

350		=  640.350

400		=  640.400

480		=  640.480

512		=  512.256

(4)	Remaining letters define display model, special keyboard option, touch screen, and other options that differ from standard version:

(nothing)	=  VT220 and IBM-XT keyboards, Alpha Point TSO,

standard version

AA		=  EL640.480-AA color EL display

C		=  EL640.350-DA color EL display

S		=  Sharp EL display (if different from Planar)

U		=  EL640.200-U3 display

CT or C		=  Carroll Touch TSO

M		=  Membrane TSO

AT		=  IBM-AT keyboard

HEX		=  Hexadecimal keyboard at membrane connector

25		=  25 lines in terminal mode (standard = 24 lines)

In addition to the above, several customer specific options may be indicated in the EPROM ID.

Note:  If you change the EPROM in order to use different display type, you may need to change the dot clock crystal, too. Standard crystal frequencies used are 15, 16 and 24 MHz. Normally, highest frequency that display panel accepts is used. When you order the controller board for specified display panel, correct crystal and EPROM will be installed by Alpha Point.

Below is a list of crystal frequencies for some of the common displays:

Planar		EL320.256	16	MHz

EL512.256	15	MHz

EL640.200-U	15	MHz

EL640.350  *)	16	MHz

EL640.350-DA	24	MHz

EL640.400  *)	16	MHz

EL640.400-C3	24	MHz

EL640.480-A	24	MHz  

EL8358HR	16	MHz

Sharp		LJ-320U26	16	MHz

LJ-512U25	16	MHz

LJ-640U27	16 - 22	MHz

Panasonic	MD400B640PD3	16	MHz

MD400L640PG2	16	MHz

MD400B640PG3	19.66	MHz  

MD480L640PG3	24	MHz

LCD		640.400		16	MHz

640.480		24	MHz

CRT		TV, SCART	14.318	MHz  

VGA monitor	25	MHz  

*)  Old display type

  Special cable for display needed

7.2  PATCHING THE EPROM

Several default parameters on the Alphasoft-300 may be changed by patching the EPROM contents. The list below gives information about each parameter. Parameters are referred by labels. See Appendix A for the addresses corresponding each label.

SER_INI+0:	Databits, stopbits, parity

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CkP | CkF | 0 | EVP | 0 | DB | PE | SP |

Where:	SP		(bit 0)	=  1 stopbit (0) or 2 stopbits (1)

PE		(bit 1)	=  Parity disabled (0) or enabled (1).

DP		(bit 2)	=  7 databits (0) or 8 databits (1)

EVP	(bit 4)	=  Even parity (0) or odd parity (1).

CkF		(bit 6)	=  Check Framing Errors: 0 = yes, 1 = no

CkP		(bit 7)	=  Check Parity Errors: 0 = yes, 1 = no

Example:

To set 8 bits, 1 stopbit, Even parity:  change this location to hold value 16H.

SER_INI+1:	Baud rate		00H		= 38400 bps	05H	=  1200 bps

01H		= 19200 bps	06H	=    600 bps

02H		=   9600 bps	0DH	=    300 bps

03H		=   4800 bps	0EH	=    150 bps

04H		=   2400 bps

( For AP988300-F, all the baud rates are doubled )

SER_INI+2:	Handshake	00	=  none

01	=  Xon-Xoff

02	=  Hardware (CTS)

03	=  Both

SER_INI+3:	Duplex		00	=  Full duplex

01	=  Half duplex (local echo)

SER_INI+4:	Font size		00	=  8x8 font

01	=  8x16 font  (8x14 on EL640.350 display)

SER_INI+5:	Language (national replacement character set)  and GR character set

00	=  USASCII		06	=  French

01	=  Finnish		07	=  Norwegian

02	=  German		08	=  Swiss

03	=  Danish		09	=  Italian

04	=  British		0A	=  Dutch

05	=  Spanish		0B	=  Swedish

Bits 5 to 7 select the default GR set in use (for characters 80h to FFh). To select GR set, add the following values to the language selection above:

00	=  USASCII		60	=  DEC Multinational extension

20	=  National		80	=  IBM-PC extended set

40	=  DEC special graphics	A0	=  ADM3 semigraphic set

SER_INI+6:	Screen saver	00		=  Screen saver disabled

>00		=  Screen saver time in minutes (1 to 255)

note: in order to screen saver time to be displayed correctly on SETUP, the time should be set to one of: 00, 05, 0F, 1E or 3C (off, 5, 15, 30 or 60 minutes)

SER_INI+7:	National / multinational mode;  key repeat mode

00	=  Multinational mode. Use 8-bit DEC multinational character set.

01	=  National mode.  Use 7-bit NRC sets for transmitted characters.

80	=  Key repeat disable for IBM keyboard.

SER_INI+8:	Cursor mode

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | CE | 0 | 0 | B | T |

Where:	T 		(bit 0)	=  Cursor type:	 		0 = line,	1 = block

B		(bit 1)	=  Cursor blink:		0 = steady,	1 = blinking

CE		(bit 4)	=  Cursor disable: 		0 = enabled,	1 = disabled

SER_INI+9:	Emulation	00	=  VT100

04	=  VT52

08	=  ADM3  (or custom specific)

SER_INI+10:	Keyboard language and mode:

Language:				    Mode:

00	= USA			00	= Typewriter

01	= Finnish			80	= Data Processing

02	= French

03	= UK

04	= German

05	= Swiss German

06	= Dutch

07	= Italian

08	= Norwegian

09	= Danish

0A	= Swiss French

0B	= Swedish

SER_INI+11:	Net address:	00			= Wild card address, receive all messages

01 - 4F	= Address of the terminal

FF			= Do not interpret net header  (SOH...STX)

SER_INI+12:	Screen mode

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 0 | SM | 0 | 0 |

Where:	SM	= Screen Mode:	0  =  Normal

1  =  Reverse

SER_INI+13:	Keyboard  status:

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CA | KA | IM | 0 | NL | 0 | CL | 0 |

Where:	CL		(bit 1)	=  Caps Lock status after reset:  0 = off, 1 = on

NL		(bit 3)	=  New line mode:  0 = off (linefeed), 1 = on (newline)

IM		(bit 5)	=  Insert mode:  0 = off (replace), 1 = on (insert)

KA		(bit 6)	=  Numeric keypad mode:  0 = numeric, 1 = application

CA		(bit 7)	=  Cursor key mode:  0 = cursor, 1 = application

SER_INI+14:	Printer baud:	-  See SER_INI+1 (Baud Rate) above.

-  4800 bps (03H) is recommended.

-  Use value FFh for parallel printer on membrane TSO connector.

SER_INI+15:	Printer bits and parity:

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | EVP | 0 | DP | PE | SP |

Where:	SP		(bit 0)	=  1 stopbit (0)	  or  2 stopbits (1)

PE		(bit 1)	=  Parity disabled (0)  or  enabled (1).

DP		(bit 2)	=  7 databits (0)  or  8 databits (1)

EVP	(bit 4)	=  Even parity (0)  or  Odd parity (1).

TSO_INI:	TSO mode

| Bit: | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OC | SC | 0 | Net | QM | DR | FU | DB |

Where:	DB		(bit 0)	=	Debug Mode - Send message even if only one of the coordinates (x or y) detected

FU		(bit 1)	=	Finger Up detect  - send  '?? ??' message when the 'key' is released.

DR		(bit 2)	=	Double resolution mode using interpolation (optical TSO only)

QM		(bit 3)	=	Query Mode - Send message only when TSO report query received from the host.

NET	(bit 4)	=	Network mode (insert net address in message)

SC 		(bit 6)	=	Single Character mode - convert coordinates to a single character using TSOTABL.

OC 		(bit 7)	=	Old Alphasoft Compatibility mode. Set this bit to get TSO message exactly the same as Alphasoft-030 V 2.1 or older.

TSO_INI+1	TSO repeat delay		- Delay time before repeating, if same TSO position is touched constantly.

1 = shortest delay,  255 = max delay.

TSO_INI+2	TSO repeat rate		- Repeat rate, when same TSO position is touched constantly.

1 = fastest rate, 255 slowest.

0 = no repeat.

TSO_INI+3	TSO speed	a)	on OPTICAL TSO:  Tracking speed

The rate of coordinates sent, when finger is moved.

1 = fastest, 255 slowest rate.

0 = no tracking, new coordinate sent only after finger has

been removed from the screen.

b)	on MEMBRANE TSO:  Scan rate

Membrane switch matrics scan rate, ms / row.

TSOX:	TSO X-resolution	 -	This parameter must be set equal to the physical resolution of the TSO connected

to controller:

18h (24 dec)   for AP988040  TSO frame

30h (48 dec)   for AP988030/BB TSO frame

04h to 0Ch (4 to 12 dec)   for membrane TSO

TSOY:	TSO Y-resolution 	-	Optical TSO only

-	As above.  Value must be 10h (16) or 1Eh (30).

MINBRK:	Min. scans key closed until accepted  (membrane TSO only)

-	Selects, how many consecutive scans a contact must be closed until accepted. Must be less than or equal to parameter GRPSIZ. The contact matrix is scanned GRPSIZ times and then, if at least MINBRK of the scans indicate contact closed, the coordinate is accepted.

GRPSIZ:	a)  on optical TSO:	TSO Group size

08h		for AP988030/B or AP988040

10h		for AP988030/BB

b)  on membrane TSO:	Scan count

-	Number of times the membrane matrix is scanned before checking and sending the coordinates.

KEYPAD:	Key conversion table for keypad keys, function keys and special editing keys.

-	In the table, 16 bytes are reserved for each key: 8 bytes for normal mode and 8 bytes for the application mode. If less than 8 bytes are needed, the rest is filled with nulls (00h), which are not sent to the host computer.

CURPAD:	Key conversion table for cursor keys, as above.

CODESC:	Scan codes for the keys that are used as cursor keys in SETUP.

CURPA52:	Key conversion table for cursor keys in VT52 mode.

KEYPA52:	Key conversion table for keypad keys in VT52 mode.

FKEYINI:	Top-row function key strings (F6 to F20)

- The order of the keys in the table is F6, F7, F8 etc.

- Length of string can vary.  Each string is terminated by character ST (hex 9C).

FKEY100:	Top-row function key codes in the VT100, VT52  and ADM3 modes

-  1 character / key

-  Order:  F16...F20, F5...F15

REPTABL:	Characters with national variations.

- These characters are handled, when generating the national replacement character sets

(NRC-sets).

REP1:	Start of the first national replacement character set (Finnish).

-  Other tables are in the order listed at language parameter (SER_INI+5).

-	Length of each table is 12 bytes

-	Each entry in the table is the character code in the DEC SUPPLEMENTAL CHARACTER SET (if bit 7 is zero) or in the USASCII character set (if bit 7 is set). The figure of the specified character is copied to the national set in place of the ASCII character. If entry in the table is Space (20h), that character is same as in ASCII set and is thus not copied.

SET20:	Character generator for the USASCII character set, 8x16 font.

-	96 characters (codes 20 to 7Fh), 16 bytes per character.

-	First byte is the topmost pixel row of character

-	Least significant bit is the leftmost pixel of character

GRA2: 	Char. gen. for the DEC SPECIAL character set (semigraphics).

-  Contains only characters in range 60h to 7Fh.

SET21:	Char. gen. for the DEC SUPPLEMENTAL character set.

-  96 characters (codes 20 to 7Fh), 16 bytes per character.

SET23:	Char. gen. for the IBM PC character set

-  128 characters, 16 bytes per character. (IBM PC codes 80h to FFh)

SET24:	Char. gen. for the ADM3 semigraphic character set.

-  Contains only characters in range 60h to 7Fh.

SET0:	Character generator for the USASCII character set, 8x8 font.

-  96 characters (codes 20 to 7Fh), 8 bytes per character.

GRADAT: 	Char. gen. for the DEC SPECIAL character set (semigraphics), 8x8 font.

-  Contains only characters in range 60h to 7Fh.

SET1:	Char. gen. for the DEC SUPPLEMENTAL character set, 8x8 font.

-  96 characters (codes 20 to 7Fh), 8 bytes per character.

SET4:	Char. gen. for the ADM3 semigraphic character set, 8x8 font.

-  Contains only characters in range 60h to 7Fh.

DBUG_CH:	Debug Character  (Optical TSO only)

- Character sent to host in TSO single character mode, if incomplete coordinate pair is detected and debug mode is enabled.

FING_UP:	Finger Up -character  (Optical or membrane TSO)

- Character code sent to host computer, when 'key' is released in single character mode (if finger-up detect mode is enabled).

TSOTABL:	TSO single character conversion table.

- The table contains one byte for each TSO crossover point. How the characters are handled, depends on the value as follows:

00			Not sent to host. No click generated.

01 to 7F	Sent to host directly.

80 to DF	Converted to string using ALTTAB  *

E0			Caps Lock key  *

E1			Setup key  *

FD			Ctrl key  *

FE			ALT key  *

FF			SHIFT key  *

* Membrane TSO only

SHIFTBL:	Shift conversion table  (membrane TSO only)

-  For converting other than alphabetic keys.

-  Contains one byte for each character in range 20H to 3Fh.

ALTLEN:	Length of each entry in the ALTTAB.   Normally 04h.

ALTTAB:	ALT key / special key conversion table.   (membrane TSO only)

-	4 bytes reserved for each key.  If less than 4 bytes needed, fill rest which nulls (00h).  If more than 4 bytes are needed, change the ALTLEN above.

-	First entry for code 80h, next 81h etc.

MACROS:	ROM-Macros to be used in Graphics Mode

-	Macros are stored as plain ASCII text.

-	Start of macro is marked with colon (:),  followed by single character macro name,  followed by CR.

-	Each command must be terminated with CR.  (LF is not needed.)

-	Start of next macro ends the macro definition.

-	End of last macro is marked with colon followed by NUL (OOh).

   