1.   INTRODUCTION

This manual describes the operation of the Alpha Point AP988300 flat panel display controller board and the AP988040 Touch Screen Overlay (TSO) input device.  Information for user, programmer and hardware designer is provided.

1.1.   AP988300 CONTROLLER BOARD

The AP988300 is an intelligent flat panel graphics controller which is connected to the host computer using a serial line (RS232C, RS485 or Current Loop). Together with a display panel and keyboard it forms a full function graphics terminal. A Touch Screen Overlay can be connected to AP988300 as an additional feature.

AP988300 can be used to control Electroluminescent, Plasma or LCD flat panel displays. Due to the use of a Field Programmable Logic Device known as LCA (Logic Cell Array), a wide variety of display types and resolutions are supported by simply changing the EPROM and, on some cases, the crystal.

Emulations for popular terminal types are available, to provide compatibility with existing software. In graphics mode, a special command set with easy-to-learn mnemonics is used to draw the graphics primitives.

EPROM						ARCNET

(Alphasoft-300)		Z180 MPU		controller			Beeper

(optional)

TSO

i/f

Host

RS232C

i/f

Serial

Printer

i/f

Power	     Video		Dual Port		Display	       LCA		IBM-PC		VT220

supply	     processor	Video-RAM	i/f			keyboard		keyboard

i/f		i/f

Fig 1.1.   AP988300 Controller Board

Wide variety of interfaces and configuration options are available on-board.

The AP988300 is connected to the host computer via a serial interface. Standard configuration contains an RS232C interface. Optionally, an RS485 or Current Loop interface may be fitted.  For ultra-fast multipoint communications, an ARCNET network interface may be installed on board, either as RS485 or true ARCNET (coaxial) interface.

The controller board also has interfaces for an ASCII, VT220 or IBM PC compatible keyboard, for a serial or parallel printer and for an optical or membrane type TSO (Touch Screen Overlay). These interfaces may also be used to connect other devices, such as bar code readers, external locator devices, membrane keyboards etc.

The Alpha Point TSO (Touch Screen Overlay) frame board can be connected directly to the AP988300 controller. No additional hardware or I/O is needed on the host computer to interface the TSO.

The AP988300 is an enhanced version of the AP988030 Intelligent Flat Panel Graphics Controller. AP988300 is software compatible with the AP988030, but has several advantages:

- Smaller board size

- CMOS-technology, resulting lower power consumption and higher reliability

- Higher speed, up to 10 times faster than AP988030

- Four times larger video RAM (512 KB)

- Single operating voltage, on-board DC-DC converters as standard

- Single board version configurable for all display types and resolutions

- VT220 compatible keyboard interface

- Watchdog

- Optional RS485, Current Loop and ARCNET interfaces on-board

- Larger EPROM and RAM space for more sophisticated firmware

1.2.  OVERVIEW OF THE AP988300 OPERATION

1.2.1.  OPERATING MODES

The Alphasoft-300 firmware of the AP988300 has three different operating modes: terminal mode, graphics mode and mixed mode. Moving from one mode to another is done by an escape sequence entered from keyboard or from the host computer.

In addition, there is a setup mode, which can be selected from the keyboard.

Possible transitions between the three operation modes are:

Fig 1.2.    State transition diagram

Note:  The ESC G sequence can be used to enter graphics mode if VT100 emulation is in use.  In VT52 and ADM3 emulations, this sequence is reserved for other purposes. To enter graphics mode from VT52 mode, you has to enter VT100 mode first. To enter graphics mode from ADM3 mode, use command ESC g.

The SETUP-mode may be entered from any mode by using the setup-key (see chapter 3.2). After exiting the setup mode, the terminal mode is entered.

1.2.2   TERMINAL MODE

After power-on or reset, the AP988300 controller is in Terminal Mode. From graphics mode or mixed mode, user can select terminal mode  by issuing command

ESC F

In Terminal Mode, the ALPHASOFT-300 firmware emulates one of the following terminals, as selected by the EMULATION field on the SETUP:

- Lear Siegler  ADM3

- DEC  VT-52

- DEC  VT-100.

Other emulations are available on request.

The VT-100 emulation is based on the ANSI X3.64 standard, and it is the most versatile of the emulation options. Other emulations are non-ANSI modes.

The VT-100 emulation supports all ANSI cursor commands (including absolute positioning, relative movements, index, reverse index, save and restore cursor), erasing and editing commands, character attributes, double width and double height characters, semigraphic and special character sets, setting and clearing tab stops, report commands, printer support etc. In addition to standard VT-100 commands, several enhanced commands  have been added. The full explanation of each command is given in the chapter 4.

1.2.3   GRAPHICS MODE

When the AP988300 is in terminal mode or mixed mode,  the user may select graphics mode  by issuing command

ESC G

(ESC is an ASCII control character, hex 1B).

AP988300 enters graphics mode immediately after this command. Screen is cleared (when entering from terminal mode) and cursor disappears.

In the graphics mode, wide variety of high resolution graphics commands is available, including primitives to draw dot, line, arc, circle, Bezier curve, box, filled rectangle, to paint irregular area, to write text in different sizes to all eight directions, to copy pixel blocks  etc.

The addressable screen size (the virtual display area) is 2048 x 2048 pixels. The physical display acts as a window (viewport) to this virtual display (see fig. 1.3). The viewport can be moved with the viewport command (Vxx,yy). All drawing commands can use the full virtual display area, independently of the physical resolution of the display.

The graphics commands are explained in detail in the chapter 5.

Fig 1.3:  The  viewport within the virtual screen after executing command V200,300

1.2.4   MIXED MODE

The third operating mode, mixed mode, is selected by issuing command

ESC X

The mixed mode allows you to mix the VT-100 commands with high resolution graphics. The Mixed Mode works just like the Terminal Mode, except that scrolling is not allowed.

With this feature the user is able to draw logos, trade marks etc. on the terminal mode text page. Additionally this allows the use of graphic  windows  within the text mode e.g. bar graphs, pie-charts, curves etc.

From Mixed Mode, you can move to the Graphics Mode any time using the command ESC G. The display is not erased. After performing graphics commands, go back to mixed mode with the command ESC X. When the mixed mode is in use there is a cursor on the screen and when the graphics mode is entered, the cursor disappears.

The mixed more uses the lowest left part of the graphics workspace (viewport 0,0).

1.3   THE TOUCH SCREEN OVERLAY OPTION

A touch screen input device is formed by a Touch Screen Overlay (IR-LED matrix frame board) and some on-board circuitry on the AP988300 controller. (In case a display controller is not needed, a separate Touch Screen controller is also available.)

AP988040-9 is the first member of the new generation Touch Screen Overlays (TSO). It replaces the old AP988030/A TSO. The AP988040-9 is designed for 9" flat panel displays, such as Planar EL640.400, Sharp LJ640U31 etc. Its physical resolution is 24 x 16 beams. Other TSO frame boards currently available are:

AP988030/B	for 12" displays,	24 x 16  resolution

AP988030/BB	for 12" displays, 	48 x 30  resolution

The Touch Screen function is based on a matrix of infrared LED - detector pairs. The controller continuously scans the LED-matrix, typically 20 times/second. When a break on the beams is detected, the corresponding coordinates are sent to the host via the serial line.

TSO coordinates are sent to Host  as ASCII string using the following (default) format:

XX YY<CR><LF>

where XX is the corresponding x-coordinate (00 to 23) and YY is y-coordinate (00 to 15), 2 digits each, separated by a space. Lower left corner of the display is the origin (00 00).

Example:

When the upper right corner of the screen is pointed, the following message is sent:

23 15<CR><LF>

The above range of coordinates is for the standard resolution and mode. The AP988030/BB has higher resolution. Also, the logical resolution may be doubled by selecting the double resolution mode with TSO MODE command (see chapter 4.1: Touch Screen Commands and chapter 5: SET TSO command). In this mode, interpolation is used to double the physical resolution.  The possible ranges for the coordinates thus are:

X range	Y range

AP988040-9,

AP988030/B:

Single resolution 	00 to 23	00 to 15

Double resolution	00 to 46	00 to 30

AP988030/BB:

Single resolution 	00 to 47	00 to 29

Double resolution	00 to 94	00 to 58

You may simulate the TSO operation by entering the coordinates from keyboard in the above format.

If multiple coordinates are detected or larger area of beams is blocked, the coordinate pair sent to host depends on the mode selected. In normal mode, the coordinate sent is the coordinate closest to the lower right corner of the screen. In double resolution mode, the coordinate pair sent is the midpoint of the area.

The TSO has a repeat feature, similar to the key repeat on the keyboard. User may change the repeat delay and rate by using the TSO TIMING command. Repeat delay defines the time before the same coordinate pair is sent, when the finger is hold on the same position on screen. Repeat rate defines the rate on which subsequent coordinate pairs are sent after the repeat delay. Repeat may also be disabled by setting repeat rate to zero. Tracking speed defines the rate on which the coordinates are sent when finger is moved on the screen. If tracking speed is set to zero, only one coordinate pair is sent, until the finger is removed and re-inserted on screen. For all parameters, the fastest possible rate is the scanning rate.

Different operating modes may also be set, by using the TSO MODE commands in terminal mode (see chapter 4.1) or graphics mode (see chapter 5). The following modes can be set:

Debug mode:

If this mode is set, a special debug message is sent in case of incomplete coordinate pair, i.e. only X or Y coordinate is detected. The debug message is similar to the normal message, except that missing coordinate is replaced by two question marks (??). This is useful for locating the faulty component on TSO frame. If debug mode is not set (default mode), incomplete coordinates are not sent to the host.

Exit report mode (finger-up detect):

If this mode is set, a special finger-up message is sent, when the finger is removed from the screen. In this message, both coordinates are represented as question marks, i.e. the message is in form '?? ??<cr><lf>'.

Query mode:

If this mode is set, the coordinate message is sent only, when a special TSO query command (DC2) is received from the host.

Double resolution mode:

This mode doubles the logical resolution by using interpolation. The actual coordinate is calculated by adding together the lowest and highest detected coordinate.

Single character mode:

In this mode, a single character is sent to host instead of a coordinate pair. This allows the TSO to simulate a keyboard. The coordinate pair is converted to single character using a conversion table (TSOTABL) stored in EPROM. Custom specific conversion tables are available on request.

Network mode:

If this mode is set, the net address is added at the beginning of the coordinate report.

The above modes may be combined by setting and resetting the modes independently. By default, all modes are reset, unless changed by patching the EPROM (see chapter 7).

MEMBRANE SWITCH TOUCH SCREEN OVERLAY

Optionally, a transparent membrane switch matrix TSO can be used with the AP988300 controller. This requires different version of the Alphasoft EPROM. Coordinates are sent to the host similarly, except that the resolution is typically lower than on the optical TSO.

The membrane connector may also be used to connect an undecoded matrix keyboard. This option provides an easy way to build customized keyboards, ranging from simple numeric keypads or function keys to full alphanumeric keyboards. Special key conversion tables are available on request.

EXTERNAL TOUCH SCREEN OVERLAY

Third party touch screen overlay or other pointing device can also be used with the AP988300. Such device is typically connected to one of the keyboard interfaces on AP988300. Most of the touch screen modes and options described above are not available with external TSO. You need to refer TSO manufacturer manuals for configuring it. You can send configuration commands to external device using the 'Send Command to External TSO' command (See chapter 4.1).

See also:

Chapter 4.1:	Touch Screen Commands

Chapter 5:	Set TSO command

Chapter 6.2:	Jumper settings, jumpers J10.4 and J10.5

Chapter 6.3:	TSO connectors K1 and K2

Chapter 7:	TSO_INI

Rev B board only.