3.  USING THE TERMINAL

3.1  THE KEYBOARD

Many different types of keyboards can be used with the AP988300. However, a VT220-compatible keyboard is recommended. This chapter describes the use of each key on the VT220 keyboard. On other keyboards, such as IBM-PC keyboard, some of the keys may not be available and some may be labeled differently. (See page 4-41 for main differences.)

VT220 keyboards are available with several different national key-layouts. Currently, the following national layouts are supported:

North America (USASCII)	Germany/Austria

United Kingdom			Italy

Denmark			Norway

Holland				Sweden

Finland				Switzerland (German)

France/Belgium			Switzerland (French)

Figure 3.1 shows the 105-key (USASCII) keyboard which consists of the following sections:

Main keypad

Editing keypad

Numeric keypad

Top-row function keys

Four indicator lights

Two audible indicators

Each of the sections is described in the following pages.

Fig. 3.1:  North American keyboard

3.1.1 MAIN KEYPAD

Fig. 3.2.: Main keypad

This part of keyboard (fig. 3.2) operates like a standard typewriter keyboard. In addition to normal alphanumeric characters this keypad has the following special function keys:

Tab key sends a horizontal tab, which normally moves the cursor to the next tab stop.

Ctrl key pressed together with another key sends a control code to the host system. The interpretation of each control code is application dependent. Ctrl-key does not send any code when pressed alone.

Lock key is the shift lock for alphabetic part of the main keypad. When pressed once, it makes the alphabetic keys send uppercase characters. Pressing Lock again makes the alphabetic keys send lower case characters. Pressing Lock key several times toggles the Lock on and off. The Lock indicator light above the top-row function keys shows the state of the Lock-key. When the indicator light is on, the uppercase codes will be sent. Otherwise, the lower case codes will be sent.

ò or Shift key alone does not generate any key code. It needs to be pressed together with another key. For alphabetic keys the Shift key is the uppercase selector. For other keys (like numeric keys in the top-row of the main keypad) Shift selects the upper character engraved in the keycaps. For some of the function keys (like F2/Print Screen) using the Shift key with them generates a predefined control function.

¿ or Return key sends a CR (Carriage Return) code, or CR + LF codes, if Line Feed mode is set. In some cases Return moves the cursor to the next line when editing text. In other cases Return signals the end of an operation. The interpretation is application dependent. (This key should not be confused for Enter key on numeric keypad.)

Õ  (Delete) key sends a Delete (DEL) character. Normally, pressing the delete key erases one character to the left of the cursor. Typing Shift-Delete sends a Cancel (CAN) character.

Compose Character key lets you create characters that do not exist as standard keys on your keyboard. (On the IBM-XT keyboard, the ALT key acts as Compose Character key.) You start the compose sequence by pressing the Compose character key first, followed by two keys that you want to merge into one character according to the compose rules. If the compose sequence is incorrect, the Bell tone is generated and no character is composed. See chapter 3.1.7.

3.1.2 EDITING KEYPAD

Fig.3.3.  Editing keypad

This keypad (see figure 3.3) includes four cursor control keys (cursor right, left, up and down) with which you can move the visible cursor on the screen to any of the four directions.

The rest of the keys in this keypad have their editing functions engraved in their keycaps. Each key sends a special control string. The actual effect of these keys is application dependent. You should refer to the application software manuals of your host computer for the explanation of these keys.

3.1.3 NUMERIC KEYPAD

Fig. 3.4. Numeric keypad

This keypad (figure 3.4) lets you enter numeric data as you would with a standard calculator keyboard. Some of the keys ( PF1, PF2, PF3 and PF4) may have functions assigned to them by the application software.

On IBM-XT keyboard, the num-lock must be on, in order to use the numeric keys. When the num-lock is not on, these keys act as cursor keys.

3.1.4 TOP-ROW FUNCTION KEYS

Fig. 3.5:  Top-row function keys

Top-row function keys are shown in figure 3.5. The first five function keys (F1 - F5) have the following fixed meanings assigned to them:

F1 = Hold Screen freezes the screen display and stops any new characters from being displayed. The Hold Screen indicator light indicates the frozen state. Pressing the F1 again releases the hold and turns off the Hold Screen indicator light. (On the IBM-XT keyboard, use Scroll Lock key instead.)

F2 = Print Screen works only if you have a local printer connected to the printer port of your AP988300. Pressing F2 sends the text on the screen to the printer ("hard copy" of the screen).

Pressing Ctrl/F2 toggles the Auto Print mode on/off. In this mode the current display line is copied to printer when the cursor moves to the next line.

F3 = SETUP mode toggle. Press F3 once and you see the setup-menu. Select the desired values in the fields of the setup-menu and, finally, press the F3 once more. This saves all the current setup values in the nonvolatile memory of the AP988300. See chapter 3.2.

F4 = ESC. Sends an ESC control character. Note: This is a non-standard use of the F4 function key. The VT220 does not have an ESC-key. On VT220, F4 is used to control modem signals. On AP988300, only the CTS ( = Clear To Send) signal is provided for modem hardware handshake. (The IBM-XT keyboard has additionally a separate ESC key.)

F5 = Break.  Sends a break signal on the host line.

F6 to F20 are function keys used by application programs. Each key sends a special control sequence. These keys are also used as programmable soft function keys, when pressed simultaneously with the shift key.

3.1.5 INDICATOR LIGHTS

Fig. 3.6:  The keyboard indicator lights

The VT220 keyboard has four special indicator lights:

Hold Screen indicator, when lit, indicates that the screen output has been freezed by using F1 function key

Lock indicator shows the caps lock state selected by Lock key on the main keypad.

Compose indicator turns on when the Compose Character key or one of the diacritical keys is pressed. It is turned off by a successful completion of the compose-sequence or by an erroneous sequence. If the compose sequence was "illegal" the Bell tone is sounded.

Wait indicator turns on if the keyboard is prevented (locked) from sending information. The keyboard can be unlocked by a command from the host line.

On the IBM-PC/XT keyboard, only the Hold Screen (= Scroll Lock) and Lock (Caps Lock) lights are available. In addition there is Num Lock light, which indicates that the keypad acts as numeric keypad rather than as cursor keypad.

3.1.6 AUDIBLE INDICATORS

The VT220 keyboard can generate both a key-click and a bell sounds. The key-click occurs every time you press a key on the keyboard, except when you press Shift- or Ctrl-keys (these two keys do not generate any key codes on their own).

The Bell-sound is generated by the keyboard in following cases:

- When the terminal receives a BEL control character from the host computer

- After a compose character error

The durations and frequencies of the two audible indicators are:

Keyclick	:	approximately 2 milliseconds/2 kHz

Bell:		approximately 125 milliseconds/2 kHz

If the VT220 keyboard is not used on your system configuration, the bell sound may also be generated by on-board beeper on the AP988300 controller board. The on-board beeper also generates the 'key-click' sound for the Touch Screen Overlay and IBM-XT keyboard.

3.1.7	COMPOSING CHARACTERS

In AP988300 you can use so called composite characters by using two-stroke or three-stroke compose sequences which enable you to create characters that you cannot produce with normal keystrokes.

The two-stroke compose sequence is initiated by entering one of the following diacritical marks: acute accent ('), grave accent (`), umlaut (¨), circumflex accent (^), tilde mark (~) or ring mark (o). Create the two-stroke compose sequence as follows:

1.  Find the character you want to create in the first column of table 3.1.

2. Type the two characters in the third column of table 3.1 corresponding to the first column character you want to create. When you press the first character (the diacritical mark), the compose indicator turns on. After pressing the second key, the composite character is sent.

Example:

To create an È ( E grave), type ` followed by E.

Note: two-stroke compose sequences are not available with the North American (USASCII) keyboard.

A three-stroke compose sequence is created as follows:

1.  Find the character you want to create in the first column of table 3.1.

2.  Press the Compose Character-key first. (On IBM-PC keyboard, the ALT key acts as Compose Character key.) The Compose-indicator light in the keyboard turns on, indicating the compose-mode.

3.  Type the two characters in the second column of table 3.1 corresponding to the first column character you want to create. The composite characters marked with an asterisk (*) are order sensitive, and must be entered in the order given. Others may be entered in any order.

Example:

To create an Ä (A umlaut), press Compose Character, and then type " or ¨ followed by A  (or A followed by " or ¨ ).

If you succeeded in creating a "legal" compose sequence, the Compose-indicator turns off and the composite character will be transmitted to the application. If your sequence is invalid, it is aborted and the keyboard bell sounds.

You can abort a compose sequence by using the Delete-key. No character is sent to the application if a compose sequence is aborted.

Note: all the composite characters shown in table 3.1 are available only if the MULTINATIONAL mode is selected (see sections 1.3.3 and 3.4). In national mode only characters included in the selected NRC set can be created, other characters cause a bell sound to be generated.

The table 3.1 on next page lists all the compose sequences.  For 3-stroke sequence, press the compose-key first, then the two characters given in the table. The order of the characters entered is not significant expect for those composite characters marked with an asterisk (*). Note: (space) means the space key.

TABLE 3.1:     C O M P O S I T E   C H A R A C T E R S

COMPOSITE CHARACTER		3-STROKE		2-STROKE

SEQUENCE		SEQUENCE

__________________________________________________________________________

"	(quotation mark)		"(space)			¨(space)

#	(number sign)			++

'	(apostrophe)			' (space)		' (space)

@	(commercial at)			aa or AA

[	(opening bracket)		((

\	(backslash)			// or /<

]	(closing bracket)			))

^	(circumflex accent)		^(space)			^(space)

'	(single quote)			' (space)		' (space)

{	(opening brace)			(-

|	(vertical line)			/^

}	(closing brace)			)-

~	(tilde)				~(space)		~(space)

¡	(inverted !)			!!

¢	(cent sign)			c/ or /c

£	(pound sign)			l- or L-

¥	(yen sign)			y- or Y-

¶	(section sign)			so or SO

¤	(currency sign)			xo or XO

©	(copyright sign)			co or CO

ª	(feminine ordinal indicator)	a- or A-

«	(angle quotation mark left)	<<

°	(degree sign)			0^ or (space) #		°(space)

±	(plus/minus sign)		+-

2	(superscript 2)			2^

3	(superscript 3)			3^

µ	(micro sign) *			/u or /U

?	(paragraph sign)			p! or P!

·	(middle dot)			.^

1	(superscript 1)			1^

º	(masculine ordinal indicator)	o- or O-

»	(angle quotation mark right)	>>

1/4	(fraction one-quarter) *		1 4

1/2	(fraction one-half) *		1 2

¿	(inverted question mark)		??

À	(A grave)			A`			` A

Á	(A acute)			A'			' A

Â	(A circumflex)			A^			^A

Ã	(A tilde)			A~			~A

Ä	(A umlaut)			A"  or  A¨		¨A

Å	(A ring)				A* or  A°		°A

Æ	(A E ligature) *			AE

Ç	(C cedilla)			C ,

È	(E grave)			E`			` E

É	(E acute)			E'			' E

Ê	(E circumflex)			E^			^E

Ë	(E umlaut)			E" or  E¨		¨E

Ì	(I grave)			I`			` I

Í	(I acute)			I'			' I

Î	(I circumflex)			I^			^I

Ï	(I umlaut)			I" or  I¨			¨I

Ñ	(N tilde)			N~			~N

Ò	(O grave)			O`			` O

Ó	(O acute)			O'			' O

Ô	(O circumflex)			O^			^O

Õ	(O tilde)			O~			~O

Ö	(O umlaut)			O" or  O¨		¨O

Œ	(O E ligature) *			O E

Ø	(O slash)			O/

Ù	(U grave)			U`			` U

Ú	(U acute)			U'			' U

Û	(U circumflex)			U^			^U

Ü	(U umlaut)			U" or  U¨		¨U

Y	(Y umlaut)			Y" or  Y¨		¨Y

ß	(German small sharp s)		ss

à	(a grave)			a`			` a

á	(a acute)			a'			' a

â	(a circumflex)			a^			^a

ã	(a tilde)				a~			~a

ä	(a umlaut)			a" or a¨			¨a

å	(a ring)				a* or a o		o a

æ	(a e ligature) *			a e

ç	(c cedilla)			c ,

è	(e grave)			e`			` e

é	(e acute)			e'			' e

ê	(e circumflex)			e^			^e

ë	(e umlaut)			e" or e¨			¨e

ì	(i grave)				i`			` i

í	(i acute)				i'			' i

î	(i circumflex)			i^			^i

ï	(i umlaut)			i" or i¨			¨i

ñ	(n tilde)				n~			~n

ò	(o grave)			o`			` o

ó	(o acute)			o'			' o

ô	(o circumflex)			o^			^o

õ	(o tilde)				o~			~o

ö	(o umlaut)			o" or o¨			¨o

œ	(o e ligature) *			o e

ø	(o slash)			o/

ù	(u grave)			u`			` u

ú	(u acute)			u'			' u

û	(u circumflex)			u^			^u

ü	(u umlaut)			u" or u¨			¨u

ÿ	(y umlaut)			y" or y¨			¨y

* Order sensitive

3.2.  SETUP MODE

Setup mode is entered by pressing the SETUP key (F3). To exit setup mode, press SETUP key again. When the setup mode is entered, the terminal is turned into local mode and the setup screen is displayed.

AP988300 V 4.36

S  E  T  U  P

BAUD RATE   DATA BITS   STOP BITS   PARITY      HANDSHAKE    DUPLEX

9600          8           1        none       XON/XOFF      FULL

EMULATION   LANGUAGE     FONT       KEYBOARD    KB MODE      RET KEY

VT-100     Finnish      8x16       Finnish     Typewrit       CR

CURSOR      CUR.MODE    SCROLL                  SCREEN      SCR SAVER

Block        Blink       Jump                   reverse      30 min

PRINTER     PR BAUD     PR BITS     PR PARITY               NET ADDR

normal        4800        8+1         none                    (00)

Cursor keys = Select item,        Space / Enter = Change item,         F3 = exit

Ctrl-C = Cancel,       Ctrl-D = Default values,         Ctrl-P = Power-up values

Fig 3.7. The SETUP screen

To select an item to change, use Left, Right, Up and Down arrow keys to move selection bar, or Carriage Return to go to the beginning of the next line.

To change item, use Space (previous item) or numeric keypad Enter-key (next item). Old values (the values that were active when setup mode was entered) can be restored by pressing Ctrl-C. To restore values that were active when power was switched on, press Ctrl-P. To read default values from the EPROM, press Ctrl-D.

To exit setup mode, press SETUP (F3) or ESC key. The parameters are automatically saved into the non-volatile RAM, the screen is cleared and terminal is set on-line. The new setup values take effect, when you exit the setup.

On the following pages, each setup item is explained.

TABLE 3.2:     S E T U P   I T E M S

| Item | Explanation | Options available |
| --- | --- | --- |
|  | Line 1: |  |
| Baud | Host line baud rate | 150 to 38400 bps |
| Data bits | Number of data bits | 7 or 8, with no-check option |
| Stop bits | Number of stop bits | 1 or 2 |
| Parity | Parity bit | Even, Odd, no-check, or None |
| Handshake | Handshake protocol | XON/XOFF , CTS or None |
| Duplex | Local echo, On-line mode | Full, Half, Blk Mode or Local |
|  | Line 2: |  |
| Emulation | Terminal emulation level | VT52, VT100 or ADM3 |
| Language | National replacement char set | (12 sets available) |
| Font | Character font size | 8x8 or 8x16 |
| Keyboard | Keyboard language | (12 options available) |
| Kb Mode | Keyboard mode | Typewriter or Data processing |
| CR Key | Return key function | CR or CR+LF |
|  | Line 3: |  |
| Cursor | Cursor type | Line or Block, or cursor off |
| Cur mode | Cursor mode | Blinking or Steady |
| Scroll | Scrolling mode | (currently not implemented) |
| Screen | Screen mode | Normal or Reverse |
| Scr Sav | Screen saver time | 5, 15, 30 or 60 min, or off |
|  | Line 4: |  |
| Printer | Printing mode | Off, Autoprint or Printer controller |
| Pr Baud | Printer baud rate | 150 to 38400 bps |
| Pr Bits | Printer bits | 7+1, 7+2, 8+1 or 8+2 |
| Pr Parity | Printer parity | Even, Odd or None |
| Net Addr | Network address | 00 to 63 |

BAUD:

Selects the data transfer rate for the host serial interface.

Note:  Since the maximum transfer rate allowed by the RS232C standard is 20 kBaud, the RS485 interface is recommended if 38400 bps transfer rate is required.

DATA BITS:

Selects the number of data bits in character, excluding parity and start/stop bits. Number of data bits must be set equal to the setting on the host computer. If the setting is wrong,  false characters may be displayed, or framing/parity errors may occur.

If option 7 or 8 is selected, the characters with framing error are not displayed. If 7 or 8 with n/ck (no check) -option is selected, all the characters are displayed regardless of the error status.

STOP BITS:

Number of stop bits added after characters (1  or 2).

PARITY:

Selects the usage of the parity check bit on received/transmitted characters.

If parity is set to EVEN or ODD, an even or odd parity bit is added to transmitted characters, and received characters with parity error are not displayed on the screen.

If EVEN N/CK or ODD N/CK is selected, a parity bit is added to transmitted characters, but there is no parity check for received characters.

If NONE option is selected, the parity bit is not added to transmitted characters, and there is no parity check for received characters.

HANDSHAKE:

Handshake protocol used for host communication: XON/XOFF or hardware CTS handshake. See chapter 3.3 for details. The CTS handshake option affects only the CTS output. Hardware handshake input option and the connector pins used for handshake are configured by on-board jumpers.

DUPLEX:

If HALF duplex is selected, the characters entered from the keyboard are also locally displayed on screen, and the host computer should not echo them back.

If FULL duplex is selected, the characters are only sent to the host computer which may then echo them back to the terminal.

In LOCAL mode, the characters are only displayed on screen and not sent to the host computer. The characters received from host are not displayed.

If BLK MODE is selected, the text entered is displayed locally. When RETURN key is pressed, the whole line is sent to the host computer. Characters received from host are displayed normally.  (This mode is also known as Line Mode on some terminals.)

EMULATION:

VT-100:  If this option is selected, the VT100 (ANSI) commands listed in the chapter 4.1 are recognized.

VT-52: this is a non-ANSI mode that provides compatibility with Digital VT52 terminal. In this mode, a special command set is used (see chapter 4.2), and some keys (e.g. keypad and cursor keys) send different strings (see chapter 4.4).

ADM3:  the terminal emulates Lear Siegler ADM3 terminal (see chapter 4.3).

LANGUAGE:

Selects one of the 12 national replacement character (NRC) sets used to display text on screen. NRC sets are used to display national characters in 7-bit environment. If 8-bit multinational character set (IBM-PC or DEC multinational set) is used, USA should be selected  on this field.

FONT:

Selects font size to be used for any text displayed. The actual font sizes depend on the display resolution. Typically, two font sizes are available: 8x8 and 8x16. The 8x16 is the default font. For 8x8 font, some of the character sets may not be available. For some display resolutions, only one font size is available.

The font size can also be changed by SET FONT command in the Graphics Mode.

KEYBOARD:

Selects the keyboard language (national keyboard layout). You should select the keyboard layout that matches the keyboard currently connected to the terminal.

Note: the key layouts are designed for VT220 keyboard. This may not match with some of the national layouts on IBM-PC keyboards. This setting has no effect on the ASCII keyboard.

Unlike on VT100, the keyboard selected does not affect the NRC set used. The NRC set is selected from LANGUAGE field.

KB MODE:

If TYPEWRITER mode is selected, the characters marked on the left side of the keycaps are used. If DATA PROCESSING mode is selected, the characters marked on the right side of the keycaps are used. This option is ignored, if USA keyboard layout or an ASCII keyboard is in use.

CR KEY:

Selects the function of RETURN key and the effect of any LF control characters received. This field has the same effect as NEW LINE MODE command.

If CR is selected, the return key sends only a carriage return, and any received LF control characters are interpreted as LF only.

If CR+LF is selected, the return key send two characters, CR and LF. Also, any received LF control characters are interpreted as new line (CR+LF).

CURSOR:

Selects  block cursor, line cursor or cursor off.

CUR.TYP:

Cursor type: blinking or steady. The cursor type and mode may also be set by host computer commands, but the status is saved in the non-volatile memory only when set from the SETUP.

SCROLL:

This option is currently not implemented.

SCREEN:

Selects the screen mode: normal or reverse video.

SCR SAV:

The SCREEN SAVER time. The screen saver blanks the screen after the defined time has passed since the last keypress or last character received from the host. The screen is automatically turned on when a key is pressed or a character is received from the host. If the screen saver is set OFF, the screen will not blank.

PRINTER:

Printing mode. If AUTOPR (Autoprint) is selected, every Line Feed causes the current line to be printed before moving cursor to next line.

If CONTROL (Printer Controller Mode) is selected, the terminal sends all the received characters directly to the printer without displaying them.

If NORMAL mode is selected, the printer operations are performed only by host computer command or by pressing F2 key.

The printer mode may also be changed by host computer commands or by Ctrl-F2 key.

PR BAUD:

Selects the data transfer rate for the printer interface.

Note:  Baud rate 4800, with 8+1 bits and no parity is recommended for the printer interface. If any other settings are selected, the VT220 keyboard (or external Touch Screen device connected to keyboard port) is not read during the printer operations.

PR BITS:

Selects the number of data and stop bits for the printer interface.

PR PARITY:

Selects the parity check option for the printer interface: EVEN, ODD or none.

NET ADDR:

Sets the net address used for multi-point communications. If multiple terminals are connected to the same serial line (e.g. by using RS485), this allows the host computer to selectively write on any of the terminals and to identify the terminal that sends the characters. (See chapter 4.1: Multi-point Communications).

If address is set to 00, the terminal receives all the messages.

3.3.  HOST COMMUNICATIONS

3.3.1. TRANSMIT AND RECEIVE BUFFERS

The data received from the host computer is buffered with a 2 K-byte receive buffer. The host can thus continue to send data at full speed even if slow commands are executed.

The data flow between the host and the terminal is controlled either by XON-XOFF protocol or hardware CTS handshake, selectable by on-board jumpers and the SETUP screen.

When the free buffer space decreases below 400 bytes, the terminal sends an XOFF or turns the CTS line high. If the host does not stop sending the characters, and the buffer goes full, an XOFF is send at each character received. Characters lost because of buffer full condition are displayed on screen as error character (checker board).

After sending the XOFF, the terminal reads characters from the buffer. When most of the characters are read, and there are only 100 bytes left in the buffer, the terminal sends an XON to the host or turns CTS line low again.

There is also a 256 byte transmit buffer for characters to be sent to the host computer. Host may stop the terminal from sending characters with XON-XOFF protocol or hardware handshake, as above. Characters received from the keyboard or from Touch Screen Overlay are then stored in the transmit buffer, until transmission is re-enabled by the host computer. If transmit buffer becomes full, any subsequent characters from keyboard or Touch Screen device are lost.

The host may also lock the keyboard and/or disable the Touch Screen Overlay, in which case the characters from these devices are discarded, not stored in the transmit buffer.

3.3.2. RS485 INTERFACE

The optional RS485 interface allows up to 32 terminals and/or host computers to be connected on a single party line, using a single twisted pair cable (bus configuration). Alternatively, a cable of two twisted pairs can be used for full duplex communications between single host and multiple terminals. This configuration is also compatible with the RS422 interface on point-to-point communications. The receiver(s) of the data transmitted through the party line is selected by using network addresses. See chapters 4.1 and 5 (Network Commands) for details.

Cable length on either configuration may be up to 1500 metres. However, if multiple terminals are connected on the cable, the maximum data transfer distance is reduced. If more terminals or longer distances are required, repeaters may be used.

When the single twisted pair cable (bus configuration) is used, any terminal or host computer connected to the party line can send data to any other unit. However, only one terminal can send data at any given time. A handshake mechanism is therefore needed in order to avoid collision. The host computer is normally responsible of distributing the resources.

When the two twisted pair cable configuration is used, the host computer may send data to the terminals while simultaneously receiving data from a terminal. This makes it easier to implement the communications software on the host computer. In this case, one terminal can not send data to another terminal. Logically, this is a star configuration network.

The RS485 interface with two twisted pair cable may also be used in point-to-point communications, just to allow longer distances and higher baud rates. In this case, the interface can be used just like the normal RS232C interface.

When long cables are used, terminating resistors must be used on both ends of the cable. For multi-point communications, additional biasing resistors must be used to keep the bus in legal state while none of the terminals are transmitting. An application note is available for more details.

See chapter 6.2 for jumper settings for the RS485 interface.